3GPP TSG-SA WG6 Meeting #64	S6-245344

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245058)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.180 | CR | 0008 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clause A.1, A.2 and A.6 update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Generic_IOPS |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR updates the configuration data. It includes the location user profile configuration data in annex A.1, and updates the MC service UE configuration data in A.2 as well as the initial MC service UE configuration data in annex A.6 to make them generic to the network access. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - include location user profile configuration data and refer to the related subclause in TS 23.280. - update the initial MC service UE configuration data and MC service UE configuration data to include 5GS access information. - add a new annex to refer to the location user profile configuration data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing configurations related to location user profile, and the existing initial MC service UE configuration data, and MC service UE configuration data are not agnostic to the network access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.1, A.2, A.6, and A.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The reference [23.289] is included in CR0010. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

# A.1	General

This Annex provides information about the static configuration data needed for the support of MC services in the IOPS mode of operation.

The configuration data belong to one of the following categories:

-	MC service UE configuration data (see subclause A.2);

-	MC service user profile configuration data (see subclause A.3);

-	MC service group configuration data (see subclause A.4); and

-	MC service configuration data (see subclause A.5).

-	Location user profile configuration data (see subclause A.8)

The configuration data in each configuration category corresponds to a single instance of the category type i.e. the MC service UE, MC service group, MC service user and MC service configuration data refers to the information that will be stored against each MC service UE, MC service group, MC service user and MC service.

NOTE:	The configuration data described in this Annex together with corresponding configuration data provided in 3GPP TS 23.280 [3], 3GPP TS 23.289 [23.289], 3GPP TS 23.379 [5] and 3GPP TS 23.282 [6] represent the complete set of data for each configuration data category element.

The columns in the tables have the following meanings:

-	Reference: the reference of the corresponding requirement in 3GPP TS 22.346 [9] and 3GPP TS 22.280 [11] or the corresponding clause from either the present document or the referenced document.

-	Parameter description: A short definition of the semantics of the corresponding item of data, including denotation of the level of the parameter in the configuration hierarchy.

-	When it is not clear to which functional entities the parameter is configured, then one or more columns indicating this are provided where the following nomenclature is used:

-	"Y" to denote "Yes" i.e. the parameter denoted for the row needs to be configured to the functional entity denoted for the column.

-	"N" to denote "No" i.e. the parameter denoted for the row does not need to be configured to the functional entity denoted for the column.

Parameters within a set of configuration data have a level within a hierarchy that pertains only to that configuration data. The level of a parameter within the hierarchy of the configuration data is denoted by use of the character ">" in the parameter description field within each table, one per level. Parameters that are at the top-most level within the hierarchy have no ">" character. Parameters that have one or more ">" characters are child parameters of the first parameter above them that has one less ">" character. Parent parameters are parameters that have one or more child parameters. Parent parameters act solely as a "grouping" of their child parameters and therefore do not contain an actual value themselves i.e. they are just containers for their child parameters.

Each parameter that can be configured online shall only be configured through one online reference point. Each parameter that can be configured offline shall only be configured through one offline reference point. The most recent configuration data made available to the MC service UE shall always overwrite previous configuration data, irrespective of whether the configuration data was provided via the online or offline mechanism.

# * * * * Second change * * * *

# A.2	MC service UE configuration data

MC service UE configuration data has to be known by an MC service UE after MC service authorization. The CSC-4 reference point, specified in 3GPP TS 23.280 [3], is used for configuration between the configuration management server and the configuration management client on the MC service UE when the MC service UE is on-network.

MC service UE configuration data can be configured offline using the CSC-11 reference point specified in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

Within each MC service, the MC service UE configuration data can be the same or different across MC service UEs.

The MCPTT UE configuration data specified in table A.2-1 in 3GPP TS 23.379 [5] is also used, as needed, in the IOPS mode of operation for the MCPTT service.

The MCData UE configuration data specified in table A.2-1 in 3GPP TS 23.282 [6] is also used, as needed, in the IOPS mode of operation for the MCData service.

# * * * * Third change * * * *

# A.6	Initial MC service UE configuration data

Initial MC service UE configuration data is essential to the MC service UE to successfully connect to the MC system, as described in 3GPP TS 23.280 [3], and 3GPP TS 23.289 [23.289].

The configuration data defined in table A.6-1 is additionally provided to the MC service UE's clients to successfully connect to the IOPS MC system in the IOPS mode of operation. The MC service UE's clients (e.g. MC service client, IOPS connectivity client) obtain the data during the bootstrap process (described in clause 10.1.1 in 3GPP TS 23.280 [3]), and can be configured on the MC service UE offline using the CSC-11 reference point or via other means e.g. as part of the MC service client's provisioning on the UE, using a device management procedure.

Table A.6-1: Initial MC service UE configuration data (IOPS)

| Reference | Parameter description |
| --- | --- |
| Clause 5.4.3 | PDN connectivity information in IOPS (NOTE 1) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC services PDN in IOPS |
|  | >> APN |
|  | >> PDN access credentials |
|  | PDU session information in IOPS (NOTE 2) |
|  | > IOPS HPLMN ID and optionally IOPS VPLMN ID to which the data pertains |
|  | > MC service PDU in IOPS |
|  | >> DNN |
|  | >> PDU access credentials |
|  | >Default configured slice(s) S-NSSAI(s) |
|  | Application plane server identity information |
|  | > Indication of whether the UE shall use IPv4 or IPv6 for the support of MC services in IOPS |
|  | > IOPS MC connectivity function |
|  | >> Server URI |
| NOTE 1: These configurations shall only be used to access via IOPS EPS system. NOTE 2: These configurations shall only be used to access via IOPS 5GS system. |  |

# * * * * Fourth change * * * *

## A.x	Location user profile configuration data

The location user profile configuration data defined in annex A.8 of 3GPP TS 23.280 [3] is applicable as needed in the IOPS mode of operation.

# * * * * End of changes * * * *