3GPP TSG-SA WG6 Meeting #64	S6-245581

Orlando, US, 18th – 22nd November 2024	(revision of S6-245293)

Source:	Ericsson

Title:	ML model discovery with the BDT support

Spec:	3GPP TS 23.482 V0.3.0

Agenda item:	9.10

Document for:	Approval

1. Introduction

The BDT-based to the consumer is indicated in clause 8.11.3; thus, it require a proper BDT configuration provisioning in the request and response messages.

2. Reason for Change

The BDT configuration in the ML model discovery is not specified.

3. Conclusions

This pCR proposes to clarify the BDT configuration in ML model discovery.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.482 V0.3.0.

* * * First Change * * * *

### 8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model. Optionally, the ML repository can initiate a model transfer process to the consumer.

NOTE 1:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enabler consumer can rely on model repository to get the model details.

Editor’s Note 1: If List of allowed vendors is not present then how to get consent from model owner to share the model to the requestor is FFS.

Editor’s Note 2: How to consider whether model is discoverable or not is FFS.

* * * Next Change * * * *

#### 8.11.4.3	ML model information discovery request

Table 8.11.4.3-1 describes the information flow from the AIMLE server to the ML repository as a request for the ML model information discovery.

Table 8.11.4.3-1: ML model information discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the ML repository consumer performing the request. |
| Filtering criteria | M | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |
| BDT indicator | O | Indicates that BDT should be used for the ML model transfer. |

Editor’s Note: The inclusion of ML model ID and Base Model ID are FFS.

* * * Next Change * * * *

#### 8.11.4.4	ML model information discovery response

Table 8.11.4.4-1 describes the information flow from the model repository to the AIMLE server as a response for the ML model information discovery request.

Table 8.11.4.4-1: ML model information discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request. |
| ML model list | O | Represents the discovered list of ML models. This information element shall be provided if the result is success. |
| > ML model ID | M | Represents the ML model ID. |
| > ML model | O (NOTE 1) (NOTE 2) | Represents the ML model. This information element may be provided if the result is success. |
| > ML model retrieval endpoint | O (NOTE 1) (NOTE 2) | Represents the ML model retrieval endpoint (e.g., URL, URI, IP address) at Model repository that can be used to download the ML model. This information element may be provided if the result is success. |
| > BDT configuration | O (NOTE 1) (NOTE 2) | Represents the BDT configuration for the ML model transfer, i.e., the scheduled timeslot for BDT. |
| NOTE 1: Only one of these information elements shall be provided. NOTE 2: For transfer learning, the list of all possible models matching the filtering criteria can be included along with model address. |  |  |

# *** End of Changes ***