3GPP TSG-SA WG6 Meeting #64	S6-245590

Orlando, USA, 18th – 22nd November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0061 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML Model Performance Degradation Detection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TNO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2024-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The conclusion for Key Issue #2 in TR 23.700-82 indicates that solution #12 is a candidate for normative work. This contribution introduces support for ML model performance degradation detection into TS 23.436. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added functional description. - Added procedure for ML model performance degradation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conent for ML model performance degradation detection is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.y(new), 8.x(new), 8.x.1(new), 8.x.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The procedure in 8.x is referenced by the pCR S6-245589 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 6.y	Support for ML Model Performance Degradation Detection

This feature supports ML model performance degradation detection to allow a consumer (such as e.g. AIMLE Server) to be notified on performance degradation of an ML model. Such detection can be used to support application layer AI/ML operations, e.g. supporting decision making on retrain an ML model.

* * * Next Change * * * *

## 8.x	Procedure for ML Model Performance Degradation Detection

### 8.x.1	General

This clause describes a procedure for ML model performance degradation detection for supporting application layer AI/ML operations.

### 8.x.2	Procedure

Pre-conditions:

-	An ADAE Server, acting as an AIMLE consumer, receives a trained ML model (or ML model information) from the AIMLE Server.

Figure 8.x.2-1: ADAES supports for ML model performance degradation detection

This procedure corresponds to the step 0b in clause 8.x.2.1.

1.	The ADAE Server receives a request from the consumer for analytics, generates analytics by using the ML model provided by AIMLE Server, and responds/notifies to the consumer with the required analytics.

2.	The ADAE Server requests the consumer to provide feedback on the usage of the analytics.

3.	The consumer uses the analytics for its operations and collects operation results. Performance degradation may be found from the operation results. The performance degradation may be caused by e.g., insufficient analytics accuracy, or the current analytics cannot fulfill the changed conditions at the consumer.

4.	The consumer provides feedback on the result of usage of the analytics to the ADAE Server.

5.	The ADAE Server updates the analytics accuracy based on feedback information from the consumer, and checks the performance of the ML model, which is used to generate the analytics, e.g., performance degradation of the ML model.

* * * END of Changes * * * *