3GPP TSG-SA WG6 Meeting #64	S6-245638

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245566,5240)

Source:	Huawei, Hisilicon

Title:	clarification on AI/ML member selection for AI/ML task transfer assist

Spec:	3GPP TS 23.482 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yajie Hu (huyajie1@huawei.com)

1. Introduction

This paper is introduced for AI/ML member selection for AI/ML task transfer assist

2. Reason for Change

Currently, the detail of select target AI/ML member for AI/ML task tranfer is missing in the specification, then the procedure is not clear.

Thus, this paper is introduced for providing detaile of AI/ML member selection for AI/ML task transfer assist.

3. Proposal

It is proposed to agree the following pCR to TS 23.482.

* * * First Change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: According to 3GPP TS 28.105 [8], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

ML model lifecycle: The lifecycle of an ML model aka ML model operational workflow consists of a sequence of ML operations for a given ML task / job (such job can be an analytics task or a VAL automation task). This definition is aligned with the 3GPP definition on ML model lifecycle according to 3GPP TS 28.105 [8].

ML model training: According to 3GPP TS 28.105 [8], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

ML model inference: According to 3GPP TS 28.105 [8], ML model inference includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

AIMLE service: An AIMLE service is an AIMLE capability which aims assisting in performing or enabling one or more AIML operations.

AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint, and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enabler client and may be configured e.g., to provide ML model training and inference locally e.g., at the VAL UE side.

AIMLE client set identifier: an identifier of the set of selected AIMLE clients.

AI/ML server: an application layer entity which is an AI/ML endpoint, and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enablement server.

AI/ML intermediate model: For federated learning, members need to train models for multiple rounds, intermediate models indicate the model which do not meet the required training rounds and/or meet the requirements of the fereration training.

FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process.

FL client: An FL member which locally trains the ML model as requested by the FL server. Such FL client functionality can be at the network (e.g. AIMLE server with FL client capability) or at the device side (e.g. AIMLE client with FL client capability).

FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

Split AI/ML operation pipeline: A Split AI/ML operation pipeline is a workflow for ML model inference in which AI/ML endpoints are organized and collaborate to process ML models in sequential stages, where processing at each stage involves ML model inference on the output of the previous stage.

* * * NEXT Change * * * *

8.6.2.1	Request AIMLE Server to assist AI/ML task transfer

Pre-conditions:

1.	The information of target AI/ML member (e.g., another AIMLE Client or VAL Client different from the source AI/ML member) is unknown at the source AI/ML member. The source AI/ML member decides that assistance from the AIMLE Server is needed.

2. The source determines which AI/ML tasks to transfer. For example, if the AI/ML task is model training, AI/ML model are obtained by source AI/ML member performing AI/ML model training. Then the source AI/ML member determines the remaining AI/ML model training requirement based on the AI/ML operation results and AI/ML model traning requirement.

Figure 8.6.2.1-1: Procedure for requesting AIMLE Server assist AI/ML task transfer

Figure 8.6.2.1-1 illustrates the procedure for requesting AIMLE Server to assist the AI/ML task transfer. The detailed corresponding procedure is detailed as follows:

1.	The source AI/ML member (e.g., an AIMLE Client) sends an AI/ML task transfer assist request to AIMLE Server for assisting the AI/ML task transfer. The request message contains the information as specified in Table 8.6.3.2-1.

Editor’s Note:	The procedure can also be used for transfer AI/ML services (e.g., ML model performance evaluation, ML model update) between AI/ML members. The details are FFS.

2.	The AIMLE Server discovers other AI/ML members (e.g., other AIMLE Clients) directly or via ML repository, and selects one or more target AI/ML member(s) based on the request from the source AI/ML member. For example if the task is model training, the AI/MLE server determines the target AI/ML member(s) bases on AI/ML member(s)’s capability and task’s AI/ML model training requirement. The task’s AI/ML model training requirement indicates the AI/ML Information.

The AIMLE Server sends request to the selected target AI/ML member(s) for intermediate AI/ML operation information transfer, as defined in clause 8.6.2.2. Then, the AIMLE Server determines the transfer mode, i.e., transfer with server-controlled or transfer directly from the source AI/ML member to the target AI/ML member.

If target AI/ML member(s) is discovered, the AIMLE Server generates assistance information for the AI/ML task transfer from the source AI/ML member to the target AI/ML member(s) (e.g., time window for the transfer).

3.	The AIMLE Server sends an AI/ML task transfer assist response to the source AI/ML member with the information of the target AI/ML member(s), and assistance information generated in step 5. The response message contains the information as specified in Table 8.6.3.3-1.

8.6.3.2	AI/ML task transfer assist request

Table 8.6.3.2-1 describes information elements for the AI/ML task transfer assist request from the source AI/ML member to the AIMLE Server.

Table 8.6.3.2-1: AI/ML task transfer assist request

| Information element | Status | Description |
| --- | --- | --- |
| Request ID | M | The identifier of the VAL UE ID (the source AI/ML member). |
| VAL service ID | O | The identifier of the VAL service for which the assistance information is requested. |
| AI/ML Task Type | M | The type of the AI/ML operation (e.g. ML model training). |
| AI/ML Information Type | M | The type of the AI/ML information in the AI/ML task need be transferred (e.g. intermediate AI/ML operation status, intermediate AI/ML operation results). |
| > AI/ML model remaining training requirement (NOTE) | O | Indicate the requirement for AI/ML model training including, required remaining training resource, required remaining training number of iterations, |
| > AI/ML intermediate information (NOTE) | O | Indicate the AI/ML intermediate information for intermidate AI/ML operation, including AI/ML intermediate model, AI/ML intermediate model used training time, used training resource, used traning number of iterations. |
| Time validity | O | The time validity of the request. |
| NOTE: The IE only present when AI/ML Task Type is ML model training. |  |  |

8.7.3	Information flows

8.7.3.1	AIMLE client registration request

Table 8.7.3.1-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration procedure.

Table 8.7.3.1-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.1-2. |
| List of supported services | O | Supported service information. |
| > List of service ID(s) | O | List of service ID(s) that the AIMLE client supports |
| > Corresponding service permission level(s) | O | Service permission level(s) (e.g., premium resource usage, standard resource usage, limited resource usage). |

Editor’s Note: The addition of AIMLE client profile ID is FFS.

Table 8.7.3.1-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client schedule | O | Indicates the availability schedule of the AIMLE client for the AIML service. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIML client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

* * * END of Change * * * *