3GPP TSG-SA WG6 Meeting #64	S6-245734

Orlando, USA, 18th – 22nd November 2024	(revision of S6-245269, 5426, 5691)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0230 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE-deployed API invoker accessing other UEs’ resources of a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2024-11-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TR 23.700-22 conclusion for KI#6, Solution#19 is being proposed for normative work There can be business relationships between group of UEs (e.g. in IoT use case) where one UE of the group is authorized to provide resource owner authorization for accessing the resources related to the group of UEs. In IoT use case, for the group of UEs (Smart phone, watch, ring) single GRO (Smart phone) is assigned as authorized for accessing the resources (e.g. location) related to the group of UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New procedure introduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature will not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new), 8.x.1 (new), 8.x.2 (new), 8.x.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.X	UE-deployed API invoker accessing other UEs’ resources of a group

### 8.x.1	General

This clause specifies the procedures for API invoker on a UE to be able to access the services exposing resources of other UEs of a group. The UEs belong to the same group and the same VAL service provider (IoT case) or the same user.

### 8.x.2	Information flows

Editor’s note:	The security aspects of this procedure are to be specified by SA3.

### 8.x.3		Procedure

Figure 8.x.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs.

Pre-condition:

1.	The network hosts individual resources for a group of UEs which can be accessed via AEF provided the necessary security conditions are met.

2.	UE 1 and UE 2 belong to the same group and the same VAL service provider (IoT case) or the same user.

3.	CCF is provisioned with the information of a Group Resource Owner (GRO) which is ROF at UE 1 who has been authorized (e.g. by business relationship between the group of UEs) to provide resource owner authorization for the group of UEs.

Figure 8.x.3-1: UE-deployed API invoker accessing other UEs’ resources

1.	The API invoker (e.g., in UE 2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API (and other UEs’ resources hosted in the network – e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. The request includes the UE identifier of UE 2, the group identifier, and optionally the list of UEs in a group whose resources are to be accessed. Additionally, the API invoker may include in the request information about the purpose and operations (scope) to be performed on the targeted resources.

2.	CCF performs authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122 [12].

3.	CCF checks if UE 2 belongs to the group and further resolves the identity of the GRO responsible for the group of UEs based on the provisioned information related to the GRO.

4.	If step 3 is successful, the CCF performs the resource owner authorization with the GRO.

Editor’s note:	The authorization details for step 4 of how CCF interacts with GRO to obtain authorization which also includes the group identification is FFS and will be specified by SA3.

5.	Based on the resource owner authorization received in step 4, CCF sends an Obtain service API authorization response to the API invoker.

6.	The service API invocation is performed by validating the authorization information received in the previous step.

Editor’s note:	How the AEF performs validation of the received authorization information is FFS and will be specified by SA3.