3GPP TSG-SA WG6 Meeting #65	S6-250031

Athens, Greece, 17th – 21st February 2025	(revision of S6-xxxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0002 | rev |  | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unified use of AIML enablement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are chaotic usages for AIML enablement which should be corrected - “AI/ML enablement” in clause 4.1, 8.21, C.3 - “AI/ML enabler” in clause C.1, C.2, C.3 - “AIML enabler” in clause 3.1, 8.11.3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Same as before. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Chaotic usages for AIML enablement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.1, 8.11.3, 8.21, C.1, C.2, C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

ML model: According to 3GPP TS 28.105 [8], mathematical algorithm that can be "trained" by data and human expert input as examples to replicate a decision an expert would make when provided that same information.

ML model lifecycle: The lifecycle of an ML model aka ML model operational workflow consists of a sequence of ML operations for a given ML task / job (such job can be an analytics task or a VAL automation task). This definition is aligned with the 3GPP definition on ML model lifecycle according to 3GPP TS 28.105 [8].

ML model training: According to 3GPP TS 28.105 [8], ML model training includes capabilities of an ML training function or service to take data, run it through an ML model, derive the associated loss and adjust the parameterization of that ML model based on the computed loss.

ML model inference: According to 3GPP TS 28.105 [8], ML model inference includes capabilities of an ML model inference function that employs an ML model and/or AI decision entity to conduct inference.

AI/ML intermediate model: For federated learning, members need to train models for multiple rounds, intermediate models indicate the model which do not meet the required training rounds and/or meet the requirements of the federation training.

AIMLE service: An AIMLE service is an AIMLE capability which aims assisting in performing or enabling one or more AIML operations.

AI/ML client: an application layer entity (also referred as ML client) which is an AI/ML endpoint, and performs client-side operations (e.g. related to the ML model lifecycle). Such AI/ML client can be a VAL client or AIML enablement client and may be configured e.g., to provide ML model training and inference locally e.g., at the VAL UE side.

AIMLE client set identifier: an identifier of the set of selected AIMLE clients.

AI/ML server: an application layer entity which is an AI/ML endpoint, and performs server-side operations (e.g. related to the ML model lifecycle). Such AI/ML server can be a VAL server or AIML enablement server.

FL member: An FL member or participant is an entity which has a role in the FL process. An FL member can be an FL client performing ML model training, or an FL server performing aggregation/collaboration for the FL process.

FL client: An FL member which locally trains the ML model as requested by the FL server. Such FL client functionality can be at the network (e.g. AIMLE server with FL client capability) or at the device side (e.g. AIMLE client with FL client capability).

FL server: An FL member which generates global ML model by aggregating local model information from FL clients.

Split AI/ML operation pipeline: A Split AI/ML operation pipeline is a workflow for ML model inference in which AI/ML endpoints are organized and collaborate to process ML models in sequential stages, where processing at each stage involves ML model inference on the output of the previous stage.

# * * * * Second change * * * *

4.1	General requirements

[AR-4.1-a]	The AIML enablement layer shall be able to support one or more VAL applications.

[AR-4.1-b]	Supported AIML enablement capabilities shall be offered as APIs to the VAL applications.

[AR-4.1-c]	The AIML enablement layer shall support interaction with 3GPP network system to consume network and AI/ML support services.

[AR-4.1-d]	The AIMLE client shall be capable to communicate with one or more AIMLE servers of the same AIMLE service provider.

# * * * * Third change * * * *

8.11.3	ML model information discovery

Figure 8.11.3-1 illustrates the procedure of ML model information discovery.

Pre-condition:

1.	Either AIMLE consumer has requested to discover ML model or AIMLE server decides to discover model for training.

Figure 8.11.3-1: ML model information discovery

1.	The AIMLE server sends an ML model information discovery request to the ML repository. The request contains information elements as described in Table 8.11.4.3-1.

2.	Upon receiving the ML model information discovery request, the ML repository verifies if the requestor is authorized to discover the ML model(s) identified by the ML model ID. Further, it verifies whether the requestor is present in the list of allowed vendors or not. If the ML repository consumer is authorized, the ML repository processes the request and discovers the model based on ML model ID, Analytics ID, Base Model ID, and/or ML model interoperability information.

If the discovery request is for transfer of learning, the ML repository identifies all candidate models matching the domain and required accuracy level.

After the AIMLE server receives the response from the ML repository, the AIMLE server determines whether the received ML model satisfy the ML model requirement of the AIMLE server and whether the AIMLE server needs to continuously train the model. The determination for continuous training is based on the AIMLE server capability for continuous training, its own requirements for the ML model and ML model profile (Indication of continuous model training, continuous training model parameter).

The ML repository sends the response message to the ML repository consumer. The response includes information of the discovered ML model.

NOTE:	When the ML model is within training or re-training phase, the ML model is not available for discovery. The details is indicated within the Result IE in Table 8.11.4.4-1. The training and re-training phase is used during federated learning so that AIML enablement consumer can rely on model repository to get the model details.

# * * * * Fourth change * * * *

8.21	ML model update

8.21.1	General

This clause provides the procedures to support ML model re-training and update when model performance degradation is observed by the AIML enablement layer. The model update procedure also supports using an existing model to re-train the model using Transfer Learning. Additionally, if the degraded model is related to other models due to e.g., Transfer Learning, the AIMLE server may trigger the update of those related models as well.

8.21.2	Procedure

Figure 8.21.2-1 depicts the procedure where the AIML enablement capability can trigger model update upon detecting model performance degradation.

Pre-conditions:

1.	The AIMLE Server has provided a ML model to the AIMLE Consumer. The AIMLE consumer can be a VAL server, AIMLE client, or ADAE server.

2.	The AIMLE Consumer detects a performance degradation of the ML model. If the consumer is an ADAE server, performance degradation may be detected as described in clause 8.17 in 3GPP TS 23.436 [4].

Figure 8.21.2-1: Support for ML model update

1.  The AIMLE Consumer sends an ML model update request to the AIMLE Server that includes the ID of the model and performance degradation information.

2.	Based on the performance degradation information, the AIMLE Server determines whether to update the ML model. If the AIMLE Server does not update the model, steps 3 and 4 are skipped.

3.	The AIMLE Server retrieves the ML model information from the ML Repository as described in clause 8.11.3. The AIMLE Server may also perform ML model discovery to determine whether an existing ML model stored by the ML Repository can be used to replace the degraded model or train the new model (e.g., using Transfer Learning). If an existing model can be used to replace the degraded model, step 4 is skipped, and the identified model is provided in step 5.

The AIMLE Server can also discover models that are related due to Transfer Learning or the use of the same training data, to identify additional models that may require an update.

4.	The AIMLE Server performs ML model re-training, which corresponds to the ML model training procedure as described in clause 8.3. The updated model is stored in the ML repository once re-training is complete.

If the degraded model is linked to other models (e.g., due to Transfer Learning, or the same training data has been used), the AIMLE Server may trigger the re-training and update of those related models.

5.	The AIMLE Server provides the updated ML model to the AIMLE Consumer either by sending it directly, or by providing endpoint information to retrieve it from the ML Repository.

8.21.3	Information flows

8.21.3.1	ML model update request

Table 8.21.3.1-1 details the ML model update request IEs.

Table 8.21.3.1-1: ML model update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | The identity of the AIMLE Consumer sending the request. |
| ML model ID | M | Provides the ID of ML model for which the performance degradation has been detected. |
| Performance degradation information | O | Provides details about the detected performance degradation, such as the time, instances, or information on the degraded metrics (e.g. accuracy, recall, F1score). |
| ML model retrieval endpoint | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |

8.21.3.1	ML model update response

Table 8.21.3.2-1 details the ML model update response IEs.

Table 8.21.3.2-1: ML model update response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | Provides the ID of the updated ML model. |
| Successful response | O (NOTE 1) | Indicates that the model has been updated. |
| > ML model | O (NOTE 2) | Provides the updated ML model. |
| > ML model retrieval endpoint | O (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model information | O | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| Failure response | O (NOTE 1) | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: At least one of these information elements shall be provided. |  |  |

# * * * * Fifth change * * * *

C.1	General

ML model lifecycle (aka ML model operation workflow) contains several parts including ML model training, ML model testing, AI/ML inference emulation, ML model deployment, AI/ML inference, Data Management, intermediate model aggregation, Trained AI/ML model delivery, and so on. The consumer can delegate all or part of ML model lifecycle to the AIML enablement layer. In doing so the AIML enablement layer can take over some AI/ML related work for the consumer and reduce the complexity of implementation for consumers. In the below sub-clauses, some options for the role of AIMLE in the ML model lifecycle are provided.

C.2	Role#1 of AIMLE in ML Model Lifecycle

In this scenario, the consumer completely rely on the AIML enablement layer for managing the ML operational workflow. That is, the consumer send the requirements of the AI/ML application, then the AIML enablement layer can perform ML operational workflows and determine the required AI/ML model to send to the consumer. Such role of AIMLE is captured in the capability related to AIML service operations control and management as described in clause 8.20.

Figure C.2-1: Example of Role#1

C.3	Role#2 of AIMLE in ML Model Lifecycle

In this scenario, consumer would like to perform part of the ML operational workflow, then consumer partially rely on the AIML enablement layer to assist with ML operational workflowss. Due to different ML operational workflow division, the AIML enablement layer can provide different levels of assistance on ML workflows for consumers.

Such capability is captured for example for ML model training (as in clause 8.3), HFL training (as in clause 8.12), model evaluation (which is captured in clause 8.19 in model capability evaluation, and 8.22 in model performance monitoring procedures). Figure C.3-1 illustrates an example where ML model training is handled by the AIMLE where the other operations are performed by the VAL / ASP.

Figure C.3-1: Example of Role#2

# * * * * End of changes * * * *