3GPP TSG-SA WG6 Meeting #65	S6-250384

Athens, Greece 17th – 21st February 2025	(revision of S6-250151)

Source:	China Mobile

Title:	pCR on clause 9 Deployment Guideline

Spec:	3GPP TS 23.392 v1.0.0

Agenda item:	9.13

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR is proposed to add the clause 9 Deployment Guideline.

2. Reason for Change

This pCR is proposed to add the clause 9 Deployment Guideline.

4. Proposal

This pCR is proposed to add the clause 9 Deployment Guideline.

* * * First Change * * * *

9	Deployment Guideline

9.1	General

This clause describes deployments of the functional model specified in clause 6.

9.2	MMTel Enabler server deployed in PLMN operator domain

The MMTel Enabler server is recommended to be deployed in the PLMN operator domain. Figure 9.2-1 illustrates the MMTel Enabler server deployed in PLMN operator domain.

Figure 9.2-1: MMTel Enabler server deployed in PLMN operator domain

The Application server can be deployed in PLMN operator domain or Application service provider domain and interacts with MMTel Enabler server via MMTel-2 reference point and MMTel-2e reference point respectively.The Application server deployed in PLMN operator domain, e.g. Application server 2 can only interact with the MMTel Enabler server deployed in the same PLMN operator domain, i.e. MMTel Enabler server 2. The Application server deployed in Application service provider domain, e.g. Application server 1, can interact with MMTel Enabler servers deployed in different PLMN operator domains, i.e. MMTel Enabler server 1 and MMTel Enabler server 2, based on business agreement between PLMN operator and Application service provider. In this case, the MMTel-2 reference point becomes MMTel-2e reference point.

The Controlling Application server can only be deployed in PLMN operator domain and interacts with MMTel Enabler server via MMTel-3 reference point. The Controlling Application server can only interact with the MMTel Enabler server deployed in the same PLMN operator domain.

NOTE:	In this release, the MMTel Enabler Server is deployed in the PLMN operator domain.