3GPP TSG-SA WG6 Meeting #65	S6-250402

Athens, Greece 17th – 21st February 2025	(revision of S6-250239)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0011 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Spatial anchor usage reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital, ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Clause 8.4 defines procedures for a SAn server to provide spatial anchor usage related information to a VAL server. The procedures specify that spatial anchor usage related information includes "spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information." Some of this information (e.g., number of times spatial anchor is accessed) can only be provided the a SAn consumer that discovered the spatial anchor. It is therefore needed to define an additional procedure to enable SAn consumers to provide spatial anchor usage information to the SAn server. The SAn server can then determine the spatial anchor usage related information to return to the VAL server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Spatial anchor usage reporting procedures and information flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to provide SA usage information remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.4.2.2	Procedure

Figure 8.4.2.2-1 depicts the procedure for subscribing to receive information related to spatial anchor usage for metaverse applications. The service is provided by SAn server.

Figure 8.4.2.2-1: Spatial anchor usage information subscription

1.	A consumer (e.g. VAL server) sends a spatial anchor usage information subscription request to the SA server). The request can include the VAL server identifier, security credentials, a notification endpoint (e.g., IP address, FQDN, URI), and spatial anchor filters like spatial anchor identifiers, spatial anchor location information, position of the user, proximity distance, and application service identifier. It may also include the periodicity for reporting notification and sampling interval.

2.	Upon receiving the request, the MMES authorizes the VAL server subscription request and sends a spatial anchor usage subscription response to the consumer.

3.	The SAn server samples the information related to spatial anchor usage related information based on the obtained data and the stored spatial anchor data as per the sampling interval. It provides response to the consumer as per the reporting interval. The response message includes spatial anchor ID, number of times spatial anchor discovered and accessed, spatial anchor density per location, user density per spatial anchor, spatial anchor services accessed information.

* * * Next Change * * * *

### 8.4.x	Spatial anchor usage reporting

#### 8.4.x.1	General

Spatial anchor usage reporting enables a spatial anchor consumer (e.g., SAn client or VAL server) to report spatial anchor usage to the SAn server.

#### 8.4.x.2	Procedure

Figure 8.4.x.2-1 illustrates the spatial anchor usage reporting procedure.

Pre-conditions:

1.	The SAn client or VAL server has received information (e.g. URI, IP address) related to the SAn server;

2.	The SAn client or VAL server has received security credentials authorizing it to communicate with the SAn server.

Figure 8.4.x.2-1: Spatial anchor usage reporting

1.	The requestor (e.g., SAn client or VAL server) sends a spatial anchor usage report request to the SAn server. The request includes a requestor identifier, security credentials, and spatial anchor usage information as described in clause 8.4.x.3.1.

2.	Upon receiving the request, the SAn server validates if the requestor is authorized for the request. If the requestor is authorized, the SAn server updates the spatial anchor usage information. The SAn server uses the spatial anchor usage information when notifying subscribers(s) as specified in clause 8.4.2.2.

3.	The SAn server sends a spatial anchor usage report response to the requestor. If the SAn server successfully stored the spatial anchor usage information, the response includes an indication of success. Otherwise, the response includes an indication of failure and can include a reason for failure.

#### 8.4.x.3	Information flows

##### 8.4.x.3.1	Spatial anchor usage report request

Table 8.4.x.3.1-1 describes information elements for the spatial anchor usage report request from the SAn client or VAL server to the SAn server.

Table 8.4.x.3.1-1: Spatial anchor usage report request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., VAL server or VAL UE and VAL user). |
| Security credentials | M | The security credentials of the requestor. |
| List of spatial anchor usage information | M | List of usage information for spatial anchors. |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Number of access | M | The number of times the spatial anchor was accessed. |
| > Report period | O | The time period for which the number of access applies. If omitted, the number of access is the number of access since the last spatial anchor usage report request. |

##### 8.4.x.3.2	Spatial anchor usage report response

Table 8.4.x.3.2-1 describes information elements for the spatial anchor usage report response from the SAn server to the SAn client or VAL server.

Table 8.4.x.3.2-1: Spatial anchor usage report response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor usage report request was successful. |
| Failure response | O | Indicates that the spatial anchor usage report request has failed. |
| > Cause | O | The cause for the request failure. |

* * * Next Change * * * *

### 8.5.1	General

Table 8.5.1-1 illustrates the SEAL APIs for spatial anchor management.

Table 8.5.1-1: List of SEAL APIs for spatial anchor management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_SAnManagement | Create | VAL server, SAn client | Request/Response |
|  | Update | VAL server, SAn client |  |
|  | Delete | VAL server, SAn client |  |
|  | Get | VAL server, SAn client |  |
|  | Subscribe | VAL server, SAn client | Subscribe/Notify |
|  | Notify | VAL server, SAn client |  |
|  | UpdateSubscription | VAL server, SAn client |  |
|  | Unsubscribe | VAL server, SAn client |  |
| SS_SAnDiscovery | Request | SAn client, VAL server | Request/Response |
| SS_SAnUsageInfo | Request | VAL server | Request/Response |
|  | Subscribe | VAL server | Subscribe/Notify |
|  | Notify | VAL server |  |
| SS_SAnUsageReport | Request | VAL server, SAn client | Request/Response |

* * * Next Change * * * *

### 8.5.y	SS_SAnUsageReport API

#### 8.5.y.1	General

Service description: This API enables a VAL server or SAn client to report spatial anchor usage information to the SEAL SAn server.

#### 8.5.y.2	Request operation

API operation name: SS_SAnUsageReport_Request

Description: The consumer provides spatial anchors usage information.

Inputs: See clause 8.4.x.3.1.

Outputs: See clause 8.4.x.3.2.

See clause 8.4.x for details of usage of this operation.

* * * End of Changes * * * *