3GPP TSG-SA WG6 Meeting #65	S6-250424

Athens, Greece 17th – 21st February 2025	(revision of S6-250256, merges 104 & 46)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0261 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Proposal for AEF instantiation support in CAPIF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Huawei, Hisilicon, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-22 proposed solution for AEF instantiation support with API management function interacting with a Mgmt System in Mgmt plane upon receiving triggers from CCF. In TS 23.222, clause 8.8 supports the functionality for a CAPIF’s AMF to consume events from CCF. Further, CAPIF architecture supports MNO’s AEF and 3rd party AEFs, which are managed by different technologies (e.g. SA5 specified Mgmt System, AWS). So, CAPIF should not limit the utilization of different Compute/Software mgmt systems. CAPIF’s API provider domain functions utilize the information of instantiated AEFs to manage service API information with the CCF. The decision for AEF instantiation is the responsibility of the Compute/Software Mgmt system. How API provider domain functions utilize the services of different mgmt system information is upto implementation. With the above perspectives, the following is proposed: a) Create events from CCF which can be consumed by AMF b) AMF to utilize services of different Compute/Software mgmt systems (based on implementation) |  |  |  |  |  |  |  |  |
| Summary of change: |  | For a) above, introducing new events - Service API related CAPIF-1 interaction events For b) above, add informative annex about utilization of different mgmt systems |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This feature will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.8.6	List of CAPIF events

Table 8.8.6-1 provides a non-exhaustive list of CAPIF events.

Table 8.8.6-1: List of CAPIF events

| Events | Events Description |
| --- | --- |
| Availability of service APIs | Availability events of service APIs (e.g. active, inactive) |
| Service API updated | Events related to change in service API information |
| Monitoring service API invocations | Events corresponding to service API invocations |
| API invoker status | Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.) |
| API topology hiding status | Events related to API topology hiding status in CAPIF (created, revoked) |
| System related events | Alarm events providing fault information |
| Performance related events | Events related to system load conditions |
| CAPIF-1/1e interaction events | Events related to CAPIF-1/1e interactions for a service API (e.g. service API discovered) |

* * * Next Change * * * *

Annex X (informative):
AEF instantiation

As illustrated in figure X-1, CAPIF architecture supports AEFs from multiple API providers (e.g. MNO, 3rd party). The instances of such AEFs are managed by the respective management systems of the API provider (e.g. PLMN management system, 3rd party management system).

The AMF monitors the events reported by the CAPIF core function. These events can be utilized to trigger AEF instantiation by the PLMN management system or 3rd party management system. The AMF can utilize the services of PLMN or 3rd party management systems (e.g. trigger instantiation of corresponding AEFs) via the APIs exposed by the PLMN or 3rd party management systems. It is upto implementation of CAPIF to utilize the services of the PLMN and/or 3rd party management systems for AEF instantiation.

Figure X-1: AEF instantiation