3GPP TSG-SA WG6 Meeting #65	S6-250425

Athens, Greece 17th – 21st February 2025	(revision of S6-250050)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0241 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of response elements in the CAPIF procedure in TS23.222 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-2-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This contribution adds details about the response elements in the CAPIF procedure. While the cause of failure is described clause 8.25, there is no mention of the cause prior to that section. So, this corrections were made. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When there is a failure in the response, the cause is being appended. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The response elements are inadequate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2, 8.6.2.2, 8.8.2.2, 8.8.2.6, 8.8.2.8, 8.15.2.2, 8.17.2.2, 8.19.2.2, 8.23.2.2, 8.25.2.2, 8.25.2.4, 8.25.2.6, 8.25.2.8, 8.25.2.10, 8.25.2.12, 8.25.2.15, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.5.2.2	Service API get response

Table 8.5.2.2-1 describes the information flow service API get response from the CAPIF core function to the API publishing function.

Table 8.5.2.2-1: Service API get response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of retrieving the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | The service API information as specified in Table 8.3.2.1-1. |
| NOTE: Shall be present if the Result information element indicates that the service API get request is successful. Otherwise service API information shall not be present. |  |  |

* * * Next Change * * * *

8.6.2.2	Service API update response

Table 8.6.2.2-1 describes the information flow service API update response from the CAPIF core function to the API publishing function.

Table 8.6.2.2-1: Service API update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

* * * Next Change * * * *

8.8.2.2	Event subscription response

Table 8.8.2.2-1 describes the information flow for event subscription response from the CAPIF core function to the subscribing entity.

Table 8.8.2.2-1: Event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation |
| Cause | O | The cause for the request failure. |
| Subscription identifier | O (see NOTE) | The unique identifier for the event subscription. |
| NOTE: Shall be present if the Result information element indicates that the event subscription operation is successful. Otherwise subscription identifier shall not be present. |  |  |

8.8.2.6	Event unsubscription response

Table 8.8.2.6-1 describes the information flow for event unsubscription response from the CAPIF core function to the subscribing entity.

Table 8.8.2.6-1: Event unsubscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event unsubscription operation |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.8.2.8	Event subscription update response

Table 8.8.2.8-1 describes the information flow for event subscription update response from the CAPIF core function to the subscribing entity.

Table 8.8.2.2-1: Event subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription update operation |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.15.2.2	Service API invocation response

The information flow service API invocation response from the AEF to the API invoker is service API specific and the complete detail of the service API invocation response is out of scope of the present document. Table 8.15.2.2-1 describes only the CAPIF related information elements which are included in the service API invocation response.

Table 8.15.2.2-1: Service API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.17.2.2	Service API invocation response

The information flow service API invocation response from the AEF to the API invoker is service API specific and the complete detail of the service API invocation response is out of scope of the present document. Table 8.17.2.2-1 describes only the CAPIF related information elements which are included in the service API invocation response.

Table 8.17.2.2-1: Service API invocation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.19.2.2	API invocation log response

Table 8.19.2.2-1 describes the information flow API invocation log response from the CAPIF core function to the API exposing function.

Table 8.19.2.2-1: API invocation log response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of API(s) invocation log request |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.23.2.2	Revoke API invoker authorization response

Table 8.23.2.2-1 describes the information flow revoke API invoker authorization response from the CAPIF core function to the API exposing function or from the API exposing function to the CAPIF core function.

Table 8.23.2.2-1: Revoke API invoker authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of revoke API invoker authorization. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.25.2.2	Interconnection API publish response

Table 8.25.2.2-1 describes the information flow interconnection API publish response from CAPIF core function to CAPIF core function.

Table 8.25.2.2-1: Interconnection API publish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of publishing the service API information |
| Cause | O | The cause for the request failure. |
| Service API published information reference | O (see NOTE) | The information which can be used for referencing the information (set) about the published service API by the CCF which publishes service APIs |
| NOTE: This information element is included when the Result indicates success. |  |  |

* * * Next Change * * * *

8.25.2.4	Interconnection service API discover response

Table 8.25.2.4-1 describes the information flow interconnection service API discover response from one CAPIF core function to another CAPIF core function.

Table 8.25.2.4-1: Interconnection service API discover response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the discovery of the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | List of service APIs corresponding to the request, including service API information as specified in Table 8.25.2.1-1. |
| CAPIF core function identity information | O (see NOTE) | Indicates the CAPIF core function matching the query criteria |
| NOTE: The service API information or the CAPIF core function identity information or both shall be present, if the Result information element indicates that the interconnection service API discover operation is successful. Otherwise, both shall not be present. |  |  |

* * * Next Change * * * *

8.25.2.6	Interconnection API unpublish response

Table 8.25.2.6-1 describes the information flow interconnection API unpublish response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.6-1: Interconnection API unpublish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of unpublishing the service API information |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.25.2.8	Interconnection get service API response

Table 8.25.2.8-1 describes the information flow interconnection get service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.8-1: Interconnection get service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of getting the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O (see NOTE) | The service API information includes the service API name, service API category, communication type, description, Serving Area Information (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional) , and Network Slice Info (optional). |
| NOTE: Shall be present if the Result information element indicates that the interconnection get service API request is successful. Otherwise, service API information shall not be present. |  |  |

* * * Next Change * * * *

8.25.2.10	Interconnection update service API response

Table 8.25.2.10-1 describes the information flow interconnection update service API response from a CAPIF core function to another CAPIF core function.

Table 8.25.2.10-1: Interconnection update service API response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the service API information |
| Cause | O | The cause for the request failure. |
| Service API information | O | The authorized service API information during update, applicable when the update result is success. |

* * * Next Change * * * *

8.25.2.12	Interconnection revoke API invoker authorization response

Table 8.25.2.12-1 describes the information flow interconnection revoke API invoker authorization response from one CCF to another CCF.

Table 8.25.2.12-1: Interconnection revoke API invoker authorization response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of interconnection revoke API invoker authorization. |
| Cause | O | The cause for the request failure. |

* * * Next Change * * * *

8.25.2.15	Interconnection obtain access control policy response

Table 8.25.2.15-1 describes the information flow interconnection obtain access control policy response from one CCF to another CCF.

Table 8.25.2.15-1: Interconnection obtain access control policy response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the obtain access control policy operation |
| Cause | O | The cause for the request failure. |
| Access control policy information | O (see NOTE) | The access control policy information corresponding to the requested service API. (See Table E-1) |
| NOTE: Shall be present if the Result information element indicates that the obtain access control policy operation is successful. Otherwise access control policy information shall not be present. |  |  |

* * * End of Changes * * * *