3GPP TSG-SA WG6 Meeting #65	S6-250430

Athens, Greece, 17th – 21st February 2025	(revision of S6-250209)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0251 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to Procedure for API invoker obtaining authorization from resource owner |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There is a mismatch in the description of the steps 1 and 2, being the description of the parameters included in step 1 developed only in step 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move the description of the request parameters to step 1. Indicate in step 2 that the parameters received in step 1 are included in the request to the ROF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear flows may lead to interpretation and implementation mistakes |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.31.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.31.3	Procedure

Figure 8.31.3-1 illustrates the procedure for API invoker obtaining authorization from resource owner.

Pre-conditions:

1.	The resource owner function (ROF) can communicate with the CCF (Authorization function).

2.	Access to an API exposing function offered service API requires obtaining authorization from a resource owner (RO).

Figure 8.31.3-1: Procedure for API invoker obtaining authorization from resource owner

1.	The API invoker requests to obtain RO authorization information to invoke the service API exposed by the API exposing function (AEF) and to access information owned by the resource owner at the AEF through the invocation of an obtain service API authorization request to the CCF. The request contains API invoker information (its identity and optionally application service information such as application service provider and application identifier), the purpose for data processing and any information required for authentication of the API invoker. The request may include resource owner-related finer level service API access requirements (e.g., access per service API operation or access per API service resource) and resource owner-related information the API invoker intends to provide to the AEF at service invocation (i.e., service API data model information elements the API invoker seeks authorization to provide to the AEF).

2.	The CCF validates the authentication of the API invoker (using authentication information). Then the authorization function determines the authorization by interacting with the RO via the ROF, in the case that authorization information is not available. The request to the ROF contains the request parameters described in step 1.

NOTE 1:	The detailed procedure to obtain the RO's authorization information is specified in TS 33.122 [12].

NOTE 2:	Step 1 can occur when the API invoker receives a failure in Service API invocation response indicating that authorization information from resource owner(s) is required.

Editor's Note:	Authorization for finer level service API access and information intended to be provided by the API invoker at invocation, whilst accounting for the provided purpose for data processing, is to be detailed by SA3.

3.	Based on the RO provided authorization, the CCF sends an obtain service API authorization response to the API invoker.

4.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 1.

5.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information.

* * * END of Change * * * *