3GPP TSG-SA WG6 Meeting #65	S6-250431

Athens, Greece, 17th – 21st February 2025	(revision of S6-250210)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0252 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to UE-access of other UEs’ resources of a group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following text needs to be clarified in 8.34: - To be able to authenticate the API Invoker of UE2, all the users of the group belong to the same CAPIF provider domain. - It is unclear how the CCF determines that the RO authorization is going to be provided by a GRO. - The GRO authorization information may already be available when the CCF needs to check it, if it is not available the Authorization information capture procedure from GRO is triggered. - Whether the UE identity behind the API invoker is included in the Obtain Service API authorization. - The information included in the authorization response and then presented to the AEF. For Rel-19 it is proposed to specify two variants of the solution, based on: - The control of whether the UEs belongs to the group is kept at application level. Or - The control of the UEs that belong to the group is ket within CAPIF provider domain. In addition: - UE2 and UE1 belong to the same CAPIF provider domain. - The authorization token contains resource owner permissions to access to resource owner’s resources. - The changes of membership in the group triggers the GRO revoke the provided authorization. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following clarifications are provided: - All users of the group belong to the same CAPIF provider domain. - The control of whether the UE behind the API invoker belongs to the group can be kept at application level or at CCF level. - The CCF determines based on the received group identifier whether the RO authorization is obtained from a GRO. - GRO authorization information may be prepovisioned. If not available, the CCF triggers the procedure to obtain it from the GRO. - The authorization information provided by the GRO indicates the authorization to access to resource owner’s resource(s). The AEF enforces the received authorization information. - The procedure is to access to the resources of another UE (and not of a list of UEs) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and uncomplete procedure for the UE access to resources of other UEs that belong to the same group. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.34.1, 8.34.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.34.1	General

This clause specifies the procedures for API invoker on a UE to be able to access the services exposing resources of other UEs of a group. The UEs belong to the same group and the same VAL service provider (IoT case) or the same user and all of them belong to the same CAPIF provider domain.

The following scenarios are supported:

1.	Obtaining resource owner authorization information by the API invoker when the authorization relationship between UE2 (the API invoker is in UE2) and UE1 is given by the membership to the same group, where the group is within the application control. The difference with the procedure described in clause 8.31 is that the API invoker is deployed in a UE different to the UE whose resources are being accessed, the API invoker needs to provide in the request, in addition to the identity of the UE whose resources are being accessed, the group identifier. and the CAPIF provider domain needs to be configured with the identifier of the Group Resource Owner, which is responsible of providing authorization information on behalf of the UEs that belong to the group.

2.	Obtaining resource owner authorization information by the API invoker when the authorization relationship between UE2 (the API invoker is in UE2) and UE1 is given by the membership to the same group, where the group information is within the CAPIF provider domain. The difference with the procedure described in clause 8.31, in addition to the differences described in bullet 1, is that the identity of the UE2 needs to be included in the request, and the group membership needs to be managed in the CAPIF provider domain.

* * * Next Change * * * *

### 8.34.3		Procedure

Figure 8.34.3-1 presents the procedure for enabling a UE-hosted API invoker accessing network-hosted resources owned by other UEs that belong to the same group.

Pre-condition:

1.	The network hosts individual resources for a group of UEs which can be accessed via AEF provided the necessary security conditions are met.

2.	UE 1 and UE 2 belong to the same group and the same VAL service provider (IoT case) or the same user.

3.	CCF is provisioned with the information of a Group Resource Owner (GRO) which is ROF at UE 1 who has been authorized (e.g. by business relationship between the group of UEs) to provide resource owner authorization for the group of UEs.

v

Figure 8.34.3-1: UE-deployed API invoker accessing other UEs’ resources of a group

1.	The API invoker (e.g., in UE 2) sends an Obtain service API authorization request to the CCF for obtaining permission to access the service API for other UE’s resources hosted in the network (e.g., location) by including the API invoker identity information and any information required for authentication of the API invoker. If the control of the authorization relationship between UEs of the group is only in the application domain. the request includes the group identifier, and the UE in a group whose resources are to be accessed. If the control of the authorization relationship between the UEs member of a group is in the CCF, then request shall include the identity of UE2. Additionally, the API invoker may include in the request information about the purpose and operations (scope) to be performed on the targeted resources.

2.	CCF performs authentication of the API invoker (using authentication information) as specified in 3GPP TS 33.122 [12].

3.	CCF, based on the group identifer determines that RO authorization is provided by a GRO. The CCF then resolves the identity of the GRO responsible for the group of UEs based on the provisioned information related to the GRO. If the UE2 identity was received in step 1, the CCF shall additionally check whether UE2 belongs to the group.

4.	If step 3 is successful, the CCF performs the resource owner authorization with the GRO.

Editor’s note:	The authorization details for step 4 of how CCF interacts with GRO to obtain authorization which also includes the group identification is FFS and will be specified by SA3.

5.	Based on the group resource owner authorization received in step 4, the CCF sends an Obtain service API authorization response to the API invoker. The authorization response indicates that the access to UE (resource owner) network resource(s) is authorized for an indicated scope.

NOTE:	In this release of the specification, the authorization revokation due to the removal of any of the UE(s) from the group is triggered by the GRO.

6.	The API invoker sends service API invocation request to the API exposing function with the RO authorization information received in step 2.

7.	The API invoker receives the service API invocation response resulting from the service API invocation once the API exposing function has checked whether the API invoker is authorized to invoke that service API based on the RO authorization information

* * * END of Change * * * *