3GPP TSG-SA6 Meeting #65	S6-250440

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025                    (revision of  S6-250140)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0135 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multi-Modal traffic indication in SEALDD layer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile M2M Company Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For the support of Multi-Modal traffic indication in SEALDD layer, multi-modal service ID can be added in AF request. The multi-modal service ID is an identifier of the multi-modal service as defined in clause 6.1.3.27.3 of 3GPP TS 23.503. Data flows belonging to the same multi-modal service share the same Multi-modal Service ID. Besides list of requested flows information can be included in the Sdd_XRTransmission request from VAL server to the SEALDD server. In step 2 of clause 9.2.2.2 in 23.433, the SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for one traffic flow. For XR service, there are multiple traffic flows, the QoS can be different for each traffic flow, the QoS information for each requested flow can be sent to 5GC. The list of requested flows information is included in step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add support for multi-modal service ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification of multi-modal SEALDD flow is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x, 9.12.2.1.2, 9.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 8	Identities and commonly used values

## 8.1	General

The common identities for SEAL refer to 3GPP TS 23.434 [4]. The following clauses list the additional identities and commonly used values for SEALDD.

## 8.2	SEALDD server ID

The SEALDD server ID uniquely identifies the SEAL data delivery server.

## 8.3	SEALDD client ID

The SEALDD client ID is a globally unique value that identifies the SEAL data delivery client.

## 8.4	SEALDD-UU flow ID

The SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to identify different VAL application traffic, which has the same attributes, e.g. the same 5-tuple, media type, QoS requirements. The SEALDD-UU flow ID should be uniquely identified with the SEALDD-UU connection.

## 8.5	Multi-modal SEALDD-UU flow ID

The multi-modal SEALDD-UU flow ID is used by the SEALDD client and SEALDD server to uniquely identify associated multi-modal VAL application traffic flows per VAL service.

## 8.x	Multi-modal service ID

The Multi-modal Service ID is an identifier of the multi-modal service as defined in clause 6.1.3.27.3 of 3GPP TS 23.503 [7].  Data flows belonging to the same multi-modal service share the same Multi-modal Service ID.

* * * Next Change * * * *

##### 9.12.2.1.2	SEALDD enabled XR data transmission establishment

Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the XR application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion.

Pre-condition:

-	The VAL server can discover and select the SEALDD server by CAPIF functions.

Figure 9.12.2.1.2-1: SEALDD enabled XR data transmission connection establishment procedure

1.	The VAL server decides to use SEALDD service for XR traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_XRTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID,  list of requested flows, including SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements.

2.	Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. The SEALDD server may send the AF request to provide the required QoS information to 5GC via N33/N5 for each requested flow provided in step 1. The AF request may also include the Multi-modal Service ID, which could be determined by the SEALDD server according to VAL service ID and/or VAL server ID.

3-5.	Same as step 3-5 of clause 9.2.2.2.

6.	The SEALDD client establishes XR transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the XR transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates.

7-8.	Same as step 8-9 of clause 9.2.2.2.

NOTE 1:	If multiple requested flows are provided in step 1, the SEALDD client repeats steps 6-8 for each requested flow provided in step 1.

The XR application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

* * * Next Change * * * *

#### 9.12.3.1	SEALDD enabled XR transmission request

Table 9.12.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting XR transmission service.

Table 9.12.3.1-1: SEALDD enabled XR transmission request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of specific UE or VAL user |
| List of requested flows | M | List of the requested multi-modal flows |
| >SEALDD-S Data transmission connection information for multi-modal XR | M | Address(s)/port(s) and/or URL(s) of the VAL server to receive the multi-flow packets from the SEALDD server (e.g. audio flow, tactile flow) |
| >QoS information | O | QoS information provided by VAL server |
| >Protocol description | O | The protocol description of VAL traffic. It includes header extension information (e.g. RTP extension with PDU set), packetization indication, payload type and format (e.g. H.264/RTP, H.265/RTP, H.264, H.265). Header extension information is only applicable when payload indicates RTP. |

* * * End of Changes * * * *