3GPP TSG-SA WG6 Meeting #65	S6-250454

Athens, Greece 17th – 21st February 2025	(revision of S6-250059)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0007 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Consistent VAL Service ID usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As the VAL layer a VAL service ID is provided and this is sufficient for the AIMLE service, i.e., there is no need to define a separate AIMLE service ID. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ensure consistent use of VAL service ID throughout the GS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent use of identifiers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.1, 8.10.3.1, 8.12.2, 8.12.3.2, 8.12.3.3 & 8.22.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev1, “supports” to “support” in Table 8.8.3.1-2 and “VAL identifier” to “VAL service identifier” in Table 8.22.3.1-1 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.3.1	AIMLE client discovery request

Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure.

Table 8.8.3.1-1: AIMLE client discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| AIMLE client discovery criteria | M | Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2. |
| Number of required AIMLE clients | O | Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria. |

Table 8.8.3.1-2: AIMLE client discovery criteria

| Information element | Status | Description |
| --- | --- | --- |
| Service requirement | M | Information about the required service |
| > VAL Service ID | M | VAL Service ID that the client is required to support. This identifies the service associated with the requester. |
| > Service permission level | O | Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage). |
| Requested ML model types | O | Requested ML model types (decision trees, linear regression, neural networks). |
| Requested AIML operations | M | Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split. |
| Application layer AIMLE client capabilities | M | Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate). |
| Dataset requirements | O | Information about dataset. |
| > Dataset availability | O | Dataset availability, including dataset identifiers, dataset size, age, list of dataset features. |
| > Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions. |
| AIML client task capability requirements | O | Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs) |
| AIMLE client velocity | O | Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static. |
| Location information | O | Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID) to discover the AIMLE clients. |
| AIMLE client QoS requirements | O | Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients. |

Editor’s Note: Whether target area for data collection is included in the AIMLE client discovery criteria is FFS.

* * * Second Change * * * *

#### 8.10.3.1	AIMLE client participation request

Table 8.10.3.1-1 shows the request sent by the AIMLE server to each AIMLE client selected for AIMLE client participation procedure.

Table 8.10.3.1-1: AIMLE client participation request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor. |
| AIMLE client set identifier | M | An identifier for the AIMLE client set. |
| Add/remove indicator | M | Indicator for adding/removing the AIMLE client to/from the AIMLE client set. |
| AI/ML model ID | M | The AI/ML model ID to use for the AI/ML operation. |
| AI/ML operations | M | A list of AI/ML operations (e.g., training) required to be performed. |
| Operational schedule | O | Schedule for the AI/ML operations. |
| Dataset requirement | M | Dataset requirements for the AI/ML operations. Requirements includes dataset identifier, dataset size and age, and/or dataset features. |
| Service requirement | M | Information about the required Service including its VAL service ID and its service permission level (e.g., premium resource usage standard resource usage, limited resource usage) |

* * * Third Change * * * *

### 8.12.2	Procedure

Pre-conditions:

1.	AIMLE client discovery and selection have been performed.

2.	Datasets are available and prepared for AI/ML training at the AIMLE clients. The datasets are assigned identifiers for use in HFL training.

Figure 8.12.2-1: HFL training

1.	An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2.

2.	The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1.

3.	For the HFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training.

4.	Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the HFL training as described in the clause 8.10.

5.	The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1.

6.	Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 8.

7.	Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule.

8.	Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an VAL service ID for the HFL training operation.

9.	If errors were encountered, the procedure skips to step 10.

If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients.

If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 8 are repeated for the next training round.

When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository.

10.	The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

* * * Fourth Change * * * *

#### 8.12.3.2	HFL training subscription response

Table 8.12.3.2-1 shows the response sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.2-1: HFL training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the subscription request: success, fail. |
| Subscription ID | O | An identifier for the subscription only if the status is success. |
| VAL service ID | M | The VAL service identifier for the AIMLE HFL training operation. |

* * * Fifth Change * * * *

#### 8.12.3.3	HFL training subscription notification

Table 8.12.3.3-1 shows the notification sent by AIMLE clients to the AIMLE server for the HFL training subscription procedure.

Table 8.12.3.3-1: HFL training subscription notification

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | Status for the request: success, fail. |
| VAL service ID | M | The VAL service identifier for the AIMLE HFL training operation. |
| HFL training output | M | ML model parameters from HFL training. |
| Errors list | O | A list of errors encountered during a HFL training round. |
| Timestamp | O | A timestamp for the notification. |

* * * Sixth Change * * * *

#### 8.22.3.1	ML model performance monitoring subscription request

Table 8.22.3.1-1 shows the request sent by a VAL server to an AIMLE server for the ML model performance monitoring.

Table 8.22.3.1-1: ML model performance monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identifier of the requestor (e.g., VAL server). |
| ML model identifier | M | The identifier of the ML model for which the monitoring applies. |
| Notification endpoint | M | The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to. |
| AIML operation information | O | The AIMLE operation (ML model training, HFL, VFL, TL) for which the ML model is used. |
| > VAL service ID | O | The VAL service identifier of the AIMLE service using the ML model (if known by the requestor). |
| > AIMLE client ID(s) | O | The identifier(s) of the AIMLE client(s) training the ML model (if known by the requestor). |
| > AIMLE service KPI | O | One or more KPIs for the AIMLE service performance (latency, accuracy, etc). |
| Monitoring report configuration | M | The reporting configuration for the monitoring service (thresholds for triggering a monitoring event, e.g. minimum accuracy, delay, whether the reporting is one time or periodical or event-based). |
| Area of interest | O | The geographical or service area for which the monitoring applies. |
| Time validity | O | The time validity for the monitoring subscription. |
| Trigger Action requirement | O | This requirement identifies policies for triggering an action based on a monitoring event (e.g. if degradation is detected, to train a new model or re-selecting AIMLE clients). |

* * * End of Changes * * * *