3GPP TSG-SA WG6 Meeting #65	S6-250455

Athens, Greece 17th – 21st February 2025	(revision of S6-250060)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0008 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to AIMLE client registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the AIMLE client registration request and update request, the VAL services IDs and service permissions are provided as lists within the List of supported services and profiles. This creates a list within list, which isn’t necessary and creates confusion with regards to the relation between each VAL service ID and service permission level. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Restructure the AIMLE client registration request to remove the list within list and correct the attribute status. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Erroneous data type specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.3.2 & 8.7.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | In rev1, reverted attributes to optional in Table 8.7.3.4-1, removed “can be provided” from the description of List of supported services in Table 8.7.3.4-1 and set VAL service ID to mandatory (which applies if “List of supported services” is included). |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.7.3.2	AIMLE client registration request

Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request.

Table 8.7.3.2-1: AIMLE client registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of supported profiles | M | Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided. |
| > AIMLE client profile | M | Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2. |
| > List of supported services | M | List of VAL service IDs with corresponding permissions. |
| >> VAL service ID | M | The identifier of the VAL service. |
| >> Service permission level | O | Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage). |

Table 8.7.3.2-2: AIMLE client profile

| Information element | Status | Description |
| --- | --- | --- |
| Supported AI/ML model types | O | AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks). |
| Supported AI/ML operations | M | AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split). |
| AIMLE client time schedule configurations | O | Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations in the given time slot(s) and/or day(s) of the week. |
| AIMLE client location configurations | O | Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID. |
| AIMLE client capabilities | M | AIMLE client capability information (e.g. ML application type, allowed resource usage level). |
| Dataset availability | O | Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers. |
| Data capabilities | O | A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions. |
| AIMLE client task capability | O See NOTE | Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs) |
| NOTE: The Green and Energy-efficient task performance may not be applicable to a UE. |  |  |

* * * Second Change * * * *

#### 8.7.3.4	AIMLE client registration update request

Table 8.7.3.4-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update request.

Table 8.7.3.4-1: AIMLE client registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | Identifier of the existing registration for which the update request applies. |
| AIMLE client profile | O | Update information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.2-2. |
| List of supported services | O | Update to supported service information. Services identified by their VAL Service ID are either removed from the list when the AIMLE client ceases to provide a service and added when the AIMLE client starts to provide a service. |
| > VAL service ID | M | The identifier of the VAL service. |
| > Service permission level | O | Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage). |

* * * End of Changes * * * *