3GPP TSG-SA WG6 Meeting #65	S6-250544

Athens, Greece, 17th – 21st February 2025	(revision of S6-250434)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0256 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the ROF and Authorization Function responsibilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | CAPIF_Ph3 |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of the CAPIF Authorization Function and the Resource Owner Function are incomplete and have ambiguities, unclear understanding of the scope of their responsibilities. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarifications of the Authorization function responsibilities, consistency alignment. Clarification of the ROF, which is sometimes perceived as just a front end that helps the user provide consent, and sometimes is a back-end with more functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete solution that does not work as specified, ambiguity of the roles and responsibilities of Authorization function and the ROF |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2.3, 6.3.3, 6.3.7, 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 5.2	CAPIF business relationships for RNAA

Figure 5.2-1 shows the CAPIF business relationships for the resource owner-aware northbound API access (RNAA).

Figure 5.2-1: CAPIF business relationships for RNAA

The business relationships the API invoker, the CAPIF provider, and the API provider follow the description in the clause 5.1. In addition to them, the resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider. The API invoker and the resource owner can be the same entity or separate entities. In the current release, the resource owner is a user of a UE and can provide authorization information using the UE.

The CAPIF provider and the API provider can belong to the same organization (e.g. PLMN operator), in which case the service API arrangement is not required explicitly.

The CAPIF provider and the API provider (e.g. 3rd party CAPIF provider or 3rd party API provider) can belong to different organizations, in which case the service API arrangement is required. The specification of service API arrangement is out of scope of 3GPP.

* * * Next Change * * * *

### 6.2.3	Functional model description to support RNAA

Figure 6.2.3-1 shows the architectural model for the RNAA which allows the resource owner to provide authorization to access their resources through Service API invocation.

Figure 6.2.3-1: High level functional architecture for CAPIF supporting RNAA

The authorization function is an internal entity of the CAPIF core function and performs API invoker authorization based on the API invoker authorization policies available in the CCF and the RO authorization information obtained via the ROF.

The resource owner function interacts with the authorization function in the CAPIF core function via CAPIF-8. The authorization function in the CAPIF core function interacts with the resource owner function to obtain the resource owner authorization. The CAPIF core function (authorization function)  uses resource owner authorization information obtained via the ROF as part of the processing of the authorizaiton requests received from API invokers.

The API exposing function (e.g. NEF, SCEF) interacts with the authorization function in the CAPIF core function via CAPIF-3. The API exposing function acts as an authorization enforcement point according to the authorization token received from the API invoker.

NOTE 1:	RNAA is supported for both 4G and 5G networks.

The API invoker interacts with authorization function in the CAPIF core function via CAPIF-1/CAPIF-1e.

NOTE 2:	In the current release, 3rd party API providers (i.e., API providers outside the PLMN trust domain) are not supported for RNAA.

NOTE 3:	The interaction between resource owner function and CCF over CAPIF-8 is not specified in the current release of the specification.

NOTE 4:	The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122 [12]) is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501 [16]). In the current release of 3GPP specifications, no synergy between CCF and UDM is specified.

The security aspects of CAPIF supporting RNAA are specified in 3GPP TS 33.122 [12].

* * * Next Change * * * *

### 6.3.3	CAPIF core function

The CAPIF core function consists of the following capabilities:

-	Authenticating the API invoker based on the identity and other information required for authentication of the API invoker;

-	Supporting mutual authentication with the API invoker;

-

-	Publishing, storing and supporting the discovery of service APIs information;

-	Controlling the service API access based on PLMN operator configured policies;

-	Storing the logs for the service API invocations and providing the service API invocation logs to authorized entities;

-	Charging based on the logs of the service API invocations;

-	Monitoring the service API invocations;

-	Onboarding a new API invoker and offboarding an API invoker;

-	Storing policy configurations related to CAPIF and service APIs;

-	Support accessing the logs for auditing (e.g. detecting abuse);

-	Supports publishing, retrieving, unpublishing, updating,and discovering service APIs information with another CAPIF core function in CAPIF interconnection; and

-	Supports slice related API exposure in, e.g., API publish, API discovery, API invoker authorization, API access contr

* * * Next Change * * * *

### 6.3.7	Authorization function

The authorization function has the following capabilities:

-	Acts in an authorization server role as specified in clause 1.1 of IETF RFC 6749 [14],

-	Verifies authorization information for service APIs, upon request from AEF (when the AEF does not have information required to authorize the service API invocation),

-	Uses the RO authorization information (such as user consent) obtained via ROF, when legally required, as part of the authorization of the API invoker to access the RO resources related to a service API.

NOTE 1:	In the current release, the authorization function is an internal entity of the CAPIF core function.

NOTE 2: 	The persistent storage of the resource owner authorization data is subject to local/regional regulations and is not in scope of this specification.

* * * Next Change * * * *

## 7.5	RNAA deployments

CAPIF supports RNAA and has enabled API invoker(s) to have authorized access to resources of a resource owner provided by service APIs offered by the AEF. The CCF acts as the Authorization Function and supports the authentication and authorization of API invoker(s). After checking the resource owner authorization information (e.g., user consent) via the ROF, the CCF (Authorization function) provides the access token for a service API access to the API invoker. The API invoker performs service API invocations on the AEF by utilizing the access token.

The API invoker may be deployed in the following ways:

a.	API invoker may be deployed as AF on the UE (i.e. 3rd party application).

b.	API invoker may be deployed as AF on the UE supporting several other 3rd party applications deployed on the UE.

c.	API invoker may be deployed on the network as AF.

The resource owner is connected via a UE and can use a Resource Owner Function on the UE to interact with the CCF acting as the Authorization Function for to grant RO permission to the API invoker to access resource(s) of the resource owner provided by the service API. The CCF (Authorization Function) uses the RO authorization information, as part of API invoker authorization process.

NOTE:	The details of the protocol for CAPIF supporting RNAA is specified in 3GPP TS 33.122 [12].

When API invoker is deployed on a UE (cases a and b), the API invoker is allowed to access the resources of the resource owner corresponding to the UE.

* * * END of Change * * * *