3GPP TSG-SA WG6 Meeting #65	S6-250572

Athens, Greece 17th – 21st February 2025	(revision of S6-250483, S6-250245)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0141 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Client initiated data transmission |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | SEALDD specification TS 23.433 in the clause for SEALDD services over Satellite Access, currently there is support only for SEALDD enabled data transmission for delay-tolerant satellite services and the support for client initiated data transmission for delay-tolerant satellite services is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding procedure for client initiated data transmission for delay-tolerant satellite services based on the existing clause 9.2.2.7 Client initiated regular data transmission path establishment procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service may be impacted when delivering via satellite access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New{10.2.2.x, 10.2.3.x}, 2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 10.2.2.x	Client initiated data transmission for delay-tolerant satellite services

Figure 10.2.2.x-1 illustrates the procedure for establishing S&F data transmission connection, and the SEALDD facilitates the delay-tolerant satellite services to transmit data initiated by the VAL client to VAL server.

Pre-condition:

-	The VAL client is aware of the availability of delay-tolerant satellite services to transmit data.

Figure 10.2.2.x-1: Client initiated data transmission for delay-tolerant satellite services

1.	The VAL client decides to use SEALDD service for delay-tolerant traffic transfer.

2.	Same as steps 2-4 of clause 9.2.2.7.

3.	The SEALDD client provides the acknowledgment for the request made in step 1.

4.	The SEALDD client sends the UL data to the SEALDD server via the 3GPP network.

5.	The SEALDD server subscribes the recipient UE S&F status events from the 3GPP CN as specified in clause 5.6.3.10 of TS 23.682 [x] and is notified about the recipient UE S&F status events information e.g., whether UE is registered in S&F Mode, the estimated delivery time indicating the DL estimated time to send data from the gateway to the recipient UE. The SEALDD server stores the received recipient UE S&F status events information for further control the downlink application data transmission.

Editor's note:	How SEALDD server knows the recipient UE information is FFS.

6.	The SEALDD server controls the downlink application data delivery to the SEALDD client based on the UE S&F status events information, when the recipient UE is in S&F mode and receives the DL estimated delivery time is received from 3GPP CN. If the recipient UE is not available or not registered to S&F mode, then the SEALDD server handles the DL data as per the service policy.

7.	The SEALDD server sends the Sdd_S&FTransmission_notification to the SEALDD client and VAL client including the DL estimated delivery time of the recipient UE.

NOTE 1:	VAL client can use the DL estimated delivery time of the recipient UE to control the UL application data delivery.

8.	The SEALDD server sends the DL data to the VAL server.

NOTE 2:	Details of step 1, step 3 and step 7 are out of scope of the current specification.

* * * Next Change * * * *

#### 10.2.3.x	Sdd_S&FTransmission_notification

Table 10.2.3.x-1 describes the information flow from the SEALDD server to the SEALDD Client to notify the DL estimated delivery time.

Table 10.2.3.x-1: Sdd_S&FTransmission_notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service. |
| Identity | M | Identifier of specific UE or VAL user |
| Identity | M | Identifier of specific recipient UE or VAL user |
| > DL estimated delivery time | M | DL estimated delivery time for a recipient UE |

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[3]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[7]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[8]	3GPP TS 23.548: "5G System Enhancements for Edge Computing ".

[9]	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[10]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[11]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics (MDA)".

[12]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[13]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[14]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[15]	3GPP TS 26.506: "5G Real-time Media Communication Architecture (Stage 2)".

[16]	3GPP TS 23.303: "Proximity-based Services (ProSe); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[18]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[19]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[20]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[21]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[22]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service".

[x]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

* * * End of Changes * * * *