3GPP TSG-SA WG6 Meeting #65	S6-250594

Athens, Greece, 17th – 21st February 2025	(revision of S6-250587)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0728 | rev | 4 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Solving ENs on Service provisioning considering EES onboard |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following Editor’s Notes need to be solved: Editor's Note: Whether and how the Satellite Connectivity Information is required is FFS. Satellite ID is not available in NR. In NR the UE is aware of the satellite ephemeris, which is associated with a PCI (Physical Cell Identifier), which changes constantly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Indicate that the UE can use the satellite Id information, only if available, i.e., if received from gNB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1, 11.1.2, 11.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 11.1.1	EES determination based on UE serving satellite ID Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.

When UE connects to the satellite, the service provisioning request message contains the UE serving Satellite ID if available (i.e., serving gNB corresponding Satellite ID, if received). If the service provisioning request message contains the UE serving Satellite ID, then the ECS determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

When UE connects to the satellite, and the service provisioning request message does not contain the UE serving Satellite ID, then service provisioning response message may contain the list of EES along with the EES Satellite ID, and then the EEC determine EES where the Satellite ID in the EES profile matches the UE serving Satellite ID, if available in the UE.

* * * Next Change * * * *

### 11.1.2	EES determination based on UE location and route Procedure

This procedure is applicable for EDN deployment with gNB on satellite.

A satellite EDN may be deployed as described in Annex A.2.5. The service provisioning procedure for satellite EDN is the same as clause 8.3.3.2.2 and clause 8.3.3.2.3 with the following clarifications.When EEC sends the service request message to the ECS and the request message contains the UE location, predicted/expected UE location, Prediction expiration time, then the ECS determines a list of EES where the Satellite coverage availability information in EES profile matches the UE location, predicted/expected UE location, Prediction expiration time, and EES Satellite coverage availability information. Alternatively, the ECS may query external server with the UE current location and UE predicted/expected location to obtain a list of Satellite IDs and Trajectory IDs corresponding the UE location and the UE trajectory, then the ECS determines list of EES based on the Satellite IDs and Trajectory IDs in EES profiles by matching with the Satellite IDs and Trajectory IDs obtained from external server for the UE. The “Expected AC Geographical Service Area” in the AC profile, as specified in clause 8.2.2, is used by the ECS to request the external server (managed by the satellite service provider or operator) for such satellite IDs and trajectory IDs. When ECS sends the service provisioning response which contains a list of EES information which EES is deployed on MEO or LEO satellites, the EEC determines the EES where the Satellite ID in the EES profile matches the UE serving Satellite ID.

In addition, when a serving satellite is providing EES and EAS, and due to discontinuous coverage the same satellite comes back for serving, then the EEC connects to same EES and EAS (i.e. to which the EEC had previously connected to), without needing to discover EES and EAS endpoints again.

Editor's Note: Whether and how the ACR procedure need to be enhanced is FFS.

* * * Next Change * * * *

#### 11.1.2.2	Service provisioning request

Table 11.1.2.2-1 is same as clause 8.3.3.3.2 Table 8.3.3.3.2-1 with the following additions.

Table 11.1.2.2-1: Service provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| IEs from Table 8.3.3.3.2-1 |  |  |
| UE serving Satellite ID (NOTE z) | O | The identifier of the satellite which is serving UE in current location. |
| UE location (NOTE x) | O | The location information of the UE. The UE location is described in clause 7.3.2, or Present UE location (e.g. latitude and longitude) for a reference grid point. |
| NOTE x: This IE is not an additional IE but update an existing IE in Table 8.3.3.3.2-1. NOTE z: The UE serving Satellite ID are available to EEC if the UE has received it from satellite access. |  |  |

Editor's Note: Whether and how the Satellite Connectivity Information is required is FFS.

* * * END of Change * * * *