3GPP TSG-SA WG6 Meeting #66	S6-251072

Göteborg, Sweden, 07th – 11th April 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0032 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove Security Credential |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-03-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The definition of security credential is unclear in the step descriptions and information flows. Any security credential(s) for authorization and verification for the request from the consumer as well as other security mechanims are in the remit of SA3. This paper proposes to remove “security credential(s)” in information flows and add general description on security aspects. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Removed “security credential(s) in information flows. - Added general description for security aspects and SA3 reference to clause 8.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is EN left not resolved. The definition of security credential is unclear, it is in the remit of SA3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 8, 8.1, 8.2.3.1, 8.2.3.3, 8.2.3.6, 8.2.3.8, 8.4.4.2, 8.4.4.4, 8.5.5.2, 8.11.2.1, 8.11.4.1, 8.14.3.2, 8.14.3.4, 8.14.3.6, 8.14.3.8, 8.14.3.10, 8.14.3.12, 8.14.3.15, 8.14.3.17, 8.14.3.19, 8.14.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[7]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[8]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[9]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 22.261: "Service requirements for the 5G system".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[13]	3GPP TS.23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[xx]	3GPP TS.33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

* * * Next Change * * * *

# 8	Procedures and information flows

## 8.1	General

Security mechanims are in the remit of SA3. The security aspects for the AIMLE services shall be aligned to 3GPP TS 33.434 [xx].

* * * Next Change * * * *

#### 8.2.3.1	ML model retrieval request

Table 8.2.3.1-1 describes information elements for the ML model retrieval request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.1-1: ML model retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Next Change * * * *

#### 8.2.3.3	ML model retrieval subscribe request

Table 8.2.3.3-1 describes information elements for the ML model retrieval subscription request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.3-1: ML model retrieval subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Notification Target Address | M | Notification target address (e.g. URL) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Next Change * * * *

#### 8.2.3.6	ML model retrieval subscription update request

Table 8.2.3.6-1 describes information elements for the ML model retrieval subscription update request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.6-1: ML model retrieval subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated. |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP address) where the notifications should be sent. |
| Proposed expiration time | O | Proposed expiration time for the subscription. |
| Filtering criteria | O | Represents the filtering criteria, which can be any of the ML model information as in Table 8.11.4.1-2. |

* * * Next Change * * * *

#### 8.2.3.8	ML model retrieval unsubscribe request

Table 8.2.3.8-1 describes information elements for the ML model retrieval unsubscribe request from the AIMLE client or VAL server to the AIMLE server.

Table 8.2.3.8-1: ML model retrieval unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor (e.g., AIMLE client or VAL server). |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be unsubscribed. |

* * * Next Change * * * *

#### 8.4.4.2	FL member registration request

Table 8.4.4.2-1 describes information elements for the FL member registration request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.2-1: FL member registration request

| Information element | Status | Description |
| --- | --- | --- |
| FL member ID | M | The identifier of the candidate FL member. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| FL member supported AI/ML role | M | The supported AI/ML role of FL member which can be used as FL server or FL client. |
| FL member capabilities | O (NOTE) | FL member capability information (e.g. ML application type, allowed resource usage level). |
| FL member velocity | O | Indicates the FL member velocity (e.g., mobile or static). |
| FL member Location information | O | Indicates the location information of the UE (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses). |
| FL member Availability schedule | O (NOTE) | Availability schedule of the FL member for a certain FL member. |
| Supported ML model ID list | O | The list of ML model IDs for which the FL member can be used. |
| Area of Interest | O | The area of interest (i.e. location co-ordinates) for which the registration applies. |
| Time validity | O | The time validity for the registration of the FL member. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.4.4.4	FL member registration update request

Table 8.4.4.4-1 describes information elements for the FL member registration update request from the candidate FL member (VAL server, AIMLE server) to the ML repository/registry.

Table 8.4.4.4-1: FL member registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration ID | M | Identifier of the existing registration for which the update request applies. |
| FL member ID | M | The identifier of the candidate FL member for which the update applies. This can be the VAL server ID, the EAS ID, the AIMLE server ID based on which entity is the candidate FL member. |
| Updated registration parameters | M | The updates of the registration parameters. |
| > FL member supported AI/ML role | O (see NOTE) | The updated supported AI/ML role of FL member which can be FL server, FL client. |
| > FL member Availability schedule | O (see NOTE) | Updated Availability schedule of the FL member. |
| > supported ML model ID list | O (see NOTE) | The updated list of ML model IDs for which the FL member can be used |
| > area of interest | O (see NOTE) | The updated area of interest for the registration update. |
| > time validity | O (see NOTE) | The update time validity for the FL member registration update. |
| NOTE: At least one of these information elements shall be present. |  |  |

* * * Next Change * * * *

#### 8.5.5.2	FL-related event subscription request

Table 8.5.5.2-1 describes information elements for FL-related event subscription request from the subscriber (AIMLE server, or VAL server (via AIMLE server)) to the ML repository.

Table 8.5.5.2-1: FL-related event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | The identifier to determine the identity of the requestor entity. This can be either the AIMLE server ID, or AIMLE server ID and VAL server ID. |
| Time of validity | M | The time validity for the subscription. |
| ML model information | M | Information of the ML model for, specified in Table 8.11.4.1-2, which the subscription is needed. |
| FL member information | M | Information on the FL member (candidate or selected). |
| >FL member type | O (NOTE) | The type of FL member which can be an FL server or FL client. |
| >FL member ID | O (NOTE) | The ID of the entity serving as FL member. This can be used for events where the FL member is known and possibly selected, and the availability needs to be checked. |
| Event Information | M | Information on the FL-related event. |
| > Event ID | O | The identity of the FL-related event (if known by the requestor). |
| > Event type | M | The event type requested, based on the Definition of FL related events (shown in Table 8.5.4-1). |
| Notification endpoint | M | The information of the endpoint for receiving the notifications for the event. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.11.2.1	AIMLE server-initiated ML model information storage

Figure 8.11.2.1-1 illustrates the procedure of the AIMLE server-initiated ML model information storage.

Pre-condition:

1.	The AIMLE server has either received application specific model details from AIML consumer or produced analytics model.

Figure 8.11.2.1-1: AIMLE server-initiated ML model information storage

1.	The AIMLE server sends a ML model information storage request to the ML repository to store an ML model. The ML model included in the request can be one trained by the ML repository consumer or its information can be provisioned to the ML repository consumer. The request contains information elements as described in Table 8.11.4.1-1. The request message may contain Indication of continuous training and continuous training model parameter.

NOTE:	The security requirements within the ML model storage requirements are out of scope.

2.	Upon receiving the ML model information storage request, the ML repository verifies if the AIMLE server is authorized to store the ML model identified by ADAE Analytics ID and/or list of the allowed vendors provided within the ML model information attribute. If the AIMLE server is authorized, the ML repository processes the request and records information of the ML model (e.g., by creating a ML model profile). The ML repository sends an ML model information storage response to the AIMLE server with an identifier of the created ML model profile.

* * * Next Change * * * *

#### 8.11.4.1	ML model information storage request

Table 8.11.4.1-1 describes the information flow from the AIMLE server to the ML repository or from the AIMLE client/VAL server to the AIMLE server as a request for the ML model information storage.

Table 8.11.4.1-1: ML model information storage request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identity | M | The identity of the requestor performing the request. |
| Security Credentials | O (NOTE 1) | Security credentials of the requestor performing the request. |
| ML model | O (NOTE 2) | The ML model to be stored in the ML repository. |
| ML model address | O (NOTE 2) | Represents the ML model address that can be used by Model repository to download the ML model. |
| ML model information | M | Provides information of the ML model, as described in Table 8.11.4.1-2. |
| NOTE 1: This information is needed if the requestor is at the VAL service provider domain. NOTE 2: At least one of these information elements shall be provided. |  |  |

Table 8.11.4.1-2: ML model information

| Information element | Status | Description |
| --- | --- | --- |
| ML model identifier | M | An identifier for the ML model |
| ADAE Analytics ID | O | Represents ADAE analytics ID for which the model can be used. |
| ML model size | O | Indicates the size of the ML model. |
| ML model source identifier | O | The identifier of ML model source (e.g., VAL server ID, VAL client ID) that stored the model in the ML repository. |
| VAL service ID(s) | O | Identify the VAL service ID(s). |
| Domain | O | Specifies domain for which the model can be used (e.g., for speech recognition, image recognition, video processing, location prediction, etc.). |
| List of allowed vendors | O (NOTE 1) | Indicates which vendors that are allowed to use the ML model and thereby also are interoperable to the model. |
| ML model interoperability information | O (NOTE 1) | Represents the vendor-specific information that conveys, e.g., requested model file format, model execution environment, input/output parameters of the ML model, etc. The encoding, format, and value of ML Model Interoperable Information is not specified since it is vendor specific information, and is agreed between vendors, if necessary for sharing purposes. |
| ML Model phase | O (NOTE 1) | Represents the ML model phase, e.g., in training, trained, re-training, deployed. |
| > Observed performance | O (NOTE 2) | Provides information on the performance of the model e.g. accuracy, or application-specific performance metrics (if ML model is in trained or deployment phase). |
| > Training information | O (NOTE 2) | If the ML model is in trained or deployed phase: Information on the data that has been used to train the model (e.g. data sources, volume, freshness), and the base model ID in case of Transfer Learning. |
| > Indication of continuous model training | O | Indicates whether the model can be continuously trained or not. |
| > Continuous model training parameter | O | Parameters required for continuous model training. |
| ML model storage and discovery requirements | O (NOTE 1) | Represents the requirements for the ML repository for the ML model storage and discovery. |
| > Storage duration | O | Represents the ML model storage duration time. When the storage duration time is expired, the stored ML model and the related information shall be deleted. |
| > Access requirements | O | Represents ML model access requirements (e.g., publicly available, private use only, or available for the list of VAL server IDs or VAL client IDs, time period and location access). If the access requirement is private use only, then the model is not discoverable by other consumers. |
| ML model usage requirements | O | Represents the requirements for using the ML model (e.g. for inference or for training). The requirements are used by the AIMLE server to determine whether an AIMLE client is capable of using the model based on comparing the requirements with information in the AIMLE client profile in Table 8.7.3.1-2. |
| NOTE 1: At least one of these information elements shall be provided. NOTE 2: This IE is included only if trained ML model is available. |  |  |

* * * Next Change * * * *

#### 8.14.3.2	Split operation pipeline discovery request

Table 8.14.3.2-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline discovery request.

Table 8.14.3.2-1: Split operation pipeline discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier) |
| Split operation discovery filters | M | Split operation discovery filters. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.) |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.) |
| > number of nodes | O | Minimum number of nodes required to support AIML operation splitting |

* * * Next Change * * * *

#### 8.14.3.4	Split operation pipeline create request

Table 8.14.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline create request.

Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the tail node. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

* * * Next Change * * * *

#### 8.14.3.6	Split operation node register request

Table 8.14.3.6-1 shows the request sent by a VAL server to the AIMLE server for split operation node register request.

Table 8.14.3.6-1: Split operation node register request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Node information | M | Information about the VAL server node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | M | Split operation capabilities of the VAL server. |
| > Model information | M | Information about ML model capabilities of the VAL server for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

* * * Next Change * * * *

#### 8.14.3.8	Split operation node registration update request

Table 8.14.3.8-1 shows the request sent by a VAL server to the AIMLE server for split operation node registration update request.

Table 8.14.3.8-1: Split operation node registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |
| Node information | O | Information about the VAL server node (e.g., identifier, endpoints, etc.). |
| Split operation capabilities | O | Split operation capabilities of the VAL server. |
| > Model information | O | Information about ML model capabilities of the VAL server for split operation (e.g., identifiers, versions, etc.). |
| > Usage information | O | Information about usage capabilities of the VAL server for split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Expiration time | O | The proposed expiration time of the registration. |

* * * Next Change * * * *

#### 8.14.3.10	Split operation node de-register request

Table 8.14.3.10-1 shows the request sent by a VAL server to the AIMLE server for split operation de-register request.

Table 8.14.3.10-1: Split operation node de-register request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the registration. |

* * * Next Change * * * *

#### 8.14.3.12	Split operation subscribe request

Table 8.14.3.12-1 shows the request sent by an AIMLE client or a VAL server to the AIMLE server for split operation subscribe request.

Table 8.14.3.12-1: Split operation subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Split operation pipeline identifier | M | Identifier of the split operation pipeline for which the requestor is subscribing. |
| Notification endpoint | M | The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to. |
| Event identifier | M | The event identifier for the subscription: - Split operation pipeline information - Split operation assistance information |
| Discovery filters | O | The set of characteristics to determine matching split operation profiles or nodes (as detailed in Table 8.14.3.2-1). Applicable for "Split operation pipeline information". |
| Assistance information | O | The assistance information for the subscription: - Aggregate the collected assistance information from NEF, NWDAF, and/or ADAES to generate assistance information, e.g. Time (time point(s) or time window(s)) to deliver the task or data for the split operation. - Perform inference using ML model to generate assistance information, e.g. achievable QoS with current configuration for task or data delivery, or suggestion of QoS for task or data delivery. Applicable for "Split operation assistance information". |
| Expiration time | O | The proposed expiration time of the subscription. |

* * * Next Change * * * *

#### 8.14.3.15	Split operation subscribe update request

Table 8.14.3.15-1 shows the request sent by an AIMLE client or a VAL server to the AIMLE server for split operation subscription update request.

Table 8.14.3.15-1: Split operation subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Notification endpoint | O | The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to. |
| Event identifier | O | The event identifier for the subscription: - Split operation pipeline information - Split operation assistance information |
| Split operation discovery filters | O | The set of characteristics to determine matching split operation profiles or nodes (as detailed in Table 8.14.3.2-1). Applicable for "Split operation availability" event. |
| Expiration time | O | The proposed expiration time of the subscription. |

* * * Next Change * * * *

#### 8.14.3.17	Split operation unsubscribe request

Table 8.14.3.17-1 shows the request sent by an AIMLE client or a VAL server to the AIMLE server for split operation unsubscribe request.

Table 8.14.3.17-1: Split operation unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |

* * * Next Change * * * *

#### 8.14.3.19	Split operation pipeline update request

Table 8.14.3.19-1 shows the request sent by an AIMLE client to the AIMLE server for split operation pipeline update request.

Table 8.14.3.19-1: Split operation pipeline update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Split operation pipeline identifier | M | The identifier of the AI/ML split operation pipeline. |
| Split operation pipeline information | M | Split operation pipeline information to be updated; this may include information elements defined in the split operation profile in Table 8.14.3.3-2. |

* * * Next Change * * * *

#### 8.14.3.21	Split operation pipeline delete request

Table 8.14.3.21-1 shows the request sent by an AIMLE client to the AIMLE server for split operation pipeline delete request.

Table 8.14.3.21-1: Split operation pipeline delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Split operation pipeline identifier | M | The identifier of the AI/ML split operation pipeline. |

* * * END of Changes * * * *