3GPP TSG-SA WG6 Meeting #66	S6-251073

Göteborg, Sweden, 07th – 11th April 2025	(revision of S6-25xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.482 | CR | 0033 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Information Flows for Responses |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The information flows for responses contains either Successful response or Failure response. A NOTE, for example “One of the IEs shall be present”, is needed to make the present of Successful response and Failure response clear. However, such NOTE is missing in some of the service responses. This paper proposes to add NOTE for these service responses to make it clear that either Successful response or Failure shall be present. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added NOTE to service responses to make it clear that either Successful response or Failure shall be present. - Corrected errors. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s unclear of the present of Successful response and Failure response in some of the service responses. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.3.2, 8.2.3.4, 8.2.3.7, 8.2.3.8, 8.7.3.3, 8.7.3.5, 8.7.3.7, 8.13.3.3, 8.13.3.6, 8.13.3.8, 8.14.3.3, 8.14.3.5, 8.14.3.7, 8.14.3.9, 8.14.3.11, 8.14.3.13, 8.14.3.16, 8.14.3.18, 8.21.3.1, 8.22.3.2, 8.24.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.2.3.2	ML model retrieval response

Table 8.2.3.2-1 describes information elements for the ML model retrieval response from the AIMLE server.

Table 8.2.3.2-1: ML model retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the ML model retrieval request was successful. |
| >List of ML models | M | List of retrieved ML models; the nested information elements are provided for each ML model in the list. |
| >> ML model identifier | M | The ML model identifier. |
| >> ML model retrieval endpoint (NOTE 1) | O | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| >> ML model (NOTE 1) | O | The ML model containing a retrieved ML model. |
| Failure response | O (NOTE 2) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE 1: At least one of these information elements is needed. NOTE 2: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.2.3.4	ML model retrieval subscription response

Table 8.2.3.4-1 describes information elements for the ML model retrieval subscription response from the AIMLE server.

Table 8.2.3.4-1: ML model retrieval subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.2.3.7	ML model retrieval subscription update response

Table 8.2.3.7-1 describes information elements for the ML model retrieval subscription update response from the AIMLE server.

Table 8.2.3.7-1: ML model retrieval subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.2.3.9	ML model retrieval unsubscribe response

Table 8.2.3.9-1 describes information elements for the ML model retrieval unsubscribe response from the AIMLE server.

Table 8.2.3.9-1: ML model retrieval unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model retrieval unsubscribe request was successful. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.7.3.3	AIMLE client registration response

Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.3-1: AIMLE client registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration request failed. |
| > Cause | O | Indicates the cause of registration request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.7.3.5	AIMLE client registration update response

Table 8.7.3.5-1 shows the AIMLE client de-registration update response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.5-1: AIMLE client registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update request failed. |
| > Cause | O | Indicates the cause of registration update request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.7.3.7	AIMLE client de-registration response

Table 8.7.3.7-1 shows the AIMLE client de-registration response sent by the AIMLE server to the AIMLE client.

Table 8.7.3.7-1: AIMLE client de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration request was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration request failed. |
| > Cause | O | Indicates the cause of de-registration request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.13.3.3	AIMLE client selection subscription response

Table 8.13.3.3-1 shows the response sent by the AIMLE server to the VAL server for the selected AIMLE client selection subscription response.

Table 8.13.3.3-1: AIMLE client selection subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the subscription was successful. |
| > Subscription ID | M | The identifier of the subscription. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription status, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated subscription never expires. |
| Failure response | O (NOTE) | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.13.3.6	AIMLE client selection subscription update response

Table 8.13.3.6-1 shows the AIMLE client selection subscription update response sent by the AIMLE server to the VAL server.

Table 8.13.3.6-1: AIMLE client selection subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription status, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated subscription never expires. |
| Failure response | O (NOTE) | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.13.3.8	AIMLE client selection unsubscribe response

Table 8.13.3.8-1 shows the AIMLE client selection unsubscribe response sent by the AIMLE server to the VAL server.

Table 8.13.3.8-1: AIMLE client selection unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the unsubscribe request was successful. |
| Failure response | O (NOTE) | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.3	Split operation pipeline discovery response

Table 8.14.3.3-1 shows the response sent by sent by the AIMLE server after processing of split operation pipeline discovery request.

Table 8.14.3.3-1: Split operation pipeline discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the request was successful. |
| > List of nodes (NOTE 1) | O | The list of discovered nodes. |
| > List of split operation profiles (NOTE 1) | O | The list of split operation profiles as specified in Table 8.14.3.3-2. |
| Failure response | O (NOTE 2) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE 1: Only one of the IE shall be present for successful response. NOTE 2: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.5	Split operation pipeline create response

Table 8.14.3.5-1 shows the response sent by a AIMLE server after processing of split operation pipeline create request.

Table 8.14.3.5-1: Split operation pipeline create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > Split operation profile | O | The split operation profile as specified in Table 8.14.3.3-2. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.7	Split operation node register response

Table 8.14.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request.

Table 8.14.3.7-1: Split operation node register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > Registration identifier | M | The identifier of the registration. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.9	Split operation node registration update response

Table 8.14.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request.

Table 8.14.3.9-1: Split operation node registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > Expiration time | O | The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.11	Split operation node de-register response

Table 8.14.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request.

Table 8.14.3.11-1: Split operation de-register response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.13	Split operation subscription response

Table 8.14.3.13-1 shows the response sent by an AIMLE server after processing of split operation subscription request.

Table 8.14.3.13-1: Split operation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > Subscription identifier | M | The identifier of the subscription. |
| > Expiration time | O | The expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the subscription never expires. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.16	Split operation subscribe update response

Table 8.14.3.16-1 shows the request sent by an AIMLE server after processing of split operation subscription update request.

Table 8.14.3.16-1: Split operation subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > Expiration time | O | The expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the subscription never expires. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.14.3.18	Split operation unsubscribe response

Table 8.14.3.18-1 shows the response sent by an AIMLE server after processing of split operation unsubscribe request.

Table 8.14.3.18-1: Split operation unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.21.3.1	ML model update response

Table 8.21.3.2-1 details the ML model update response IEs.

Table 8.21.3.2-1: ML model update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 1) | Indicates that the model has been updated. |
| > ML model | O (NOTE 2) | Provides the updated ML model. |
| > ML model retrieval endpoint | O (NOTE 2) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| > ML model information | O | Provides information of the ML model, specified in Table 8.11.4.1-2. |
| Failure response | O (NOTE 1) | Indicates that the request has failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE 1: One of these information elements shall be provided. NOTE 2: At least one of these information elements shall be provided. |  |  |

* * * Next Change * * * *

#### 8.22.3.2	ML model performance monitoring subscription response

Table 8.22.3.2-1 shows the response sent by the AIMLE server to the VAL server for the ML model performance monitoring subscription.

Table 8.22.3.2-1: ML model performance monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model performance monitoring request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of these IEs shall be present. |  |  |

* * * Next Change * * * *

#### 8.24.3.2	AIMLE context transfer response

Table 8.24.3.2-1 shows the AIMLE context transfer response that is sent by the target edge AIMLE server to the source edge AIMLE server.

Table 8.24.3.2-1: AIMLE context transfer response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | Indicates the cause of request failure |
| NOTE: One of the IEs shall be present. |  |  |

* * * END of Changes * * * *