3GPP TSG-SA WG6 Meeting #66	S6-251144

Gothenburg, Sweden, 7th – 11th April 2025	(revision of S6-245xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0150 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support 5G MBS in MC gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon,China BroadNet |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2025-03-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In SA6#66, the CR0624 was agreed to support MBMS in MBMS gateway UE. However, 5MBS is used to deliver the one-to-many group communication. Some concepts and terms are changed. So a new clause of supporting 5MBS for MC gateway UE should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new clauses to support the 5MBS for MC gateway UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5MBS is not supported by MC gateway UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {7.x, 7.x.1, 7.x.2, 7.x.2.1, 7.x.2.2, 7.x.2.3, 7.x.2.4, 7.x.2.5, 7.x.2.6, 7.x.2.7, 7.x.2.8, 7.x.3, 7.x.3.1, , 7.x.3.2 , 7.x.3.3, 7.x.3.4, 7.x.3.5} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /**************************** First changes **********************/

7.x	5MBS support for MC clients residing on non-3GPP devices

7.x.1	General

This subclause addresses the MBS support for the MC clients residing on the non-3GPP devices associated with an MC gateway UE. The MC clients instruct the MC gateway UE about the corresponding MBS session details to enable MC gateway listening on them. On demand, MC gateway UE forwards the traffic received over MBS session to the MC clients residing on non-3GPP devices. With the procedure defined in this subclause MBS session can be supported for the MC clients residing on non-3GPP devices. Changes required are confined to the reference point between MC gateway UE and the MC clients residing on non-3GPP devices. MC service server may consider the location of the corresponding MC clients as defined in subclause 11.5.2 while deciding to establish MBS session.

7.x.2	Information flows

7.x.2.1	MC GW MBS session announcement

Table 11.5.3.2.1-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MBS sessionMBS session announcement received by the MC Client from the MC Service server.

Table 11.5.3.2.1-1: MC GW MBS session announcement

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the requesting MC client. |
| MBS session ID | M | MBS session ID information |
| MBS session mode | M | Indicate the service type of the MBS session, either a multicast MBS session or a broadcast MBS session |
| Multicast MBS session related information (see NOTE 1) | O | Additional information to be used by the MC service client to join the multicast MBS session such as PLMN ID of the default PLMN service provider, NID (for the case of SNPN deployments) in case of source specific IP multicast address, DNN, and SNSSAI of the PDU session associated with the multicast MBS session |
| List of MBS Service Area information (see NOTE 2) | O | For the case of local MBS services, it indicates either multicast service area identifier(s) for multicast MBS session, or broadcast service area identifier(s) for broadcast MBS session |
| Frequency | O | Identification of frequency if multi carrier support is provided |
| >MBS Frequency Selection Area ID (MBS FSA ID) (see NOTE 3) | O | The frequency associated to a certain broadcast area, if multi carrier support is provided |
| SDP information | M | SDP with media and floor control information applicable to groups that can use this MBS session (e.g. codec, protocol id, FEC information) |
| Monitoring state | O | The monitoring state is used to control if the client is actively monitoring the MBS session quality or not. |
| >MBS session announcement acknowledgement | O | Indicate if the MC service server requires an acknowledgement to the MBS session announcement |
| ROHC information | O | Indicate the usage of ROHC and provide the parameters of the ROHC channel to signal to the ROHC decoder. |
| NOTE 1: Such information may be pre-configured in the MC gateway UE, or provided in any other implementation specific way NOTE 2: Details of MBS service area information is defined in 3GPP TS 23.247 [15]. NOTE 3: It is applicable for broadcast MBS session. |  |  |

7.x.2.2	MC GW MBS listening status report

Table 11.5.3.2.2-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW MBS listening status report.

Table 11.5.3.2.2-1: MC GW MBS listening status report

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | MBS session ID(s) information. |
| MBS listening status(s) | M | The MBS listening status per MBS session ID. |
| MBS reception quality level | O | The reception quality level per MBS session ID |
| Non 3GPP transport resources establishment parameters (see NOTE) | O | This element contains the details of the non-3GPP transport resources establishment parameters (IP address, Port etc.) which are used by the MC gateway UE to forward the MC service communication traffic received over 3GPP MBS session to the MC client. |
| NOTE: These parameters are implementation specific and are dependent on the non 3GPP transport mechanism used between the MC client and MC gateway UE. This parameter can be present mandatorily if the MBS session listening status is success. |  |  |

7.x.2.3	MC GW MapGroupToSessionStream request

Table 11.5.3.2.3-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of MapGroupToSessionStream message received from the MC service server.

Table 11.5.3.2.3-1: MC GW MapGroupToSessionStream request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MCPTT group, in which the call is started. |
| Media stream identifier | M | This element identifies the media stream of the SDP used for the group call. |
| MBS session ID | M | The MBS session identifier. |

7.x.2.4	MC GW MapGroupToSessionStream response

Table 11.5.3.2.4-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW MapGroupToSessionStream response.

Table 11.5.3.2.4-1: MC GW MapGroupToSessionStream response

| Information element | Status | Description |
| --- | --- | --- |
| MapGroupFromSessionStream Status | M | Success or failure response |
| Non 3GPP transport resources establishment parameters (see NOTE) | M | This element contains the details of the non 3GPP transport resources establishment parameters(IP address, Port etc.,) which are used by the MC gateway UE to forward the MC service Group communication traffic received over 3GPP MBS session to the MC client. |
| NOTE: These parameters are implementation specific and are dependent on the non-3GPP transport mechanism used between the MC client and MC gateway UE |  |  |

7.x.2.5	MC GW MBS session quality report

Table 11.5.3.2.5-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW MBS session quality report.

Table 11.5.3.2.5-1: MC GW MBS session quality report

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | MBS session ID(s) information. |
| MBS listening status(s) | M | The MBS listening status per MBS session ID. |
| MBS reception quality level | O | The reception quality level per MBS session ID |

7.x.2.6	MC GW MBS session suspension indication

Table 11.5.3.2.6-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW MBS session suspension indication.

Table 11.5.3.2.6-1: MC GW MBS session suspension indication

| Information element | Status | Description |
| --- | --- | --- |
| MBS session ID(s) | M | MBS session ID(s) information. |
| suspension status(s) | M | The suspension status per MBS session ID. |

7.x.2.7	MC GW UnMapGroupFromSessionStream request

Table 11.5.3.2.7-1 describes the information flow from the MC client which resides on a non-3GPP device to the MC gateway UE for sharing the details of UnMapGroupFromSessionStream message received from the MC service server.

Table 11.5.3.2.7-1: MC GW UnMapGroupFromSessionStream request

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the MC service user. |
| MCPTT group ID | M | This element identifies the MC service group related to a group call to be dissociated over the MBS session. |
| Media stream identifier | M | This element identifies the media stream of the SDP, which is no longer used for the group call within the MBS session. |
| MBS session ID | M | The MBS session identifier. |

7.x.2.8	MC GW UnMapGroupFromSessionStream response

Table 11.5.3.2.8-1 describes the information flow from the MC gateway UE to the MC client which resides on a non-3GPP device for the MC GW UnMapGroupFromSessionStream response.

Table 11.5.3.2.8-1: MC GW UnMapGroupFromSessionStream response

| Information element | Status | Description |
| --- | --- | --- |
| UnMapGroupFromSessionStream Status | M | Success or failure response |

7.x.3	Procedures

7.x.3.1	Procedure for handling MBS session announcement

MC clients residing on non-3GPP devices receive MBS session announcements from the MC system, MC clients share the details of the MBS session received in MBS session announcement to the MC gateway UE. This enables the MC gateway UE to start monitoring the corresponding MBS session.

Figure 11.5.3.3.1-1 illustrates the procedure for handling the MBS session announcement by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters to enable the use of the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.1-1: Handling of MBS session announcement

1.	The MC service server establishes the MBS session(s) according to the procedures defined in 3GPP TS 23.468 [18]. Service description associated with the MBS session(s) is returned from the BM-SC.

2.	The MC service server provides service description information associated with the MBS session to the MC client residing on non-3GPP devices via MC gateway UE.

3.	The MC client sends the MC GW MBS session announcement to the MC Gateway UE containing the MBS session related information received from the MC service server.

4.	The MC gateway UE stores the information associated with the MBS session ID (s). The MC gateway UE uses the MBS session ID and other MBS session related information to monitor the MBS session ID (i.e., TMGI) or join the MBS session if the MBS session is an multicast MBS session.

5.	The MC gateway UE that enters or is in the service area of at least one announced MBS session ID notifies to the MC client that it can receive data over MBS by sending the MC GW MBS listening status report. The MC GW MBS listening status report also contains the details of the non-3GPP transport resources related parameters. The MC gateway UE may choose to send the details of existing transport resources information as part of non-3GPP transport resources establishment parameters IE if existing communication channel can be reused.

6.	The MC client establishes the transport resources with the MC gateway UE based on the parameters received in step 5 to receive the MC service data from the MC gateway UE, if these parameters are not referring to any of the already established transport resources. The MC Gateway UE forwards the MC service data it received over the MBS session from the MC service server to the MC client over this transport resources.

7.	The MC client sends the MBS Listening Status Report to the MC service server indicating that it is able to receive the media over MBS.

7.x.3.2	Procedure for handling MapGroupFromSessionStream message

Whenever the MC client detects that traffic received from MC service server is MapGroupFromSessionStream message and if the MC client participates in the group session or communication identified by the MapGroupFromSessionStream message then it should inform the details contained in the MapGroupFromSessionStream message to MC gateway UE. When the association of group call, MBS session and the MC service ID of the MC client is known to the MC gateway UE, it can forward the traffic received over MBS session accordingly.

Figure 11.5.3.3.2-1 illustrates the procedure for handling the MapGroupFromSessionStream message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2-1: Handling of MapGroupFromSessionStream message

1.	The MC service server sends a MapGroupFromSessionStream message over a previously activated MBS session to all users that will receive the call over an MBS session. The MapGroupFromSessionStream message includes association information between the group call and MBS session. The MapGroupFromSessionStream message includes MC service group ID and information about the media stream identifier of the activated MBS session and may include the identifier (i.e. the MBS session ID) of the MBS session broadcasting the call.

2.	If the MC client is participating in the MC group communication identified by the MapGroupFromSessionStream message, it sends the details contained in the MapGroupFromSessionStream message to the MC gateway UE through MC GW MapGroupFromSessionStream request message.

3.	The MC gateway UE on receiving the MC GW MapGroupFromSessionStream Request message from the MC client it maintains the association between the MC Service ID and the corresponding MBS session.

4.	The MC gateway UE sends the MC GW MapGroupFromSessionStream response message to the MC client which contains the details of the non-3GPP transport resources related parameters. The MC gateway UE may choose to send the details of existing communication channel information as part of non-3GPP transport resources establishment parameters IE if existing transport resources can be reused.

5.	The MC client establishes the communication channel with the MC gateway UE based on the parameters received in step 4 to receive the MC service group communication data from the MC gateway UE, if these parameters are not referring to any of the already established transport resources. The MC Gateway UE forwards the MC service group communication data it received over the MBS session from the MC service server to the MC client over this transport resources.

6.	The MC service server sends the downlink media for the group communication session over the MBS session.

7.	The MC gateway UE checks which MC clients should receive the media of the MC group communication based on Step 3.

8.	The MC gateway UE forwards the downlink media to the intended MC clients over the transport resources established as in step 5.

7.x.3.3	Procedure for handling UnMapGroupFromSessionStream message

Whenever the MC client receives UnMapGroupFromSessionStream message and if the MC client participates in the group session or communication identified by the UnMapGroupFromSessionStream message then it should inform the details contained in the UnMapGroupFromSessionStream message to MC gateway UE. When the association of group call, MBS session and the MC GW service ID of the MC client is known to the MC gateway UE, it can remove the association and stop forwarding of the traffic received over MBS session accordingly.

Figure 11.5.3.3.2A-1 illustrates the procedure for handling the UnMapGroupFromSessionStream message by the MC client and the MC gateway UE.

Pre-conditions:

1.	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

2.	The MC client successfully completed service authorization via MC gateway UE.

Figure 11.5.3.3.2A-1: Handling of UnMapGroupFromSessionStream message

1.	An MCPTT group call is ongoing; the MC gateway UE forwards the downlink media to the intended MC client over the transport resources established.

2.	MCPTT server has determined to disconnect the call over the MBS session for the MC client.

3.	An UnMapGroupFromSessionStream message is sent by the MCPTT server to MC client (if in MBS coverage area) on MBS session(s).

4.	If the MC client is participating in the MC group communication identified by the UnMapGroupFromSessionStream message, it sends the details contained in the UnMapGroupFromSessionStream message to the MC gateway UE through MC GW UnMapGroupFromSessionStream request message.

5.	The MC gateway UE on receiving the MC GW UnMapGroupFromSessionStream Request message from the MC client it removes the association maintained between the MC Service ID and the corresponding MBS session.

6.	The MC gateway UE sends the MC GW UnMapGroupFromSessionStream response message to the MC client.

7.x.3.4	Procedure for MBS session suspension notification

The MC service server can choose to instruct some MC clients to send the MBS session suspension report when notified by RAN. When the MC clients are residing on non 3GPP devices, MC gateway UE would be the one listening on the MBS sessions. When RAN decides to suspend the MBS session it indicates the MC gateway UE. MC gateway UE to notify the MC clients it is serving so that MC clients can report the same to the MC service server. This procedure is applicable only if the MC client is instructed to report the MBS session suspension. Irrespective of whether the MC clients need to send the MBS session suspension report to the MC service server, MC gateway can choose to notify the MC clients it is serving whenever RAN suspends the MBS session. MC clients can then decide to send the MBS session suspension report to the MC service server only if they are instructed by the MC service server.

Figure 11.5.3.3.3-1 illustrates the procedure for MC clients residing on non-3GPP devices reporting the MC service server about the MBS session suspension.

Figure 11.5.3.3.3-1: MBS session suspension notification

1.	The MC service server sends an MBS suspension reporting instruction to the MC client residing on non-3GPP device.

NOTE:	This message may be included in the MBS session announcement message and may be sent both on a unicast PDU session and an MBS sesison.

2.	RAN decides to suspend the MBS session.

3.	An MBS suspension indication is sent to the MC gateway UE.

4.	The MC gateway UE detect the MBS suspension and sends an MC GW MBS suspension indication to the MC Client residing on non-3GPP device.

5.	The MC client MBS suspension report to the MC Service server via MC gateway UE.

7.x.3.5	Procedure for reporting MBS session quality

The MC gateway UE listening on the MBS session has to report the MBS session quality to the MC clients so that MC clients can report the same to the MC service server. MC Gateway UE monitors an MBS session to receive MC service media. Based on the received quality (e.g. radio level quality) the MC gateway UE needs to inform the MC Clients which requested the MC gateway UE to listen on MBS session, whether it is able to receive the MC service media on the MBS session with sufficient quality or not the MC Clients can inform the MC service server accordingly.

Figure 11.5.3.3.4-1 illustrates the procedure for MC clients residing on non-3GPP devices reporting the MC service server about the MBS session quality.

Pre-conditions:

1.	There is an MBS session activated and the MBS session information is announced to the MC gateway UE.

2.	The MC gateway UE is located in the MBS broadcasting area

Figure 11.5.3.3.4-1: Reporting MBS session quality

1.	The MC gateway UE follows the Step 1 of the procedure as described in clause 10.7.3.6.2 for the MC service UE. The corresponding MBS session quality information is forwarded to all the MC clients which have asked the MC gateway UE to listen on the particular MBS session.

2.	If the MBS session quality reaches a certain threshold, the MC gateway UE sends an MC GW MBS session quality report to the MC client. The threshold is used to define the MBS listening status, which indicates if the MBS session quality has been acceptable or not to receive a specific MC service media. If the MBS session quality is mapped to a different MBS reception quality level, the MC gateway UE may send an MBS session Quality report including the MBS reception quality level to the MC Client.

NOTE:	The threshold used to indicate MBS session quality depends on service type (i.e. MCPTT, MCVideo or MCData) and the metrics used. The metrics used and the associated thresholds are out of scope of this specification.

3.	The MC Client sends the MBS listening status report to the MC Service server via MC gateway UE containing the information received in the MC GW MBS session quality report.

4.	The MC service server may send an additional proposal for measurements e.g. information about neighbouring MBS sessions. This message may be an MBS session announcement message.