3GPP TSG-SA WG6 Meeting #66	S6-251151

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on New key issue on SEALDD-S and SEALDD-UU user plane protocol

Spec:	3GPP TR 23.700-52 v0.0.0

Agenda item:	9.7

Document for:	Approval

Contact:	Cuili Ge – gecuili@huawei.com

1. Introduction

This pCR is proposed to introduce a new Key issue.

2. Reason for Change

According to the objective the FS_SEALDD_Ph3 SID, the user plane aspect of SEALDD-S and SEALDD-U needs to be studied.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-52 v0.0.0.

* * * First Change * * * *

5.x	Key issue #x: SEALDD-S and SEALDD-UU user plane

5.x.1	Description

Currently, the SEALDD defined in 3GPP TS 23.433 [23.433] is mainly focus on the signalling aspect of delivery service over SEALDD-S and SEALDD-U, i.e., only the control signalling messages over SEALDD-UU and SEALDD-S are specified by SA6 and implemented by CT working groups. However, SEALDD user plane (i.e., SEALDD-UU, SEALDD-S) protocol including e.g., the applicable protocol to be used , any 3GPP extension if required is not well specified. In fact, several user plane functions are already described in the specification, including but not limited to:

1) The SEALDD flow ID is allocated to uniquely identify the application traffic. Such ID should be in the header of the each SEALDD UU packet for this purpose.

2) The SEALDD enabled data transmission quality measurement in clause 9.7.2 requires the monitoring header with timestamp and the dummy DL SEALDD packet.

3) SEALDD enabled E2E redundant transmission provides the replication and deduplication between the two SEALDD flows. How to manage sequence number of the SEALDD UU packets with the same application content in the two individual SEALDD flows, especially considering the packet loss and retransmission.

In addition, different SEALDD delivery services may use different SEALDD user plane protocols to convey the application traffic, e.g., the user plane protocol used by SEALDD E2E redundant transmission service could be different from the one used by SEALDD regular connection management service.

Whether choosing an existing protocol like RTP, QUIC, to convey the SEALDD UU flow packets, introducing the necessary extensions to cover the above information, or SA6 defines a totally new protocol has to be further studied.

5.x.2	Open issues

1.	Analyze and classify the protocol(s) of user plane of SEALDD-S and SEADD-U to be used for the SEAL data delivery services;

2.	Whether choosing an existing protocol like RTP, QUIC, to convey the SEALDD UU flow packets, introducing the necessary extensions to cover the above information, or SA6 defines a totally new protocol has to be further studied.