3GPP TSG-SA WG6 Meeting #66	S6-251206

Gothenburg, Sweden 7th – 11th April 2025	(revision of S6-251xxx)

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on the brief description of SEAL services

Spec:	3GPP TR 23.700-35 v0.3.0

Agenda item:	8.9

Document for:	Approval

Contact:	Yanmei Yang yangyanmei@huawei.com

- Introduction

As following content mentioned in Gap#5-4-2-1, it is important to address the valued use case and advantages to applications to improve the adoption of SEAL services:

“It is essential to study how SEAL services benefit application developers (e.g., streamlined development time, enhanced robustness and reliability, abstraction of complicated APIs, etc.) and explore what new use cases can be enabled by these advantages.”

This contribution is to provide an overall summary of the values and beneficial use case(s) of each SEAL service.

- Reason for Change

The following aspects are important to let external Ecosystem partners improve the understanding of the SEAL services:

-What kind of function each SEAL service support?

-What kind of benefits a SEAL service can bring to Applications?

-The advantages compared to competing technologies

- Beneficial use case(s)

It is proposed to make overall summary of SEAL services from the above aspects.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-35 v0.3.0.

* * * First Change * * * *

8.2.5	Adoption Solution#5: Target application use case of SEAL services

8.2.5.1	Description

8.2.5.1.1	General description

This solution is to address the Generic gap #4. It provides the content about the value and beneficial use case and advantages of SEAL services. –

8.2.5.1.2	Overall introduction of SEAL services

This solution is to address the overall summary of the values and beneficial use case(s) of each SEAL service.

The supporting of the following are the key services that make SEAL layer distinct with other non-3GPP basis developer API Platform:

-NSCE/NRM/SEAL-DD which are specific to the usage of 5G/4G network capabilities.

-EMMTel enabler which is specific the usage of IMS system capabilities.

-LMS utilize both 3GPP based position and non-3GPP based position

-AI/ML enabler could support AI/ML service related to mobile UE, leveraging on the 3GPP network-for-AI capabilities

| Service type | Service description | Benefits to application developers | Beneficial use case(s) example | Advantages compared with competion technogies |
| --- | --- | --- | --- | --- |
| NRM | - Provide value added APIs by abstrating the capabilties served by core network layer entities MB-SMF/PCF/NEF APIs/SCEF/BMSC, to enable third party to use 3GPP QoS service in a simple way | -Improve the transmission qualitiy of application data compared to best effort transmission | - Applications requiring URLLC as mentioned in clause 7.2 of 3GPP TS 22.261 - Applications requiring High data rate and low latency as mentioned in clause 7.6 of 3GPP TS 22.261 - Applications requiring Low latency e.g IMS/MCX Voice call/video call -Application requiring low cost like backgroup data -Application requiring low power comsumption like NB-IOT applications | Compared to SA2 defined NEF APIs, it futher simplify the implement of Application deveoplers |
| SEAL DD | - Provide Caching and Distribution between application content server and mobile UE by using various 3GPP network capabilities designed for different type of application data to optimization purpose. For example reliable transmission mechanism specific to URLLC data. | -xx | The application service providers who want to use the Content Caching and Delivery services provied by SEAL DD deloyed close to mobile end users for further improving the user experience compared to use existing CDN based on OTT technology | existing CDN based on OTT technology, SEAL DD could Provid better QoE performance leveraging with the usage of 3GPP network layer transmision optimization capabilities |
| NSCE | - Provide the API services to enable third party to retrive the slice SLA requirement automaticly, order slice product from MNO system, mornitor/change slice SLA, by aggregating the API services from OAM and Core network, Client on the UE. | - Enable third party to self-mange the dedicate slice product rented from MNO | Some of the vertical industry players may rent dedicate slice(s) for their vertical applications. They may intent to have some slice management operation by themselves. The NSCE support the required capabilities to enable the slice Customer do the related operation by a management client which use the API services from the NSCE. | No competed technologies |
| AI/ML enabler | TBD | TBD | TBD | TBD |
| LMS | TBD | TBD | TBD | TBD |
| Group management | TBD | TBD | TBD | TBD |
| Configuration management | TBD | TBD | TBD | TBD |
| Digital asset | TBD | TBD | TBD | TBD |
| Spatial Anhor enabler | TBD | TBD | TBD | TBD |

8.2.5.1.3	SEALDD service to target application use case

This solution provide clarification on SEALDD service linked to target application use case, and provide analysis on SEALDD mechanism and CDN mechanism.

SEALDD provides some enhanced content delivery functions, which fully use of 5G network to perform content delivery, thus the SEALDD can provide content distribution with QoS guarantee which provide better service performance to third party.

In the real business deployment, CDN can be deployed by PLMN operators, or third party. Existing widely deployed CDN server use OTT mechanism. It cannot provide QoS guarantee benefitting from the capabilities of 5G/5GA enhancement for XR/Video/URLLC.

PLMN operators or third party CDN providers may support SEALDD functions in their system to offer CDN services for further enhancing the data delivering performance.

Editor’s Note: The analytics of CDN and the Comparisons between CDN and SEALDD is FFS.

* * * Next Change * * * *

* * * End of  Change * * * *