3GPP TSG-SA6 Meeting #66	S6-251218

# Stor-Göteborg, Sweden, 7th Apr 2025 - 11th Apr 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0385 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Establishing communication with MM service requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-03-31 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TR 23.700-21, Solution #18 “Discovery and Requirements Translation for 5G-enabled Traffic Flow Simulation and Situational Awareness” concluded to be progressed to the normative phase of XRM_Ph2_App. The new feature based on this solution is proposed in this CR to meet the Stage 1 requirements for MM services. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The new capability is provided as part of SEAL NRM to undertake the requirements translation and connectivity support for situational awareness scenarios. The change includes the information flows and the procedure for the new capability based on the concluded solution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new feature related to metaverse study will not be included in Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: 14.3.2.xx, 14.3.2.xy, 14.3.2.xz, 14.3.2.yy, 14.3.2.zz, 14.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 14.3.2.xx	MM service requirements request

Table 14.3.2.xx-1 describes the information flow MM service requirements request from the VAL server to the NRM server.

Table 14.3.2.xx-1: MM service requirements request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | Identity of the requestor (VAL server). |
| VAL service ID | M | The VAL service identity for whom the service requirement occurs. |
| MM performance requirements | M | The mobile metaverse QoS/QoE requirements and KPIs (latency, error rate, ..) for the MM end to end service. This may optionally include information which will support the NRM server to identify the per session QoS requirements. |
| List of VAL UEs or VAL Group ID | O | List of VAL UEs or the group of VAL UEs for whom the MM service requirement applies |
| >VAL UE/user ID | M | Identity of the VAL UE. |
| > Connection information | O | Connection information including address / port and / or URL of the VAL UE. |
| Service area | M | The area where the MM service is deployed. This can be geographical area, or topological area. |
| Time validity | O | The time of validity of the requirement. |

#### 14.3.2.xy	MM service requirements response

Table 14.3.2.xy-1 describes the information flow MM service requirements response from the NRM server to the VAL server.

Table 14.3.2.xy-1: MM service requirements response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The positive or negative result of the MM service requirements request. |

#### 14.3.2.xz	MM service connectivity request

Table 14.3.2.xz-1 describes the information flow MM service connectivity request from the NRM server to the NRM client 1 and 2.

Table 14.3.2.xz-1: MM service connectivity request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service identity for whom the connectivity occurs |
| Session information | M | Session information for the established connectivity for MM service communication. |
| >MM session ID | M | The identity of the session. |
| > MM session type | M | The type of session (UE to network or UE-to-UE) |
| >MM QoS configuration | M | The configuration of the QoS attributes (latency, jitter) for the VAL session within the MM service |
| >List of destination VAL UE IDs | O | The list of destination VAL UEs which are selected for the connectivity to be established for the MM service communication. |
| Service area | M | The area where the MM service is deployed. This can be geographical area, or topological area. |
| Time validity | O | The time of validity of the connectivity request. |
| MM service requirements | M | Details of service requirements |
| >Packet size (see NOTE) | O | Size of the packet to be transmitted. |
| >Packet transmission interval (see NOTE) | O | Intervals at which the packet is to be transmitted |
| >Packet E2E latency (see NOTE) | O | E2E Latency for packet transmission |
| >Packet error KPI (see NOTE) | O | The KPIs related to packet error (e.g. allowed packet loss rate, packet loss amount, packet error rate) |
| >Bitrate (see NOTE) | O | The bitrate required |
| Application connectivity context | O | Application connectivity context of the requester, used for determining the destination and/or resource parameters. If this IE is included, at least one of the information elements in Table 14.3.2.35-2 shall be provided. |
| NOTE: At least one of information elements of the application service requirements shall be included. |  |  |

#### 14.3.2.yy	MM service connectivity response

Table 14.3.2.yy-1 describes the information flow for MM service connectivity response from the NRM client to the NRM server.

Table 14.3.2.yy-1: MM service connectivity response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the NRM client in response to request indicating success or failure for accepting the request. |
| Cause | O | The cause of failure, if the result indicates failure. |

#### 14.3.2.zz	MM service connectivity notification

Table 14.3.2.zz-1 describes the information flow for MM service connectivity notification from the NRM server to the VAL server.

Table 14.3.2.zz-1: MM service connectivity notification

| Information element | Status | Description |
| --- | --- | --- |
| Session information | M | Session information for the established connectivity for application communication |
| VAL service ID | M | Identifier of the VAL service |
| List of VAL UEs | O | The list of VAL UEs which are accepted for the connectivity established for the application service communication. |
| >VAL UE/user ID | O | Identity of the VAL UE. |
| > Connection information | O | Connection information including address / port and / or URL of the VAL UE. |

* * * * Second Change * * * *

### 14.3.x	Establishing communication with Mobile Metaverse (MM) service requirements

#### 14.3.x.1	General

In Mobile Metaverse for 5G-enabled Traffic Flow Simulation and Situational Awareness use case and requirements as specified in 3GPP TS 22.156, real-time information and data about the real objects can be delivered to the virtual objects of the road infrastructure and traffic participants including vulnerable road users can form a smart transport metaverse. Then real-time processing and computing can be conducted to support traffic simulation and also situational awareness and real time path guidance and real-time safety, or security alerts can be generated for ICVs as well as the driver and passengers. To support such use case, it is essential to discover the identities of the virtual UEs (as counterparts of the physical UEs) and identify the sessions required as well as the per session requirement in both physical and virtual world.

This capability provides a mechanism applicable to mobile metaverse services, and in particular situational awareness to support the translation of MM-specific service requirements to connectivity requirements and provisioning parameters.

#### 14.3.x.2	Procedure

This clause describes the procedure for the translation of MM service requirements to network and connectivity requirements and the MM service connectivity support of all VAL UEs within the MM service area of interest.

Pre-conditions:

- NRM clients 1 and 2 are connected to NRM server.

- VAL server has deployed the MM service for the target VAL service area.

Figure 14.3.x.2-1: Establishing communication with MM service requirements

1.	The VAL server (MM service provider) sends an MM service requirement request message to the NRM server for configuring the connectivity and network requirements for the MM service.

2.	The NRM server authorizes the request and sends an MM service requirements response to the VAL server.

3.	The NRM server translates the MM service requirement to a network communication requirement for the mobile metaverse service. Such requirement includes the communication means / methods, the traffic patterns (which can be the application traffic schedules for the application sessions), the network topological area for which the service applies and the MM service area of interest.

4. 	The NRM server based on the translated network requirements, obtains information (identities/addresses and location info) for all the connected VAL UEs within the network coverage area where the MM service is deployed.

NOTE: For location information, such retrieval can be based on SEAL LMS service for receiving the list of UEs and their locations in each area/zone (see clause 9.3.12), or by querying this information from the UEs (via NRM clients) for the area of interest (see clause 9.3.4 and clause 9.3.5).

5.	The NRM server identifies the application sessions among the connected VAL UEs and between the VAL UEs and the VAL servers  required for the end-to-end MM service. Then, NRM server translates the network requirements to different per application session performance requirements for expected communications among the discovered virtual - physical UEs (also considering their relative location in both digital and physical world).

The NRM server triggers 3GPP system to establish QoS flow between the UE of NRM client 1 and the 3GPP network and the QoS flow between the UE of NRM client 2 and the network with individual QoS requirements derived from step 3 followed the procedure as specified in 3GPP TS 23.502 [12], 3GPP TS 23.501 [10].

6.	The NRM server sends an MM service connectivity request to NRM client 1 and NRM client 2.

7.	The NRM client 1 and 2 send an MM service connectivity response to NRM server.

8.	The NRM server sends the MM service connectivity notification (connectivity/session information) to VAL server indicating successful establishment of the connectivity.