3GPP TSG-SA3 Meeting #105-e 	S3-214228

# e-meeting, 8 - 19 November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | 1240 | rev | -- | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Rel-16]Clarification on KIAB generation for CP-UP separation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, Huawei, Hisilicon, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To clarify that incase of CP-UP separation of IAB-donor-CU (contains IAB-donor-CU-CP and IAB-donor-CU-UP that use different IP address), then the IAB-donor-CU-CP generates pre-shared key (KIAB) for IAB-donor-CU-UP and provide it over the E1 interface. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incase of CP-UP separation of the IAB-donor-CU, the IAB-donor-CU-CP and the IAB-node DU generate keys KIAB-CU-CP and KIAB-CU-UP. KIAB-CU-CP is used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-CP. The IAB-donor-CU-CP provides KIAB-CU-UP to the IAB-donor-CU-UP via E1 interface and KIAB-CU-UP is used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-UP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the specification on KIAB derivation for CP-UP separation of the IAB-donor-CU. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.23, M.3.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** Start of changes ****

# A.23	KIAB generation function

This input string is used when the IAB-node and the IAB-donor derive KIAB (PSK) for establishment of secure F1 interface. The following parameters shall be used to form the input S to the KDF:

-	FC = 0x83,

-	P0 = IAB-donor-CU IP address,

-	L0 = length of IAB-donor-CU IP address,

- 	P1 = IAB-node DU IP address,

-	L1 = length of IAB-node DU IP address.

The input key KEY shall be KgNB, if the key KgNB is in possession of the IAB-UE functionality in the IAB-node and in the IAB-donor-CU, after the IAB-UE setup procedure (Phase-1).

The input key KEY shall be S-KgNB, if the key S-KgNB is in possession of the IAB-UE functionality in the IAB-node and in the IAB-donor-CU, after dual connectivity procedure.

For P0, in case of CP-UP separation of IAB-donor-CU,

- 	P0 shall be set to IAB-donor-CU-CP IP address for deriving KIAB-CU-CP.

- 	P0 shall be set to IAB-donor-CU-UP IP address for deriving KIAB-CU-UP.

The entire output of the KDF (256 bits) is used as the KIAB or KIAB-CU-CP or KIAB-CU-UP.

**** Next change ****

### M.3.3.2	Security mechanisms for the F1 interface

The F1 interface connects the IAB-node (gNB-DU) to the IAB-donor-CU. It consists of the F1-C for control plane and the F1-U for the user plane.

F1 security for IAB is established using the security mechanisms for the F1 interface as specified in clause 9.8.2 of the present document, with IAB node taking the role of gNB-DU and IAB-donor-CU taking the role of gNB-CU.

In addition to the security mechanisms specified in clause 9.8.2 of the present document for the F1 interface, the IKEv2 Pre-shared Secret Key (PSK) authentication shall be supported. When IKEv2 performs a PSK authentication, in the IKE_AUTH request message, the IAB node shall set the ID type to ID_KEY-ID and set its value to PSK ID.

NOTE 1:	The PSK and PSK ID (for IKEv2 PSK authentication) can be preconfigured at the IAB node and IAB donor. Pre-configuration of the PSK(s) is out of the scope of the present document.

Additionally, to support a flexible plug and play of IAB-node and IAB-donor without a pre-configuration of the PSK(s), dynamic PSK computation for IKEv2 PSK authentication may also be supported. When dynamic PSK is used, the IAB-node and the IAB-donor shall calculate the PSK (KIAB) as specified in the Annex A.23 of this document. The IAB-donor shall uniquely identify the IAB-node’s security context (KgNB) using the IAB-node DU IP address. The IAB-donor shall use KIAB as PSK for IKEv2 between IAB-node and the IAB-donor. KIAB is stored in the IAB-node and in the IAB-donor. This key KIAB and the IPsec SA cryptographic keys are taken into use with the establishment of IPsec Security Association (SA) between the IAB-node and the IAB-donor. KIAB remains valid as long as the IAB-node is connected to the IAB-donor or until the IAB-node is re-authenticated. In case of CP-UP separation of IAB-donor-CU (IAB-donor-CU contains IAB-donor-CU-CP and IAB-donor-CU-UP that use different IP address) then, IAB-donor-CU-CP and IAB-node DU shall generate KIAB-CU-CP and KIAB-CU-UP as specified in the Annex A.23 of this document. The key KIAB-CU-CP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-CP. The IAB-donor-CU-CP shall provide KIAB-CU-UP to the IAB-donor-CU-UP via E1 interface and KIAB-CU-UP shall be used for establishment of secure F1 interface between the IAB-node DU and IAB-donor-CU-UP.

NOTE 2:	KIAB is used as the PSK for IKEv2 authentication, the interface between the IAB-donor-CU and the SEG to provision the key KIAB in the SEG is implementation specific and out of the scope of the present document.

**** End of changes ****