3GPP TSG SA WG3 (Security) meeting #49bis	S3a071018

Sophia Antipolis, France, 11-13 December 2007

Title:	DRAFT Reply to S3a071004= R2-074587 and 
S3a070987 = S2-075795 on Signalling for Paging

Work Item:		IMS security extensions

Source:		TSG SA WG3

To:		TSG SA WG2, TSG RAN WG 2

Cc:

Contact Person:

#### Name:	   Günther Horn

E-mail Address: guenther.horn@nsn.com

Attachments: none

[text highlighted in yellow points to changes from S3a070972.]

SA3 would like to thank RAN2 and SA2 for their LSs.

SA3 would like to point out to RAN2 that we do not have security concerns with respect to transferring the IMSI mod K to the eNB over the S1 interface for the purpose of scheduling the paging occasion.

SA3 came to this conclusion due to the following considerations:

SA3 assumes that the S1 interface will be protected using network domain security, such that the IMSI mod K cannot be obtained by an attacker on the S1 interface. Similarly, the interface between eNBs is secured using network domain security. Therefore transferring the IMSI mod K between eNBs during handover (if planned in RAN2) would also be acceptable. As a consequence, an attacker cannot obtain IMSI mod K when it is transferred from one network component to another.

Note also that a compromise of an eNB would have far more serious consequences than obtaining the IMSIs mod K for the connected UEs. For example traffic insertion and eavesdropping on all UE initiated and terminated traffic would be possible if an attacker could control an eNB.

SA3 would, however, like to point out to SA2 that the IMSI should not be used as identifier in the RRC paging message sent from eNB to UE over the air interface unless there is no other temporary identifier available. Also the IMSI should only be provided to eNB in case there is no temporary identifier available. I.e. the IMSI should be included the paging message sent from the network to the eNB only if no temporary identifier is available. The eNB should use the IMSI as identifier in the RRC message only if it was used in the paging message received from the MME such that the IMSI is available in eNB no longer than necessary. Also, if the IMSI is used for paging, it shall be ensured that the UE obtains a new S-TMSI before the next paging.

Actions:

SA3 kindly asks SA2 and RAN2 to take the above considerations into account.

Dates of Next TSG-SA WG3 Meetings:

SA3#50 	25th – 29th February 2008 		Asia

SA3#51	14th – 18th April 2008		North America