3GPP TSG-SA3 Meeting #100-bis-e 	S3-202416-r2

- e-meeting, 12-16 October 2020													Revision of S3-20xxxx

- Source:	InterDigital

- Title:	Solution for UAV authentication and authorization using EAP-based PDU secondary authentication

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

It is proposed to approve this new solution in TR 33.854.

# 2	References

# 3	Rationale

The contribution proposes a solution to address KI#1 "UAS Authentication and Authorization".

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854. Note that all text is new

***	BEGIN OF CHANGES	***

## 6.X	Solution #X: UAV authentication and authorization using EAP-based PDU secondary authentication

### 6.X.1	Solution overview

This solution addresses Key Issue#1 "UAS Authentication and Authorization".

This solution is applicable to 5GS and EPS for both UAV and networked UAV-C.

This solution enables a secondary authentication with a USS/UTM, reusing existing mechanisms defined for the PDU secondary authentication by an external DN-AAA procedure. An EAP-based secondary authentication is triggered by SMF during a PDU Session establishment procedure based on the UE subscription information and local policies. The authentication and authorization (A&A) by the USS/UTM procedure is performed using non-3GPP credentials (e.g., CAA-level UAV ID, certificate). The SMF acts as the EAP authenticator while the USS/UTM acts as the DN-AAA server. The same procedure can be supported in EPC by UE providing the UAV ID in a PCO and with the PGWc enhanced to support PDU secondary authentication by a DN-AAA feature (as per solutions in TR 23.754).

Editor's note: Support for equivalent to PDU Secondary authentication by DN-AAA in EPS is FFS

NOTE: for simplicity, only the non-roaming case is represented in this solution. Home Routed scenario is also supported using similar principles as for the PDU secondary authentication by external DN-AAA (i.e., V-SMF acting as a proxy for H-SMF acting as the authenticator).

The USS/UTM may initiate UAV authorization revocation at any time after successful completion of authorization procedure.

Editor's Note: UAV authorization revocation procedure details are FFS

### 6.X.2	Solution details

The procedure for UAV A&A by USS/UTM based on PDU secondary authentication is depicted in Figure 6.x.2-1. The same procedure may be used with a networked UAV-C.

Figure 6.x.2-1: Procedure for UAV Authentication and Authorization with USS/UTM PDU Session establishment

0. The UE has successfully completed a primary authentication and is registered with the network.

1. UE sends a PDU session establishment request message that may include the following parameters: a long-term UAV ID (CAA-level UAV ID) that is communicated to the USS/UTM. The UE may also provide a USS/UTM address. AMF sends corresponding request to SMF.

NOTE 1: It is assumed the UE has obtained prior to this procedure the CAA-level UAV ID from a CAA and has been configured with a USS/UTM address by means outside of 3GPP scope

2. The SMF determines whether the UE is allowed for UAS operations based on subscription information and local policies.

3. The SMF triggers an EAP-based authentication procedure towards the USS/UTM. SMF resolves the address of the USS/UTM based on provided CAA-level UAV ID or USS/UTM address (if provided). During the procedure, the SMF provides the USS/UTM with a 3GPP UAV ID (e.g., GPSI as External id) and receives from the USS/UTM a new assigned CAA-level UAV ID (e.g., a temporary Session id) upon successful authentication and authorization.

Editor's note: details of the authentication of the USS/UTM (or its address provided by the UE) by the network are FFS

4. Upon successful authorization by USS/UTM, the SMF sends a PDU session establishment accept message that includes the new CAA-level UAV ID.

5. The UE may additionally establish a separate PDU Session dedicated for UAS communications. A separate PDU session is necessary if a separate DNN from the one used to communicate with USS/UTM is used for communication with a UAV-C (e.g., while the first PDU session is being used for network Remote ID and tracking functionality). The UE provides the CAA-level UAV ID obtained from successful authorization by USS/UTM.

6. The UE receives a PDU Session Establishment Accept message authorizing UAS communications.

NOTE 2: Additional details for pairing authorization performed over first or second PDU Session are assumed to be covered in solutions for KI#2

7. The UE exchanges UAS traffic with peer UAV-C.

### 6.X.3	Solution evaluation

***	END OF CHANGES	***