- 3GPP TSG SA WG3 Meeting #100bis-e                     draft-S3-202351-r1

- e-meeting, 12-16 October 2020                         	revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	New solution on Security of Network Information Provisioning to Local Applications

- Document for:	Approval

Agenda Item:	2.8

# Decision/action requested

This contribution proposes a solution for Security of Network Information Provisioning to Local Applications (KI #7) in TR 33.839

# References

TR33. 839 Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC.

# Rational

This contribution proposes the potential solution for Security of Network Information Provisioning to Local Applications.

Request SA3 to capture the proposed solution.

# Detailed proposal

*************** Start of 1st Change ****************

## 6.X	Solution #X: Protection of Network Information Provisioning to Local AF directly

### 6.X.1	Solution overview

This solution addresses the security requirement for the case that the UPF exposes information to local AF directly in the key issue 7.

Editor’s Note: which interface for UPF and AF communicate over is FFS.

### 6.X.2	Solution details

Figure 6.x.2-1 Protection of Network Information Provisioning to Local AF

- AF establish a TLS session with the NEF, to secure the communication between the AF and NEF.

2a-2b. The AF generates a Key-ID for the UE and derives a KLE. The Key-ID is used to identify the KLE, and the KLE is used to protect the message transmission between UPF and AF. The AF provides the Key-ID to NEF in the request service.

3-4. The NEF initiates the policy authorization with PCF, including the K-ID and KLE received from the AF.  PCF initiates the PDU session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3] and provides the Key-ID and KLE to the SMF.

5. SMF sends the notification information with Key-ID and KLE to the UPF.

6. When the QoS monitoring information is received from RAN, the UPF generates MAC-I over the message using the KLE to prove its authenticity. The UPF sends the message including Key-ID and MAC-I to AF.

7. The AF retrieves the KLE based on the received Key-ID and verify the MAC-I.

### 6.X.3	Solution evaluation

TBD.

*************** End of 1st Change ****************