3GPP TSG-SA3 Meeting #100bis-e 	draft_S3-202305-r1

- e-meeting, 12-16 October 2020

- Source:	NCSC, Ericsson

- Title:	New Evaluation for Solution 2

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this evaluation of a solution for protecting the long term key during storage in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution 2 evaluation.

# 4	Detailed proposal

START OF CHANGE 1

## 7.2	Solution 2: Encrypted transfer of the LTK between UDR and UDM/ARPF

### 7.2.1	Introduction

The long term key is transported in encrypted form during its transfer from UDR to UDM/APRF, using a symmetric-key algorithm, where the decryption key is stored securely in the UDM/APRF. In order to enable the use of multiple decription keys, also a key identifier is transferred in association with the long term key. It is assumed that the long term key is stored in encrypted form in the UDR (see Solution #x), so there is no need to store the encryption key in the UDR.

### 7.2.2	Solution details

The long term key shall be transferred in encrypted form between UDR and UDM/ARPF. The encryption algorithm shall be a NIST approved symmetric-key algorithm where the secret shared key (for encryption) is generated (and securely stored) at the source of the generation and encryption of the long term-key. The secret shared key (for decryption) shall also be present in secure storage inside the UDM/ARPF.

There may be multiple encryption/decryption keys. In this case, during transfer, a key identifier shall be associated to the encrypted long term key. The key identifier shall enable the retrieval of the associated decryption key in the UDM/ARPF.

Transfer of the encrypted long term key between UDR and UDM/ARPF shall be based on secure encrypted transport mechanisms (such as SSH, HTTPS, or FTPS).

### 7.2.3	Evaluation

This solution fulfils the requirements of the KI by protecting the transfer of the long term key between the UDR and the UDM/ARPF in two ways:

-          transporting the long term key in encrypted form during its transfer from UDR to UDM/APRF, and

-          additionally, protecting the transfer of the long term key over Nudr based on secure encrypted transport mechanisms (such as HTTPS).

This solution is already specified in stage 3 TS 29.505 since Release 15 and does not require any further specification work. Furthermore, this solution is already in existing HSS/AuC deployments following the Data Layered Architecture as defined in TS 23.335 [xx] since quite some time.

END OF CHANGE 1