3GPP TSG-SA3 Meeting #100bis-e 	draft_S3-202305-r2

- e-meeting, 12-16 October 2020

- Source:	NCSC, Ericsson

- Title:	New Evaluation for Solution 5

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this evaluation of a solution for protecting the long term key during storage in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution 2 evaluation.

# 4	Detailed proposal

START OF CHANGE 1

## 7.5	Solution #5: Encrypted transfer of the long-term key between UDR and UDM/ARPF

### 7.5.1	Introduction

This solution addresses key issue #3 on "protection of long-term key during transfer out of UDR".

As described in 3GPP TS 33.501 [2], clause 5.8.1, the long-term key used for authentication and security association setup purposes shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected. If stored in the UDR, the long-term key is always transported in encrypted form during its transfer from UDR to UDM/ARPF.

### 7.5.2	Solution details

The long-term key is transferred in encrypted form between UDR and UDM/ARPF. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

Editor’s note: the encryption algorithm used and the method for the handling of the encryption/decryption key(s) is FFS.

The transfer of the encrypted long-term key over the Nudr interface is protected at transport level using the security mechanisms defined in 3GPP TS 33.501 [2], clause 13.1, as any other SBA interface.

During transfer, a key identifier is associated to the encrypted long-term key to enable identification of the decryption key in the UDM/ARPF.

NOTE:	The implementation of how to identify the decryption key is out of scope of 3GPP.

### 7.2.3	Evaluation

This solution fulfils the requirements of the KI by protecting the transfer of the long term key between the UDR and the UDM/ARPF in two ways:

-          transporting the long term key in encrypted form during its transfer from UDR to UDM/APRF, and

-          additionally, protecting the transfer of the long term key over Nudr based on secure encrypted transport mechanisms (such as HTTPS).

END OF CHANGE 1