3GPP TSG-SA3 Meeting #100bis-e 	draft_S3-202306-r3

- e-meeting, 12-16 October 2020

- Source:	NCSC, Ericsson

- Title:	New Evaluation for Solution 3

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this evaluation of a solution for protecting the long term key during transfer out of the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution 3 proposes that the long-term key is provisioned and stored in encrypted form in the UDR. This requires the creation of a key encryption key, or KEK, to encrypt the long-term key. This KEK must then be protected from being obtained or modified to the same level of security that the original long-term key was, which creates a new key issue with all the same problems. Without a solution to protecting the KEK, we have simply reduced the problem we were trying to solve, and have given the perception of security without necessarily providing any.

# 4	Detailed proposal

START OF CHANGE 1

## 7.3	Solution 3: Protection of LTK over Nudr

### 7.3.1	Introduction

This solution addresses key issue 3 on "protection of long-term key during transfer out of UDR".

The solution is based on storing the long-term key encrypted in UDR and transferring it also encrypted over Nudr. As any other SBA reference point, Nudr is additionally protected using TLS as defined in 3GPP TS 33.501 [2].

This solution is based on capabilities defined or planned to already to be defined in 3GPP TSs and does not require any additional specification work.

### 7.3.2	Solution details

The OAuth 2.0 based authorization framework defined in 3GPP 33.501 [2] is being enhanced in Release 16 to allows the possibility to generate OAuth 2.0 tokens to retrieve AuthenticationSubscription data ONLY to UDM/ARPF NF type of service consumers.

This solution proposes that the protection of the long-term key while provided to the UDM/ARPF over Nudr is two-fold:

- In the first place, the long-term key is provisioned and stored in UDR in encrypted form. This solution does not cover actual mechanisms to perform such encryption (e.g. encryption algorithms, key length, etc …).

- Secondly, as any other SBA reference point, the Nudr is protected using TLS as defined in 3GPP TS 33.501 [2].

These two protection levels make impossible for any intermediate actor to eavesdrop or modify the long-term key while in transit over Nudr.

### 7.3.3	Evaluation

This solution fulfils the requirements of the KI by protecting the transfer of the long term key between the UDR and the UDM/ARPF in two ways:

-          transporting the long term key in encrypted form during its transfer from UDR to UDM/APRF, and

-          additionally, protecting the transfer of the long term key over Nudr based on secure encrypted transport mechanisms (such as HTTPS).

END OF CHANGE 1