- 3GPP TSG SA WG3 Meeting #100bis-e                            	draft_S3-202333-r1

- e-meeting, 12 – 16 October 2020	                         revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	Clarification on Key Issue #2

- Document for:	Approval

Agenda Item:	2.12

# Decision/action requested

This contribution proposes an update on current key issue #2 for TR 33.857

# References

[1]		3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

# Rational

“Remote provisioning for SNPN credentials (Component 2 of KI#4)” is concluded in clause 8.4 in 23.700-07 [1] as

“  -	Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SNPN credentials can be transmitted to UE via UP connectivity. The UE shall be configured with Default credentials in USIM to register with a PLMN where the UE can register with the Default credentials in order to communicate with the provisioning server;

-	When User Plane is used for provisioning of SO-SNPN credentials, the User plane remote provisioning protocol used and how the UE downloads the SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA2 and uses industry developed mechanisms;

-	Control Plane remote provisioning based on UE Parameters Update Procedure as defined in TS 23.502 can be used for provisioning of SO-SNPN credentials;

Editor’s note: SA3 feedback will need to be taken into account for including of the CP based provisioning.

Editor’s note: It is FFS whether the UE Parameters Update container can be delivered to the UE during the registration procedure.

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used, Provisioning Server (PS) can provision the credential to UE over User Plane (UP) connectivity;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP connectivity;

-	It shall be possible to pre-configure the Provisioning Server (PS) address, SO-SNPN identity, neither or both DNN/NSSAI used to access PS on the UE, and it shall also be possible that the O-SNPN provides the PS address and DNN/NSSAI used to access PS to the UE after successful authentication and authorization. The PS address and DNN/NSSAI used to access PS from the O-SNPN shall be integrity protected. The PS address and DNN/NSSAI used to access PS provided by the network is prioritized, if configured and overrides any PS address, and/or DNN/NSSAI used to access PS stored in the UE. Configuration of PS address and DNN/NSSAI used to access PS from the onboarding configuration data to the UE can be supported using one of the following methods:

Editor’s note:	the need to pre-configure the S-NSSAI and DNN in the UE and to send it to the UE is FFS as default DNN and S-NSSAI could be used for provisioning to an SNPN. Also, if needed, whether to progress one or more options into normative phase is FFS.

a.	SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10).

b.	Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure using UE Route Selection Policy (URSP) that may be subject UE capabilities. As part of UE initial registration (based on received UE capability information) AMF indicates to PCF that UE has requested restricted/provisioning registration. The PCF may initiate UE Policy delivery using UE Route Selection Policies (URSP), for instance to trigger UE after successful registration to request establishment of specific type of PDU Session limited to onboarding purposes only.

c.	In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area.

-	It is assumed that the UDM (or AAA) of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed.

Editor’s Note:	Whether the specific procedure for provisioning of UDM (or AAA) with the UE credentials/subscription data is in scope of 3GPP is FFS.

-	Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address, the device uses a well-known FQDN to perform PS discovery.

NOTE:	SA3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.”

There are many security related editor’s notes, the contribution proposes some clarification on key issue #2 for provisioning of SNPN credentials.

# Detailed proposal

*************** Start of 1st Change ****************

## 5.2	Key Issue #2 Provisioning of SNPN Credentials

### 5.2.1	Key issue details

This Key Issue aims at addressing security implications introduced in solutions related to Key Issue #4 in TR 23.700-07 [3].

The objective of Key Issue #4 in TR 23.700-07 [3] is twofold, UE onboarding and then remote provisioning of non-USIM credentials for SNPN and PNI-NPN. This Key Issue aims at studying the security implications related to the provisioning of SNPN credentials which are used for primary authentication for UE to register the SNPN.

The UE can perform the onboarding procedure on onboarding network, and then the UE may be remotely provisioned with the SNPN credentials by Provisioning Server (PS) maintained by vertical.

NOTE:	Designing completely new protocols is not in scope of this key issue.

- 5.2.2	Security threats

An unauthorized UE may be able to access PS for maliciously requiring remote provisioning service.

An unauthorized PS may be able to provide wrong remote provisioning service to the UE.

Unprotected provisioning of SNPN credentials may cause the SNPN credentials to be obtained or manipulated.

### 5.2.3	Potential security requirements

The UE and the PS shall be authorized for remote provisioning.

SNPN credentials shall be securely protected during remote provisioning.

*************** End of 1st Change ****************