- 3GPP TSG SA WG3 Meeting #100bis-e                            	S3-202334

- e-meeting, 12 – 16 October 2020	                         revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	New Solution for Network Access Authentication with Credentials owned by an AAA external to the SNPN

- Document for:	Approval

Agenda Item:	2.12

# Decision/action requested

This contribution proposes a new solution for key issue #1 for TR 33.857

# References

# Rational

The contribution proposes a new solution for key issue #1.

# Detailed proposal

*************** Start of 1st Change ****************

- 6.Y	Solution #Y: Network Access Authentication with Credentials owned by an AAA external to the SNPN

- 6.Y.1	Introduction

This solution addresses key issue #1, especially for SNPN + non-PLMN scenario depicted in figure 5.1.1-2.

UE

(R)AN

UPF

N2

N4

N1

N3

N12

N13

AMF

SMF

AUSF

UDM

N11

N8

SNPN

AAA

Nxx

3rd party

PAF

TBD

The specific architecture is shown in figure 6.Y.1-1 from TR 23.700-07 [3].

Figure 6.Y.1-1: Architecture for Network Access Authentication with Credentials owned by an AAA external to the SNPN

The solution assumes that:

- The 3rd party provides AAA, and the UE credentials are stored in the AAA.

- Primary Authentication Function (PAF) is introduced in SNPN for translation of SBI protocol and AAA protocol. The function can be collocated with NSSAAF, or AUSF.

The UE provides SUCI to the SNPN, and the AUSF retrieves UE’s credentials from the AAA according to SUCI and trigger EAP based authentication. In this solution, AAA performs role of authentication server.

PAF

AAA

AUSF

UE

SEAF

1. Registration Request

(UE ID)

2. Nausf_UEAuthentication_

Authenticate Request (UE ID)

3. SBI

(EAP trigger, AAA address)

4.AAA

(EAP trigger)

5. EAP (e.g. EAP-TLS)

6. AAA (EAP success, MSK)

10. Nausf_UEAuthentication_

Authenticate Response (EAP success, Kseaf)

7. SBI (EAP success, MSK)

9. Derive Kseaf according to Kausf

11. Auth-Req. (EAP success, ngKSI, ABBA)

12. Derive Kamf according to MSK

8. Derive Kausf according to MSK

6.Y.2	Solution details

Figure 6.Y.2-1: Network Access Authentication

1. 	The UE sends the Registration Request message to the SEAF, containing UE ID.

2.	The SEAF sends Nausf_UEAuthentication_Authenticate Request message to AUSF. The message includes the UE ID.

3.	The AUSF invokes external primary authentication service provided by PAF. The AUSF sends SBI message containing AAA address and EAP trigger (e.g. EAP-TLS start) message. The AUSF derives AAA address according to UE ID.

4.	The PAF finds AAA according to AAA address, translates SBI message to AAA protocol, and sends the EAP trigger message to the AAA. The EAP trigger message can be EAP-start message to trigger AAA for EAP authentication.

5.	The AAA triggers EAP authentication based on EAP trigger message, and plays as authentication server role. PAF, AUSF, and SEAF transparent the EAP messages exchanged between UE and AAA.

6.	If the authentication successes, the AAA derives MSK and EMSK, the AAA sends EAP success message and MSK with AAA protocol to the PAF.

7.	The PAF sends EAP success message and MSK via SBI to the AUSF.

8.	The AUSF derives KAUSF according to MSK.

9.	The AUSF calculates KSEAF from KAUSF.

10.	The AUSF sends the Nausf_UEAuthentication_Authenticate message to the SEAF, the message includes EAP success message together with the derived KSEAF.

11.	The SEAF sends Authentication Request message to the UE, the authentication procedure is finished. The message includes EAP success message, ngKSI and ABBA parameter. The SEAF derives the KAMF according the KSEAF.

12.	Upon receiving the EAP-Success message, the UE derives MSK and EMSK and uses the MSK to derive the KAUSF, and then derives KSEAF according to KAUSF. The UE derives the KAMF from the KSEAF. The KAMF will be used to enable NAS and AS security.

Editor’s Note: Security implications of deriving Kausf from MSK is ffs.

Editor’s Note: It needs to be clarified whether and how SUPI concealment can be used.

Editor’s Note: It needs to be clarified how the UE knows that the K_AUSF is derived from the MSK instead of the EMSK."

- 6.Y.3	System impact

TBA.

- 6.Y.4	Evaluation

TBA.

*************** End of 1st Change ****************