3GPP TSG-SA3 Meeting #100bis-e 	draft_S3-202346-r3

- e-meeting, 12 - 16 October 2020	                                                           Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	New key issue on security of one-to-one communication over PC5

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

Approve this contribution to add new Key Issue to TR 33.847

# 2	References

[1]	3GPP TS 23.303: "Technical Specification Group Services and System Aspects; Proximity-based servces (ProSe); Stage 2".

[2]	3GPP TS 33.303: "Technical Specification Group Services and System Aspects; Proximity-based servces (ProSe); Security aspects".

[3]	3GPP TR 23.752: "Technical Specification Group Services and System Aspects; Study on system enhancement for Proximity-based servces (ProSe) in the 5G System (5GS)".

[4]	3GPP TS 33.536: "Technical Specification Group Services and System Aspects; Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

# 3	Rationale

The LTE ProSe one-to-one communication may happen after discovery procedures, or after one-to-many ProSe communications. The detailed one-to-one (i.e. unicast) communication and the corresponding security aspects are defined for LTE ProSe in 3GPP TS 23.303 [1] and TS 33.303 [2], respectively. During the architecture study in SA2, TR 23.752 [3] proposes to introduce new features to 5G ProSe from 5G V2X, this may protentially reuse the security meshanisms from 5G V2X as defined in TS 33.536 [4]. Although the 5G V2X and the ProSe one-to-one communications both rely on the PC5 reference point, the ProSe may not be able to fully reuse the security mechanisms from 5G V2X scenario which is due to the fact that they may use different processing procedures. For this reason, it’s necessary to study the security of one-to-one communication which is dedicated for 5G ProSe scenario.

# 4	Detailed proposal

*************** BEGINNING OF CHANGES***************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[aa]	3GPP TS 23.303: "Technical Specification Group Services and System Aspects; Proximity-based servces (ProSe); Stage 2".

[bb]	3GPP TS 33.303: "Technical Specification Group Services and System Aspects; Proximity-based servces (ProSe); Security aspects".

[cc]	3GPP TR 23.752: "Technical Specification Group Services and System Aspects; Study on system enhancement for Proximity-based servces (ProSe) in the 5G System (5GS)".

[dd]	3GPP TS 33.536: "Technical Specification Group Services and System Aspects; Security aspects of 3GPP support for advanced Vehicle-to-Everything (V2X) services".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to 21.905 be the first in the list.

*************** END OF 1st CHANGE***************

*************** BEGINNING OF 2nd CHANGE***************

## X.Y	Key Issdue #Y: Security of one-to-one communication over PC5

### X.Y.1	Key issue details

One-to-one ProSe communication is realised by establishing a secure link over PC5 between initiating UE and peer UE, it is used by two UEs that want to directly exchange traffic or when a remote UE attaches to ProSe relay.

Editor’s Note: the one-to-one communication policy/parameter provisioning procedure shall inline with SA2.

Editor’s Note: it’s FFS whether this KI covers the out-of-coverage scenario.

The LTE ProSe one-to-one communication may happen after discovery procedures, or after one-to-many ProSe communications. The detailed one-to-one (i.e. unicast) communication and the corresponding security aspects are defined for LTE ProSe in 3GPP TS 23.303 [aa] and TS 33.303 [bb], respectively. During the architecture study in the TR 23.752 [cc] proposes to introduce new features to 5G ProSe from 5G V2X, this may protentially reuse the security meshanisms from 5G V2X as defined in TS 33.536 [dd]. Although the 5G V2X and the ProSe one-to-one communications both rely on the PC5 reference point, the ProSe may not be able to fully reuse the security mechanisms from 5G V2X scenario which is due to the fact that they may use different processing procedures. For this reason, it’s necessary to study the security of one-to-one communication which is dedicated for 5G ProSe scenario. 5G ProSe needs a reliable mechanism to establish and to use one-to-one communication over PC5.

### X.Y.2	Security threats

If the two UE cannot be mutually authenticated during one-to-one communication, a peer may connect to an attacker.

The signalling and user plane message exchanges during one-to-one communication may be seen in cleartext, modified or replayed by an attacker if lack of confidentiality protection and integrity protection.

If one-to-one communication (unicast) mechanism in 5G V2X is reused, an attacker may deploy bidding-down attack to force establishing unprotected connection between initiating UE and peer UE.

### X.Y.3	Potential Security requirements

The initiating UE shall establish a different security context for each receiving UE during the PC5 one-to-one communication establishment if the security is activated.

The mutual authentication between two UEs during one-to-one communication shall be supported.

The one-to-one communication link security establishment shall be protected from MitM attacks.

The PC5 one-to-one communication signalling shall support confidentiality protection, integrity protection and anti-replay protection.

The PC5 one-to-one communication user plane shall support confidentiality protection, integrity protection and anti-replay protection.

The system shall support means of providing particular PC5 one-to-one communication’s signalling and user plane security policies to UEs.

The initiating UE and peer UE shall provide a means to mitigate establishing unprotected connection caused by bidding-down attack.

*************** END OF CHANGES***************