3GPP TSG-SA WG3 Meeting #100Bis-e		S3-202376

e-meeting, 12 – 16 October 2020                                                                         revision of S3-

- Source:	Interdigital, CableLabs

- Title:	Update of KI#8 for TR 33.847 – privacy of identities for UE2UE Relay path switch

- Document for:	Approval

Agenda Item:	2.9		Study on Security Aspects of Enhancement for Proximity Based Services in 5GS. FS_5G_ProSe_Sec

# 1	Decision/action requested

This contribution proposes modification for  KI#8 for TR 33.847.

# 2	References

[1]	3GPP TR 23.752 V0.5.0 (2020-09) Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)

# 3	Rationale

TR 23.752 [1] in Clause 5.4, Key Issue #4: Support of UE-to-UE Relay, has the following key issue:

-	How to provide a mechanism for path changing in case of e.g. UE-to-UE Relay changes?

NOTE 1:	For the involvement of NG-RAN, coordination with RAN WGs is needed.

NOTE 2:	For security aspects, coordination with SA WG3 is needed.

The following solutions in TR 23.752 [1] address either entire UE to UE Path Change or its  aspects:

- Solution #9: Connection establishment via UE-to-UE Layer-2 Relay and has the following Editor's note: “The Details of the authentication and security procedure will be investigated by SA WG3 group.”

- Solution #10: ProSe 5G Layer-3 UE-to-UE Relay based on IP routing

- Solution #32: Support Layer-3 UE-to-UE Relay Based on IPv6 link-local addresses and has the following Editor’s Note: “Whether and how to support periodic change of the link-local IP address due to privacy requirement is FFS and co-ordinated with SA WG3 group.”

- Solution #50: Negotiated UE-to-UE Relay Reselection

3GPP system has to be able to preserve privacy of identities during UE to UE Relay path change. Failure to protect privacy of identities during path change will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in 5G ProSe Sec TR 33.847.

***	BEGIN OF CHANGES	***

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.8	Key Issue #8: Privacy of information over the UE-to-UE Relay

### 5.8.1	Key issue details

3GPP system has to be able to protect the privacy of identities exchanged in the communications between peer UEs over a UE-to-UE Relay. Failure to protect privacy of identities of peer UEs communicating over the UE-to-UE Relay will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

TR 23.752 [2] in Clause 5.4, Key Issue #4: Support of UE-to-UE Relay, has the following key issue:

-	How to enhance the system architecture to provide the security protection for relayed connection?

### 5.8.2	Security threats

Failure to protect privacy of identities exchanged in the communications between the peer UEs over the UE-to-UE Relay will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

Existing Link identifier update procedure specified in TS 33.536 [8] provides privacy of the identities on a per unicast link basis (e.g., the link between a UE and the UE-to-UE Relay). Therefore an attacker may be able to link identities exchanged over the link between a UE and the UE-to-UE Relay to those exchanged over the corresponding link between the peer UE and the UE-to-UE Relay

Path switch between UE-to-UE Relay UEs is a new feature aiming to preserve user experience. Such preservation may be achieved by making certain elements (e.g., IP addresses) of user experience persistent across sessions and UE-to-UE Relays.  Persistent parameters may leak unique attributes associated with UEs and other ProSe entities and allow privacy attacks on these entities (e.g., UEs). Failure to protect the privacy of entities and identities during UE to UE Relay path change will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of entities and identities.

### 5.8.3	Potential security requirements

The 5G System should provide means for mitigating trackability attacks on peer UEs during communications over a UE-to-UE Relay.

The 5G System should provide means for mitigating linkability attacks on peer UEs during communications over a UE-to-UE Relay.

3GPP system shall provide means to preserve privacy of entities and identities during path change caused by a UE-to-UE Relay change.

***	END OF CHANGES	***