3GPP TSG-SA3 Meeting #100bis-e	S3-202386r2

- e-meeting, 12 - 16 October 2020													Revision of S3-20xxxx

- Source:	Huawei, HiSilicon, InterDigital

- Title:	pCR – a new solution to UAV and UAVC pairing authorization

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

Approve this contribution to add a solution in TR33.854

# 2	References

[1]

# 3	Rationale

Note: assuming a key issue on UAV and UAVC paring is approved.

The contribution proposes a solution to address UAV and UAVC pairing authorization to prevent unauthorized control of UAVs. The main assumption is that the pairing is determined by USS/UTM or by UAS. It is not allowed to determine pairing of UAV and UAVC within themselves.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (all text are new) ***

## 6.X	Solution #X: UAV and UAVC pairing Authorization through bounded IDs

### 6.X.1	Solution overview

This solution addresses the key issue #y.

This solution assumes UAV and UAVC are equipped with SUPI and credentials from PLMN. The pairing authorization is performed after UAV or UAVC is authenticated by 3GPP systems through Primary authentication. It is performed when UAV or UAVC is being authenticated/authorized, or after it has been authenticated/authorized by USS/UTM.

### 6.X.2	Solution details

A general overview of the procedure involving UAV and UAVC paring authorization is shown in Figure below.

Figure 6.X.2-1: a general overview on UAV and UAVC pairing authorization

- Provisioning:

  - UAV and UAVC: provisioned with UAV IDs and corresponding credentials, e.g. their private/public key pairs and certificates issued by UAS service providers/operators. Regarding pairing, there are two options considered: 1) provision is not required, it will be provisioned at USS/UTM 2) pairing is provisioned at both UAV and UAVC, e.g. indicated using certificates

  - USS/UTM: provisioned with its private/public key pairs. The UAS service providers/operators have registered their public keys or their root CAs with USS/UTM so that USS/UTM can verify their issued certificates. Regarding pairing, similarly, there are two options: 1) pairing of UAV and UAVC has been provisioned and UAV-ID and UAVC-ID are bound together; 2) no pairing information provisioned

Note: UAV and UAVC pairing shall not be determined by other parties than USS/UTM or UAS itself, e.g. between UAV and UAVC. The provisioning is out of scope of 3GPP.

- Primary Authentication: UE1 (UAV) and UE2 (UAVC) are equipped with SUPI and 3GPP credentials and need to perform Primary Authentication as normal UEs before getting UAS services.

- For UAS-type UE, UAS authentication is performed for UAV and UAVC respectively.

- USS/UTM authorize UAV and UAVC pairing

  - Case 1 (pairing information is provisioned at USS/UTM): based on bound UAV-ID and UAVC-ID to determine whether pairing request from UAV (with UAV-ID and GPSI) or UAVC (with UAVC-ID and GPSI) can be authorized.

  - Case 2 (UAV-ID and UAVC-ID are paired and bound): based on bound information sent from UAV or UAVC to determine whether pairing request from UAV (with UAV-ID and GPSI) or UAVC (with UAVC-ID and GPSI) can be authorized. USS/UTM may need to verify the certificates presented by UAV/UAVC.

Note: UAV (or UAVC) does not send GPSI to AMF. Instead, it sends UE ID (e.g. GUTI or SUCI) as a normal UE and AMF will convert the UE ID into GPSI.

Inform PLMN the authorization results with GPSI pair.

Note: step 3 and step 2 may be combined depending on scenarios.

Editor's note: 	What identities and credentials (if any) are exchanged with USS/UTM to enable the pairing  authorization is FFS

- UAVC communicates with UAV through UPF (UP)

Editor's note: How to support revocation of pairing authorization is FFS

### 6.X.3	Solution evaluation

TBC

***	END OF CHANGES	***