3GPP TSG-SA3 Meeting #100bis-e 	S3-202395

- e-meeting, 12 -16 October 2020

- Source:	KPN

- Title:	New Key Issue – Operator control of ProSe direct communication

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

It is requested that SA3 approves this new key issue on the operator control of ProSe direct communication.

# 2	References

[1]	3GPP TR 33.847 Study on Security Aspects of Enhancement for Proximity Based Services in 5GS, v0.1.1

[2]			Operator controlled device-to-device communications in LTE-advanced networks

# 3	Rationale

In case of ProSe communication between ProSe UEs, operators have limited or no control over the communication. Such control is desirable, for instance to:

- Manage the frequency spectrum that is licenced to the operator

- Grant specific users the access to ProSe direct communication for a specific period etc.

This control refers mainly to the following four aspects (e.g. see [2]):

- Access authentication

- Connection control

- Resource allocation

- Lawful interception

# 4	Detailed proposal

START OF CHANGE 1

## 5.X	Key Issue #X: Authorisation in the UE-to-UE ProSe direct communication scenario

### 5.X.1	Key issue details

The 3GPP system has to be able to authorise UEs to perform UE-to-UE ProSe direct communication. This authorisation can take into account the duration of ProSe direct sessions and number of ProSe sessions established by the ProSe UEs. It can be necessary to revoke a previously granted authorisation even in cases where the ProSe UE is outside the coverage area of the network.

5.X.2	Security threats

Illegitimate access of resources to perform ProSe direct communication between ProSe UEs without control of the operator or beyond the operator dependent limit such as the duration of ProSe direct sessions and number of ProSe direct sessions established by the ProSe UEs.

### 5.X.3	Potential security requirements

-	Authorisation of a ProSe UE to perform ProSe direct communication session shall take into account the following:

-	Duration of usage of ProSe communication by the ProSe UEs;

-	Number of ProSe direct sessions established by the ProSe UEs.

-	It shall be possible to revoke the authorisation of ProSe UEs from performing ProSe direct communication even in cases when ProSe UE is outside the coverage area of network.

END OF CHANGE 1