3GPP TSG-SA3 Meeting #100bis-e 	S3-202404

- e-meeting, 12 - 16 October 2020

- Source:	KPN

- Title:	New solution for KI4: Encrypted storage of Milenage OPc value in UDR

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this solution based on encryption for protecting the storage of the Milenage OPc value in the UDR

# 2	References

[1]	3GPP TR 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.4.0

# 3	Rationale

A new solution is proposed for the protection of the Milenage OPc value during storage in the UDR based on storing the Milenage OPc value in encrypted form in the UDR.

# 4	Detailed proposal

START OF CHANGE 1

## 7.x	Solution #<x>: Encrypted storage of Milenage OPc value in the UDR

### 7.x.1	Introduction

This solution addresses key issue #4 on "protection of Milenage OPc value during storage in UDR".

If the Milenage OPc value is stored in the UDR, it is stored in encrypted form in the UDR. During primary authentication, the UDM/ARPF retrieves authentication subscription data for the UE which can include the Milenage OPc value as stored in UDR. That is, the Milenage OPc value is never provided by the UDR in clear text and there is no need for the UDR to decrypt the Milenage OPc value.

At generation of the Milenage OPc value an encryption key is used that is shared with the UDM/ARPF where it is decrypted during primary authentication. The generation of the Milenage OPc value can be performed in the network of an operator or it can be performed at the facility where USIMs are being provisioned.

### 7.x.2	Solution details

If stored in the UDR, the Milenage OPc value is stored in encrypted form. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

Editor’s note: the encryption algorithm used and the method for the handling of the encryption/decryption key(s) is FFS.

7.x.3	Evaluation

Editor's note: to be provided.

END OF CHANGE 1