3GPP TSG-SA WG3 Meeting #100Bis-e		S3-202414

e-meeting, 12 – 16 October 2020                                                                         revision of S3-

- Source:	Interdigital

- Title:	New KI for TR 33.847 – privacy of ProSe entities while supporting Non-IP traffic

- Document for:	Approval

Agenda Item:	2.9		Study on Security Aspects of Enhancement for Proximity Based Services in 5GS. FS_5G_ProSe_Sec

# 1	Decision/action requested

This contribution proposes a new KI for TR 33.847.

# 2	References

[1]	3GPP TR 23.752 V0.5.0 (2020-09) Study on system enhancement for Proximity based Services (ProSe) in the 5G System (5GS) (Release 17)

# 3	Rationale

TR 23.752 [1] in Clause 6.5.2, Procedures, states the following:

The "Procedures for V2X communication over PC5 reference point" defined in TS 23.287 [5] clause 6.3 is reused to support ProSe communication over NR based PC5 reference point, and the differences are highlighted as followings.

- For broadcast and groupcast mode ProSe communication, the procedures as defined in TS 23.287 [5] clauses 6.3.1 and 6.3.2 are applied with the following differences are identified:

-	The following data unit types are supported: IP, non-IP, Ethernet, Unstructured and Address Resolution Protocol (see RFC 826 [10]).

NOTE:	Whether "non-IP type" is used for "Unstructured type" can be decided in normative phase.

-	The ProSe Group IP multicast address for groupcast communication may be provisioned by PCF and is used to send and receive IP data.

- For unicast mode ProSe communication, the procedure as defined in TS 23.287 [5] clause 6.3.3 is applied with the following differences are identified:

-	DHCPv4 based IP address allocation is supported.

-	Both Ethernet and Unstructured data unit types are supported.

The following solutions in TR 23.752 [1] address the need to support Non-IP:

- Solution #10: ProSe 5G Layer-3 UE-to-UE Relay based on IP routing. It states this in Clause 6.10.1:

Security handling:

-	source UE and target UE can establish bearer level security with the UE-to-UE Relay for the unicast L2 Link, using procedures defined in TS 23.287 [5].

-	If end-to-end security protection is required between source UE and target UE, IPSec can be used.

NOTE 3:	The security protection of the traffic of source UE and target UE will be specified by SA WG3.

- Solution #49: Support Layer-3 UE-to-UE Relay to Handle Non-IP Traffic has the following in its Clause 6.49.1.3, Security:

For end-to-end security, IPsec cannot be used, since it is not IP traffic. It is up to the application to implement end-to-end security for non-IP traffic

Editor’s note: 	How to handle end-to-end security for Non-IP traffic is FFS and shall be co-ordinated with SA WG3 group

3GPP system has to be able to preserve privacy of ProSe entities while supporting NoN-IP traffic. Failure to protect privacy of identities while supporting NoN-IP traffic will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in 5G ProSe Sec TR 33.847.

***	BEGIN OF CHANGES	***

# 5	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

## 5.c	Key Issue #c: privacy of ProSe entities while supporting Non-IP traffic

### 5.c.1	Key issue details

3GPP system has to be able to preserve privacy of ProSe entities while supporting NoN-IP traffic. Failure to protect privacy of identities while supporting NoN-IP traffic will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of identities.

TR 23.752 [1] in Clause 6.5.2, Procedures, states the following:

The "Procedures for V2X communication over PC5 reference point" defined in TS 23.287 [5] clause 6.3 is reused to support ProSe communication over NR based PC5 reference point, and the differences are highlighted as followings.

- For broadcast and groupcast mode ProSe communication, the procedures as defined in TS 23.287 [5] clauses 6.3.1 and 6.3.2 are applied with the following differences are identified:

-	The following data unit types are supported: IP, non-IP, Ethernet, Unstructured and Address Resolution Protocol (see RFC 826 [10]).

NOTE:	Whether "non-IP type" is used for "Unstructured type" can be decided in normative phase.

-	The ProSe Group IP multicast address for groupcast communication may be provisioned by PCF and is used to send and receive IP data.

- For unicast mode ProSe communication, the procedure as defined in TS 23.287 [5] clause 6.3.3 is applied with the following differences are identified:

-	DHCPv4 based IP address allocation is supported.

-	Both Ethernet and Unstructured data unit types are supported.

Multiple solutions in TR 23.752 address support of NoN-IP traffic and require security and privacy protection that they provide to be studied in TR 33.847.

### 5.c.2	Security threats

Failure to protect privacy of entities and identities while supporting NoN-IP traffic for unicast/multicast/broadcast communication between two UEs, and for communication via UE-to-UE and UE-to-Network relays will open vulnerability in 5GS and allow various privacy attacks including tracing and tracking of entities and identities.

### 5.c.3	Potential security requirements

3GPP system shall provide means to preserve privacy of entities and identities while supporting NoN-IP unicast/multicast/broadcast communication between two UEs, and for communication via UE-to-UE and UE-to-Network relays.

NoN-IP traffic unicast/multicast/broadcast communication between two UEs, and for communication via UE-to-UE and UE-to-Network relays.

unicast/multicast/broadcast communication between two UEs, and for communication via UE-to-UE and UE-to-Network relays

***	END OF CHANGES	***