3GPP TSG-SA3 Meeting #100-bis-e 	S3-202415-r1

- e-meeting, 12-16 October 2020													Revision of S3-20xxxx

- Source:	InterDigital

- Title:	Solution for UAV authentication and authorization during registration

- Document for:	Approval

Agenda Item:	2.7

# 1	Decision/action requested

It is proposed to approve this new solution in TR 33.854.

# 2	References

# 3	Rationale

The contribution proposes a solution to address KI#1 "UAS Authentication and Authorization".

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.854. Note that all text is new

***	BEGIN OF CHANGES	***

## 6.X	Solution #X: UAV authentication and authorization during registration

### 6.X.1	Solution overview

This solution addresses Key Issue#1 "UAS Authentication and Authorization".

This solution is applicable to 5GS and to both UAV and networked UAV-C.

This solution enables an authentication and authorization (A&A) with a USS/UTM during registration after primary authentication successful completion in a procedure similar to Network Slice Specific Authentication and Authorization (NSSAA). An EAP-based authentication procedure is triggered by AMF following a Registration procedure based on the UE subscription and capabilities information. The procedure for authentication and authorization (A&A) by the USS/UTM is performed using non-3GPP credentials (e.g., CAA-level UAV ID, certificate). The AMF acts as the EAP authenticator while the USS/UTM acts as the AAA server.

The USS/UTM may initiate UAV authorization revocation at any time after successful completion of authorization procedure.

Editor's Note: UAV authorization revocation procedure details are FFS

### 6.X.2	Solution details

The procedure for UAV Authentication and Authorization by USS/UTM during registration ,  is depicted in Figure 6.x.2-1. The same procedure may be used with a networked UAV-C.

Figure 6.x.2-1: Procedure for UAV Authentication and Authorization with USS/UTM during registration

Pre-condition: UAV is configured with a long-term UAV ID (e.g., serial number, CAA registration id) and credentials used for authentication by USS/UTM. The UAV ID and credentials are obtained by means outside of 3GPP scope

1. The UE sends a Registration Request message including its UE id, a UAV id and UAV communications capabilities.

NOTE 1: If the UAV id is subject to privacy protection, existing partial cyphering mechanisms may be used to protect it during initial Registration transmission.

2. If the UE is not already authenticated by the network, a primary authentication procedure is performed.

3. The AMF determines whether a UAV A&A by USS/UTM is required based on:

- Subscription information (i.e., whether the UE is authorized for UAS operations)

- If the UAV is undergoing A&A by USS/UTM procedure or UAV has previously performed such procedure succesfully and the authorization was allowed and still valid.

4. AMF sends in the Registration Accept message a pending UAV A&A indication. UE refrains from establishing PDU Session dedicated to UAS communications until the successful completion of the following A&A steps. The Registration Accept message may include some other configuration information such as allowed UAS communication modes/types (e.g., network assisted, direct). The UE sends a Registration Complete if this is an initial Registration.

5. AMF triggers an EAP-based UAV A&A by USS/UTM procedure. UE is authenticated using UAV credentials (e.g., CAA-level UAV ID, certificate). During the procedure, the AMF provides the USS/UTM with a 3GPP UAV ID (e.g., GPSI as External id) and AMF may receive a CAA-level UAV id (e.g., a temporary Session id) from USS/UTM. The AMF stores the CAA-level UAV id in the UE context. The AMF may use the CAA-level UAV id to determine whether to perform UAV A&A as described in step 2. The AMF provides the CAA-level UAV id and to the UE in the following step.

NOTE 2: It is assumed that the AMF may communicate with the USS/UTM using an A&A proxy function (similar to NSSAAF). This proxy function provides USS/UTM discovery/address resolution, authentication messages routing and protocol translation capabilities.

6. Upon successful UAV A&A by USS/UTM, AMF initiates the UE Configuration Update procedure to deliver authorized UAS Configuration parameters to the UE. The UAS Configuration may include the following parameters to be used for UAS communication setup: the CAA-level UAV ID, S-NSSAI/DNN.

7. The UE establishes a PDU Session using authorized UAS parameters as provided in step 6 (e.g., CAA-level UAV ID)

8. The UE receives a PDU Session Establishment Accept message authorizing UAS communications.

9. The UE exchanges UAS traffic with peer UAV-C.

NOTE 3: PDU Session establishment and UAS communications steps above may be subject to additional pairing with UAV-C authorization. Additional details for pairing authorization are assumed to be covered in solutions for KI#2

### 6.X.3	Solution evaluation

***	END OF CHANGES	***