3GPP TSG-SA3 Meeting #100bis-e 	S3- 202464-r1

- e-meeting, 12-16 October 2020

- Source:	CableLabs, Philips International B.V, Apple, Deutsche Telekom AG, InterDigital Communications, Charter Communications

- Title:	Updates to solution #20 (6.20.2.1)

- Document for:	Approval

Agenda Item:	2.1

# 1	Decision/action requested

It is proposed to approve the changes to solution #20 in TR 33.809.

# 2	Rationale

This contribution addresses the editor notes in 6.20.2.1.

# 3	Detailed proposal

****START OF CHANGES ***

#### 6.20.2.1	Digital Signatures of System Information

System information is digitally signed along with a number of other attributes, including time information (such as a Time Counter as proposed in solution #7), physical cell ID, downlink frequency, etc.

More specifically, the (single) signature is calculated over the following information attributes:

- PCI (PSS, SSS)

- MIB

- SIB1

- Time counter

- Downlink frequence

Other SIBs can be signed along with SIB1 or individually depending on their periodicity.

Signing of dynamic fields such as SFN are described in 6.20.2.2.1.

***END OF CHANGES***