- 3GPP TSG-SA3 Meeting #100bis-e 	S3-202485-r1

- e-meeting, 12 – 16 October 2020	Merge of S3-202485, S3-202322, and S3-202323

- Source:	Huawei, Hisilicon, ZTE

- Title:		EC: New solution on transport security for EDGE-1-9 interfaces

- Document for:	Approval

Agenda Item:	2.8

# 1	Decision/action requested

This contribution proposes a new solution on transport security for EDGE-1-9 interfaces.

# 2	References

[1]	3GPP TS 33.839 	Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC

# 3	Rationale

This contribution proposes a new solution on transport security for EDGE-1-9 interfaces, by reusing the existing transport security protection for NEF/SCEF APIs, etc.

# 4	Detailed proposal

It is proposed to approve the following changes.

*************** Start of the changes ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications."

[3]	3GPP TR 23.748: "Study on enhancement of support for Edge Computing in the 5G Core network (5GC)".

[4]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[5]	3GPP TS 23.502: "Procedure for the 5G System; Stage 2".

[6]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[aa]	3GPP TS 33.501: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[bb]	3GPP TS 33.187: "Security aspects of Machine-Type Communications (MTC) and other mobile data applications communications enhancements".

[cc]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[dd]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

*************** Next change ****************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| Solution #1: DNS request protection |  |  |  |  |  |  |  |  | x |  |
| Solution #2: Authentication between EEC and ECS based on primary authentication |  | x |  |  |  |  |  |  |  |  |
| Solution #3: Authentication/Authorization framework for Edge Enabler Client and Servers | x | x |  |  |  |  |  |  |  |  |
| Solution #X: Transport security for EDGE-1-9 interfaces |  |  |  |  |  | X |  |  |  |  |
| #X: <Solution name> | X |  |  |  |  |  |  |  |  |  |

*************** Next change ****************

## 6.X	Solution #X: Transport security for EDGE-1-9 interfaces

### 6.X.1	Introduction

This solution addressed the transport security requirements for EDGE-1-9 interfaces in key issue #6. Generally, NDS/IP should be used here for data protection.

### 6.X.2	Solution details

There are three types of interfaces related with edge application architecture defined in TR 23.558 [2]. Hence, the transport security will be discussed separately within three subclauses.

#### 6.X.2.1 Type A

Interfaces of type A (Between UE and Edge servers) are as follows:

- EDGE-1: between EEC and EES

- EDGE-4: between EEC and EAS

- EDGE-5: between EEC and Application Client(s)

Editor’s Note: How to protect the EDGE-1 and EDGE-4 interfaces is FFS.

According to TS 23.558[2], details of the EDGE-5 is out of scope of this release of this specification. Hence the transport security of EDGE-5 is out of scope of this release, and can be left for implementation.

#### 6.X.2.2 Type B

Interfaces of Type B (Between 3GPP core and Edge servers) are as follows:

- EDGE-2: between 3GPP Core network and EES

- EDGE-7: between 3GPP Core network and ECS

- EDGE-8: between 3GPP Core network and EAS

How to protect the interface between 3GPP Core network and EES/ECS/EAS, depends on the functionality, which will be performed on this interface.

As defined in TS 23.558 clause 6.4.2, it says.

EDGE-2 reference point enables interactions between the Edge Enabler Server and the 3GPP Core Network. It supports:

a)	access to 3GPP Core Network functions and APIs for retrieval of network capability information, e.g. via SCEF and NEF APIs as defined in 3GPP TS 23.501 [2], 3GPP TS 23.502 [3], 3GPP TS 29.522 [4], 3GPP TS 23.682 [17], 3GPP TS 29.122 [5], and with the EES acting as a trusted AF in 5GC (see 3GPP TS 23.501 [2] clause 5.13, 3GPP TS 23.503 [12]).

NOTE:	EDGE-2 reference point reuses 3GPP reference points or interfaces of EPS or 5GS considering different deployment models.

Similarly, EDGE-7/8 is used to support the same functionality as the EDGE-2.

Therefore, For EDGE-2/7/8,

- if the NEF APIs is selected, security aspects of Network Exposure Function including the protection of NEF-AF interface and support of CAPIF defined in TS 33.501 clause 12 [aa] can be reused here to protect the EDGE-2/7/8 interfaces, i.e. TLS shall be used.

- if the SCEF APIs is selected, the Security procedures for reference point SCEF-SCS/AS defined in TS 33.187 clause 5.5 [bb] can be reused here, i.e. TLS shall be used.

NOTE: Transport security protection of EDGE-2/7/8 should take the other deployment models in the future.

#### 6.X.2.3 Type C

Interfaces of type C (Between Edge servers) are as follows:

- EDGE-3: between EAS and EES. The supported functionalities include EAS registration, de-registration, etc.

- EDGE-6: between EES and ECS. The supported functionalities include EES registration.

- EDGE-9: between EES(s). The supported functionalities include discovery of target EAS.

As all the exchanged data of EDGE-3/6/9 is in the application layer, transport security protection on the SBI interface can be reused here. Hence TLS shall be used as specified in TS 33.210 [cc], unless security is provided by other means, e.g. physical security. A SEG may be used to terminate the NDS/IP IPsec tunnels.

For the EDGE-3, if the CAPIF capability is consumed by the EAS, the interface security defined in the TS 33.501 clause 12 can be reused here to protect the CAPIF related data transferred in the EDGE-3 interfaces, i.e. TLS shall be used.

NOTE 1: 	Regardless of whether TLS is used or not, NDS/IP as specified in TS 33.210 [cc] and TS 33.310 [dd] can be used for network layer protection.

Editor’s Note: whether the above transport security protection mechanism for Type C interfaces is sufficient is FFS.

### 6.X.3	Evaluation

TBD

*************** End of the changes ****************