- 3GPP TSG-SA3 Meeting #100bis-e 	S3-202486-r1

- e-meeting, 12 – 16 October 2020

- Source:	Huawei, Hisilicon

- Title:		5G ProSe: New key issue on UE-to-UE Relay or UE-to-Network Relay selection

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

This contribution proposes a new key issue on UE-to-UE Relay or UE-to-Network selection.

# 2	References

[1]	3GPP TS 33.847 Study on Security Aspects of Enhancement for Proximity Based Services in 5GS

# 3	Rationale

This contribution proposes a new key issue on UE-to-UE Relay or UE-to-Network Relay selection from the security point of view.

# 4	Detailed proposal

It is proposed to approve the following changes.

*************** Start of the changes ****************

## 5.X	Key Issue #X: Security of UE-to-UE Relay or UE-to-Network Relay selection.

### 5.X.1	Key issue details

TR 23.752 [2] in Clause 5.3, Key Issue #3: Support of UE-to-Network Relay has the following key issue:

-	How to (re)select a UE-to-Network Relay for communication path selection between two indirect network communication paths (i.e. path #2 and path #3 in figure 5.3.1-1).

NOTE 1: Security and privacy aspects will be handled by SA WG3.

TR 23.752 [2] in Clause 5.4, Key Issue #4: Support of UE-to-UE Relay has the following key issue:

-	How to (re)-select a UE-to-UE Relay UE in proximity?

NOTE 2: For security aspects, coordination with SA WG3 is needed..

Both the UE-to-UE relay and UE-to-Network relay are normal UEs, the security capabilities of which are limited, such as the data encryption or decryption computing. Hence, if a large number of remote UEs connected to the two relays and consumer the relay services, the encryption or integrity protection processing capability of the relay will be reduced due to these connections. In this case, the relay may discard the unprocessed data fo the remote UE. This key issue is to study the UE-to-UE Relay or UE-to-Network relay selection from the security point of view.

### 5.X.2	Security threats

The data of the remote UE will be discarded by the relay if the protection can not be fully performed in the UE-to-UE Relay or UE-to-Network Relay.

### 5.X.3	Potential security requirements

The system shall support UE-to-UE Relay or UE-to-Network Relay selection according its security capability.

*************** End of the changes ****************