3GPP TSG-SA3 Meeting #100bis-e 	S3-202492-r1

- e-meeting,  12 – 16 October 2020												Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	pCR: Revocation of authorization of MBS communication service

- Document for:	Approval

Agenda Item:	2.11

# 1	Decision/action requested

SA3 is kindly requested to approve the proposed new solution to MBS authentication and authorization.

# 2	References

[1]	3GPP TR 23.757 Study on architectural enhancements for 5G multicast-broadcast services

# 3	Rationale

KI#1 of 33.850 studies the authentication and authorization for a user to join a multicast group. The authorization revocation is considered as part of the solution to the KI#1 based on the discussion from last meeting, otherwise, once a user gains the authorization to a multicast service, the authorization remains valid forever.

This contribution proposes a solution to revoke the authorization for a UE to use a multicast communication service, for the KI#1 Security of authentication and authorization for multicast communication services in 33.850.

# 4	Detailed proposal

***	BEGINNING OF CHANGES (all text are new) ***

## 6.X	Solution #X: Authorization revocation

### 6.X.1	Solution overview

This solution proposes how the authorization revocation is performed, for KI#1. When the content provider decides that the user authorization for a multicast service needs to be revoked, the content provider will inform the UDM/UDR about the revocation. The UDM/UDR will accordingly instructs the SMF to release the corresponding resources established for the user for the multicast service.

### 6.X.2	Solution details

In the solution below, the (MB-)SMF is an enhanced SMF supporting 5G MBS. The solution works with both baseline architectures defined in TR 23.757 on 5G MBS.

Note:  (MB-SMF) is the MB-SMF in the baseline architecture 2 or the enhanced SMF in the baseline architecture 1, as defined in the TR 23.757 on 5G MBS.

Figure 6.X.2-1: Authentication revocation

- The AF of the content provider provisions the information on the multicast service/application including the authorization information to the UDM/UDR.  The NEF is involved in the provisioning if the content provider belongs to a 3rd party.

- The UE has successfully joined a multicast service/application and the PDU session for the multicast service/application has been established.

- The (MB-)SMF subscribes to the UDM/UDR on the changes of the multicast information including the authorization information.  Step 2 may also be performed during step 1.

- The content provider updates the multicast information.  The NEF is involved in the provisioning if the content provider belongs to a 3rd party.

- The UDM/UDR notifies the (MB-)SMF when the authorization for a UE to join the multicast service/application is revoked. The identifier of the multicast service/application is included in the notification.

- The (MB-)SMF releases the PDU session for the multicast service/application identified by the received  identifier.

Editor's note:  The identifier for the multicast service/application is to be updated according to SA2 progress.

Editor’s note: It is FFS whether UE triggering authorization revocation is needed.

### 6.X.3	Solution evaluation

TBC

***	END OF CHANGES	***