3GPP TSG-SA3 Meeting #100bis-e 	draft S3-202534-r5

- e-meeting, 17 -28 August 2020													      Merger of S3-202534, S3-202517, S3-202601, S3-202422, S3-202423

- Source:	China Mobile, Lenovo, Motorola Mobility, Huawei, Hisilicon, Nokia, Nokia Shanghai Bell

- Title:	Key issue on cyber-attacks detection supported by NWDAF

- Document for:	Approval

- Agenda Item:	2.16

# 1	Decision/action requested

This contribution proposes a key issue to TR 33.866 on cyber-attacks detection supported by NWDAF. SA3 is kindly requested to approve this contribution.

# 2	References

[1]	SP-200722 New SID on security aspects of enablers for Network Automation (eNA) for the 5G System (5GS) Phase 2

[2]           3GPP TR23.700-91v0.4.0 Study on Enablers for Network Automation for 5G - phase 2 (Release 17)

# 3	Rationale

The New SID on security aspects of enablers for Network Automation (eNA) for the 5G System (5GS) Phase 2 [1] was approved at SA#89-e. This contribution proposes a key issue related to this study. In TR 23.700-91[2], the use case of NWDAF supporting the detection of cyber-attacks (Use Case #5) was proposed and corresponding SA3 actions were indicated, therefore a key issue should be identified and further studied in SA3.

# 4	Detailed proposal

*************** Start of the 1st change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[X]	3GPP TR 23.700-91: " Study on enablers for network automation for the 5G System (5GS); Phase 2".

*************** End of the 1st change ****************

*************** Start of the 2nd change ****************

## 4.X	Key Issue #X: Cyber-attacks Detection supported by NWDAF

### 4.X.1	Key issue details

NWDAF has been defined to offer automatic network analytics and alarming, with possible capabilities of artificial intelligence and machine learning to help proactively manage the 5G network. TR 23.700-91[X] has identified the use case of NWDAF detecting cyber-attacks by monitoring events and data packets in the UE and the network, with support of machine-learning algorithms. To achieve cyber-attacks detection, the NWDAF can collaborate with UE and any other NFs to collect related data as inputs, afterwards providing alerts of anomaly events as outputs to OAM and other NFs which have subscribed to them so that they could take proper actions.

This key issue will study what kind of cyber-attacks can be detected by NWDAF. In order to mitigate the identified cyber attacks, the data/parameters collected by NWDAF need to be studied.

The specific cyber attacks include but are not limited to the following examples:

(1) MitM attacks on the radio interface: MitM attacks or fraudent relay nodes may modify or change messages between the UE and the RAN, resulting in failures of higher layer protocols such as NAS or the primary authentication. The NWDAF may detect MitM attacks.

(2) DoS attacks:, 5G has high performance requirements for system capacity and data rate, improved capacity and higher data rate may lead to much higher processing capability cost for network entities, which may make some network entities (e.g. RAN, Core Network Entities) to suffer from DDoS attack. The NWDAF may also enable the detection of DDoS attacks.

Then, in order to mitigate the identified cyber attacks, the data/parameters collected by NWDAF should also be studied.

4.X.2	Security threats

Cyber attacks or anomaly events may not be detected by the 5G network, thus further attacks could be conducted.

### 4.X.3	Potential security requirements

TBD

*************** End of the 2nd change ****************