3GPP TSG-SA3 Meeting #100bis-e 	S3-202xxx

- e-meeting, 12 -16 October 2020				merger of S3-202570 and S3-202524

Title:	[DRAFT] reply LS on security analysis for Solution 22 in TR 23.748

Response to:	LS S2-2006033 on checking security issue for Solution 22 in TR 23.748 from SA2

Release:	Release 17

Work Item:	Study on Security Aspects of Enhancement of Support for Edge Computing in 5GC

Source:	China Mobile(will be replaced by SA3 if approval)

To:	SA2

Cc:

Contact Person:

#### Name:	Minpeng Qi

Tel. Number:	+86 13910023873

E-mail Address:	qiminpeng@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA3 thanks SA2 to send LS on security issue discussion for solution 22 in TR 23.748. The security analysis from SA3 point of view is as following:

In TR23.748, solution 22 uses DNS mechanism to help UE find a suitable Edge Application Server (EAS) in the session breakout scenario.

SA3 believes that in solution 22, PDU session establishment and DNS resolution with pre-established UL CL/BP/L-PSA section are high level description and all details are shown in following DNS resolution before and after dynamic UL CL/L-PSA insertion section. However, in DNS resolution before and after Dynamic UL CL/L-PSA insertion section, security concerns are raised.

In DNS resolution before and after dynamic UL CL/L-PSA insertion procedure, there are two phases for EAS discovery. In phase 1, the discovery is happened before dynamic UL CL/BP/L-PSA insertion. in phase 2, the discovery is happened after dynamic UL CL/BP/L-PSA insertion In these two phases, there are 3 options for EAS discovery. And they can be grouped as two alternatives show in the procedure in TR 23.748: dynamic ULCL/L-PSA & filter that option 1 and option 2 are involved, and pre-established ULCL/L-PSA & filter that option 3 are involved.

For option 1 and option 2(option 2a and option 2b), LDNSR is destination about UE’s DNS request and is responsible to transfer such request to another DNS server. The E2E security is established between UE and LDNSR using security solutions like DoT, DOH, or DNS over DTLS. SMF inserted UL CL rules after resolvers receive the DNS response. As the DNS traffic is encrypted between UE and LDSNR, we don't see any issue with options 1, 2a, 2b.

For options 3a, 3b, encrypted DNS solutions like DoT, DOH, or DTLS is not feasible as UL CL needs to process the packet to forward to either Local PSA or PSA for DNS resolution. But it may be possible to use this option without encryption in a controlled and trusted environment, and it should be left to operators to decide whether to enforce the use of DNS encryption or whether to take the informed risk of disabling it. . If the address has to be changed as shown in option 3B, it implies the destination address of DNS Query is modified before reaching its real destination or the Local DNS resolver stores the UE originated DNS Query and generates a new DNS Query towards a Local DNS Server. .When the local DNS resolver is located in a local DN which is owned by the third party , there may be a security risk, but there should be no security issue if the local DNS resolver is located in an operator-controlled network element(e.g. Local PSA UPF). SA2 should consider locating the local DNS server in the operator-controlled domain if Option 3B is selected as part of the conclusion.

2. Actions:

To SA2 group:

Based on the above analysis, SA3 believes option 1 and option 2 in DNS resolution before and after Dynamic UL CL/L-PSA insertion procedure can be securely protected and terminated in LDNSR. For options 3a, 3b, encrypted DNS traffic is not an option; in that case, using this option without encryption should be left to the operator to decide. In another aspect, for option 3, especially in option 3B, the Local DNS Resolver should be under the control of the operator in case the Local DNS Resolver needs to change the destination IP address of a DNS Query to the Local DNS Server’s IP address or generate a new DNS Query destined for the local DNS server.

SA3 requests SA2 to consider the above analysis.

3. Date of Next TSG-SA WG3 Meetings:

SA3#101e	9 - 20 November 2020	e-meeting

SA3#101-Bis	18-22 January 2020	e-meeting