3GPP TSG-SA3 Meeting #100bis-e 	S3-202592

- e-meeting, 12 - 16 October 2020													Revision of S3-20xxxx

- Source:	Ericsson

- Title:	CU-UPs supported by DC

- Document for:	Approval

Agenda Item:	2.13

# 1	Decision/action requested

It is proposed to approve the pCR

# 2	References

[1]	3GPP TR 33.840 V0.0.3, Study on Security aspects of the disaggregated gNB architecture

# 3	Rationale

A new solution is proposed that supports multiple CU-UPs while ensuring separate security keys among different PDCP termination points.

# 4	Detailed proposal

**** START OF CHANGES ****

## 6.X	Solution #X: CU-UPs supported by DC

### 6.X.1	Introduction

This solution addresses the key issue #XX of the present document and aims to support multiple CU-UPs while ensuring separate security keys among them.

### 6.X.2	Solution details

In this solution, Dual Connectivity (DC) is used to ensure that multiple CU-UPs have separate security keys. See Figure 6.X.2-1.

Figure 6.X.2-1: CU-UPs supported by DC

The CU-CP shall configure DC with MCG and SCG with different DUs with connections to separate CU-UPs for MN and SN bearers respectively such that some UP traffics are sent to one CU-UP and other are sent to another CU-UP.

Thus, each CU-UP terminates respective PDCP security towards UE and it is ensured that multiple CU-UPs have separate security keys.

Security handling of DC shall be according to clause 6.10 of 3GPP TS 33.501.

Editor’s Note: This solution introduces an architecture restriction of one to one mapping between gNB-DU and gNB-CU-UP. A generic solution which addresses the general RAN architecture is FFS.

### 6.X.3	Evaluation

TBD

**** END OF CHANGES ****