3GPP TSG-SA3 Meeting #100e 	S3-202604

- e-meeting, 17 -28 August 2020													Revision of S3-20xxxx

- Source	:	Lenovo, Motorola Mobility, Nokia, Nokia Shanghai Bell, Deutsche Telekom, Samsung, Altiostar

- Title	:	Key Issue on Impact of Simultaneous CU-UP applying common UP security

- Document for	:	Approval

Agenda Item	:	2.13

# 1	Decision/action requested

This pCR proposes a Key Issue to TR 33.840 on Impact of Simultaneous CU-UP applying common UP security for a UE’s UP security

# 2	References

[1]	3GPP TS 38.401, ‘NG-RAN Architecture Description’, Rel-16

[2]	3GPP TR 38.823, ‘Study of further enhancement for disaggregated gNB’, Rel-16.

[3]	3GPP SP-200349 Study on security aspects of the Disaggregated gNB Architecture.

# 3	Rationale

In a scenario where different gNB-CU-UPs are selected for a UE, to provide different services simultaneously, a user plane security compromise at a single gNB-CU-UP will impact the over all user plane security of the UE (as all user planes for a UE share the same UP security keys). Further if any specific service (example. a URLLC based user plane connection(s) established with one gNB-CU-UP) needs service privacy from other simultaneous user plane connections established with different gNB-CU-UPs, then it is not possible to ensure service privacy with the current UP security principle. As the 5G system is expected to provide robust security for its end user services, SA3 should consider the disaggregated gNB deployment scenario and provide proactive measures to prevent any level user plane security breach in 5GS.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.840.

*****Start of Change*****

## 5.X	Key Issue #X: Impact of simultaneous gNB-CU-UP applying common User Plane Security

### 5.X.1	Key issue details

A gNB may consist of a gNB-CU-CP, multiple gNB-CU-UPs and multiple gNB-DUs. The gNB-CU-CP selects the appropriate gNB-CU-UP(s) for the UE requested services as defined in TS 38.401. To provide different set of services simultaneously for a UE, a gNB-CU-CP can select one or more gNB-CU-UPs at different physical locations (for example one for an URLLC service and the other for an eMBB service) but establishes a common user plane security as in Rel.15. Applying same user plane securityfor all simulatenous user plane connections of a UE irrespective of its gNB-CU-UP location may impact the user plane security of the UE.

NOTE Xote: It is FFS if there is any impact on the E1 interface.

### 5.X.2	Security threats

A compromised gNB-CU-UP may impact the over all user plane security of the UE(s) which has at least one user plane connection established with the compromised gNB-CU-UP.

NOTE Y: Poorly designed gNB could also expose the gNB to the same threat but gNB design is out of scope of this study

### 5.X.3	Potential security requirements

- A compromised gNB-CU-UP shall not impact the other other gNB-CU-UPs for one UE.

- Note x: Changes are applicable to Rel-17 onwards and solution should support backward compatibility.

Potential solutions shall not compromise the requirement that the NG-RAN internal architecture (i.e., CP/UP separation and DU/CU split) is invisible to the UE.

*****End of Change*****