3GPP TSG-SA3 Meeting #100bis-e 	S3-202612

- e-meeting, 12 - 16 October 2020													Revision of S3-20xxxx

- Source:	Samsung

- Title:	[5G_ProSe] Updates to solution#2

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

It is proposed to approve updates to solution#2 in TR 33.847.

# 2	References

[1]	3GPP TR 33.847: “Study on Security Aspects of Enhancement for Proximity Based Services in 5GS (Release 17)”

# 3	Rationale

This contribution is proposed to resolve the following Editor’s Note:

Editor's Note: Whether AKMA user plane architecture is used shall be based on conclusion from SA2.The PC3 interface between UE and 5GDDNMF is considered as Ua* interface and depends on Ua* protocol.

The above EN was captured incorrectly in TR 33.847 and is corrected as below:

Editor's Note: Whether AKMA user plane architecture is used shall be based on conclusion from SA2.The PC3 interface between UE and 5GDDNMF is considered as Ua* interface and depends on Ua* protocol.

# 4	Detailed proposal

*************************1st CHANGE****************

## 6.2	Solution #2: Secure data transfer between UE and 5GDDNMF

### 6.2.1	Introduction

This solution addresses key issue#10.

### 6.2.2	Solution details

In LTE ProSe, the protection of traffic between UE and ProSe Function is as specified in clause 5.3.3.2 in TS 33.303 [6]. For 5G ProSe the security can be established using Authentication and Key Management for Applications. Where AF is the ProSe Application Function (5GDDNMF) and AF should be authenticated and authorized by the operator network before providing the AKMA Application Key (KAF) to the AF.

Editor's Note: Whether 5GDDNMF is a functionality of PCF or an AF is based on conclusion from SA2.

It is proposed to use the AKMA network model and security procedure to have a secure data transfer between UE and the 5GDDNMF.

Figure 6.2.2-1: User plane architecture

Figure 6.2.2-1 is reference model for AKMA modified for supporting Proximity based services. The Application function in AKMA is 5GDDNMF in ProSe having a service-based interface N5gddnmf with other Network Functions, to consume or provide services from or to other NFs.

Editor's Note: Whether AKMA user plane architecture is used shall be based on conclusion from SA2.

The PC3 interface between UE and 5GDDNMF is considered as Ua* interface and depends on Ua* protocol.

However, the security requirement of PC3 interface should be aligned to satisfy the Ua* interface. Also, the interface Ua* needs to have new functionalities in addition to specified in clause 4.4.1 of TS 33.535 [7] for Ua*.

Editor’s note: The need of new functionalities is FFS and whether the new functionalities can be used in Ua* is FFS.

Editor’s note: The impact on Ua* interface and PC3 interface are FFS.

### 6.2.3	Evaluation

TBD