3GPP TSG-SA3 Meeting #100bis-e 	S3-202615

- e-meeting, 12 - 16 October 2020													Revision of S3-20xxxx

- Source:	Samsung

- Title:	Transport security for the MSGin5G interfaces

- Document for:	Approval

Agenda Item:	2.15

# 1	Decision/action requested

It is proposed to approve this key issue on transport security for the MSGin5G interfaces for TR 33.862.

# 2	References

[1]	3GPP TR 33.862 Study on security aspects of the Message Service for MIoT over 5G System (5GMSG)

# 3	Rationale

New interfaces (i.e. MSGin5G-1-5) were introduced in the architecture for MSGin5G Service. This key issue studies the related transport security, i.e. confidentiality, integrity and replay-protection.

# 4	Detailed proposal

*****Start of 1st Changes*****

# 2	References

[xx]	3GPP TR 23.700-24 Study on support of the 5GMSG Service (Release 17)

[yy]	3GPP TS 23.222 Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2 (Release 17)

*****Start of 2ndChanges*****

## 5.X	Key issue #X: Transport security for the MSGin5G interfaces

### 5.X.1	Key issue details

TR 23.700-24 [xx], clause 8.2 describes an application architecture of the MSGin5G Service.

- Figure 5.X-1: Application Architecture of the MSGin5G Service

New interfaces (i.e. MSGin5G-1-5) were introduced in the architecture for MSGin5G Service. This key issue studies the related transport security, i.e. confidentiality, integrity and replay-protection.

MSGin5G-1: Between a 5GMSGS client and a MSGin5G Server. This reference point supports:

  - Registration of a 5GMSGS client to a MSGin5G Server when not using IMS base solution; and the exchange of MSGin5G messages.

MSGin5G-2: Between a MSGin5G Server and the Legacy 3GPP Message. This reference point supports:

  - Indicating the underlying message delivery mechanism to the Legacy 3GPP Message Gateway; and exchange of MSGin5G messages; and registration of Legacy 3GPP Message Gateway to MSGin5G Server.

MSGin5G-3: Between an Application Server and a MSGin5G Server. This reference point supports:

  - Access to MSGin5G Server and APIs to enable sending and receiving of MSGin5G messages; and Adherence to CAPIF as specified in 3GPP TS 23.222[yy].

MSGin5G-4: Between a Non-3GPP Message Gateway and a MSGin5G server. This reference point supports:

  - Registration of Non-3GPP Message Gateway to MSGin5G Server; and the exchange of MSGin5G messages.

MSGin5G-5: Between an application client and a 5GMSGS client. This reference point supports:

  - Providing information from application clients required to enable the 5GMSGS client to construct a MSGin5G message to be delivered to other MSGin5G service endpoints.

  - Configuring application clients with information required to enable the 5GMSGS client and MSGin5G Server to exchange and route MSGin5G messages to other MSGin5G service endpoints.

  - Sending notifications and information in the incoming MSGin5G messages received by the 5GMSGS client to the application clients from other MSGin5G service endpoints.

NOTE: As MSGin5G-5 is an internal interface between application client and a 5GMSGS client within the UE, the protection should be taken care by the UE implementation.

### 5.X.2	Threats

Without confidentiality, integrity and replay protection, an attacker may eavesdrop or manipulate or replay the communication or initiate the MitM attacks on the interface.

### 5.X.3	Potential security requirements

Confidentiality protection, integrity protection and replay-protection shall be supported on the MSGin5G-1-4 interfaces.

*****End Change*****